3GPP TSG-RAN WG5 Meeting #108	R5-253846r2

Bengaluru, India, 25th – 29th Aug, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0198 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new SLPP error handling procedure test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Support of sidelink positioning in NR systems is one of the objectibve of the work item Expanded and improved NR positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added new test case 9.5.3.1 SLPP Requested Method not Supported between LMF and UE - UE-Assisted - Added new test case 9.5.3.2 SLPP Requested Method not Supported between UEs (Target UE is in coverage) - UE-Assisted - Added new test case 9.5.3.3 SLPP Requested Method not Supported between UEs (Target UE is out of coverage) - UE-Assisted |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The release 18 positioning work item will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.571-3 CR 0176 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - Corrected the city name on the cover sheet. - Corrected editorial errors - Added new tables for OCTET STRING type IEs R2：Changed NR-35 to NR-36 |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

9.5.3	SLPP Error Handling

9.5.3.1	SLPP Requested Method not Supported between LMF and UE - UE-Assisted

9.5.3.1.1	Test Purpose (TP)

(1)

with { 	a UE supporting at least one of target UE-assisted SL-RTT, target UE-assisted SL-AOA, target UE-assisted SL-TDOA or target UE-assisted SL-TOA but not all of them }

and with { a NAS signalling connection existing }

ensure that {
  when {	UE receives a SLPP message requesting at least one positioning method not supported }

then { the UE provides location information for the supported methods}

}

9.5.3.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.355 clauses 5.3.5 and clause 5.4.3. Unless otherwise stated these are Rel-18 requirements.

[TS 38.355, clause 5.3.5]

Upon receiving a RequestLocationInformation message, Endpoint A shall:

1>	if the requested information is compatible with Endpoint A's capabilities and configuration:

2>	include the requested information in a ProvideLocationInformation message;

2>	set the field sessionID in the response message to the same value as the field sessionID in the received message if received;

2>	set the field transactionID in the response to the same value as the field transactionID in the received message;

2>	deliver the ProvideLocationInformation message to lower layers for transmission.

1>	else if one or more positioning methods are included that Endpoint A does not support:

2>	continue to process the message as if it contained only information for the supported positioning methods;

2>	handle the signaling content of the unsupported positioning methods by SLPP error detection as in 5.4.3.

[TS 38.355, clause 5.4.3]

Upon receiving any SLPP message, the receiving endpoint shall attempt to decode the message and verify the presence of any errors and:

…

1>	if the message type is an SLPP RequestAssistanceData or RequestLocationInformation and some or all of the requested information is not supported:

2>	return any information that can be provided in a normal response, which includes indications on other information that is not supported.

9.5.3.1.3	Test description

9.5.3.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

-	System information combination NR-1 as defined in TS 38.508-1 [30] clause 4.4.3.1.3 is used in NR cell.

NR-SS-UE

-	NR-SS-UE1, NR-SS-UE2 (if required) and NR-SS-UE3 (if required) as specified in clause 8.2 operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	All NR-SS-UEs uses GNSS as the synchronization reference source.

GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [30], subclause 4.4A on NR Cell 1.

Related PICS/PIXIT Statements:

-

9.5.3.1.3.2	Test procedure sequence

Table 9.5.3.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends a RESET UE POSITIONING STORED INFORMATION message. | <-- | RESET UE POSITIONING STORED INFORMATION | - | - |
| 2 | The SS sends a SLPP message of type Request Capabilities. | <-- | DLInformationTransfer (SLPP REQUEST CAPABILITIES) | - | - |
| 3 | The UE sends a SLPP message of type Provide Capabilities including the UE positioning capabilities. | --> | ULInformationTransfer (SLPP PROVIDE CAPABILITIES) | - | - |
| 4 | IF the UE SLPP message at step 3 includes an acknowledgment request THEN SS sends a SLPP Acknowledgement response. | <-- | DLInformationTransfer (SLPP ACKNOWLEDGEMENT) | - | - |
| 5 | The SS sends a SLPP message of type Provide Assistance Data containing the data for all supported positioning methods. | <-- | DLInformationTransfer (SLPP PROVIDE ASSISTANCE DATA) | - | - |
| 6 | The SS sends a SLPP message of type Request Location Information including all specified positioning methods. | <-- | DLInformationTransfer (SLPP REQUEST LOCATION INFORMATION) | - | - |
| 7 | The UE sends a SLPP message of type Provide Location Information including information for the supported method(s). | --> | ULInformationTransfer (SLPP PROVIDE LOCATION INFORMATION) | 1 | P |
| 8 | IF the UE SLPP message at step 7 includes an acknowledgment request THEN SS sends a SLPP Acknowledgement response. | <-- | DLInformationTransfer (SLPP ACKNOWLEDGEMENT) | - | - |

9.5.3.1.3.3	Specific message contents

Table 9.5.3.1.3.3-1: RESET UE POSITIONING STORED INFORMATION

| Derivation Path: 38.509 clause 6.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE Positioning Technology |  | FFS | FFS |  |

Table 9.5.3.1.3.3-2: DL InformationTransfer (steps 2, 4, 5, 6 and 8, Table 9.5.3.1.3.2-1)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLInformationTransfer ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| dlInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.5.3.1.3.3-3 | DL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.5.3.1.3.3-3: DL NAS TRANSPORT (steps 2, 4, 5, 6 and 8, Table 9.5.3.1.3.2-1)

| Derivation Path: 24.501 Table 8.2.11.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 | Downlink generic NAS transport |  |
| DL NAS TRANSPORT message identity |  | 01101000 | DL NAS transport |  |
| Payload container type |  | 1100 | SLPP message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 2: Set according to Table 8.5-1 | SLPP Request Capabilities |  |
|  |  | Steps 4 and 8: Set according to Table Table 9.5.3.1.3.3-4 | SLPP Acknowledgement |  |
|  |  | Step 5: Set according to Table 9.5.3.1.3.3-5 | SLPP Provide Assistance Data |  |
|  |  | Step 6: Set according to Table 9.5.3.1.3.3-6 | SLPP Request Location Information |  |
| Additional information |  | Present | Routing Identifier/Correlation ID |  |

Table 9.5.3.1.3.3-4: SLPP Acknowledgement (steps 4 and 8, Table 9.5.3.1.3.2-1)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Not present |  |  |
| transactionID | Not present |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Not present |  |  |
| sessionID | Not present |  |  |
| acknowledgement SEQUENCE { |  |  |  |
| ackRequested | FALSE |  |  |
| ackIndicator | (0..255) | Contains the same value of the sequenceNumber field in 1 or 7, Table 9.5.1.4.3.2-2. |  |
| } |  |  |  |
| slpp-MessageBody | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 9.5.3.1.3.3-5: SLPP PROVIDE ASSISTANCE DATA (step 5, Table 9.5.3.1.3.2-1)

| Derivation Path: Table 8.5-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 8.5-2 with the following exceptions: |  |  |  |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| sl-AoA-ProvideAssistanceData | Present if UE supports target UE-assisted SL-AoA. As defined in clause 8.5.1 |  | pc_Target_UEA_SL_AoA |
| sl-RTT-ProvideAssistanceData | Present if UE supports target UE-assisted SL-RTT. As defined in clause 8.5.1 |  | pc_Target_UEA_SL_RTT |
| sl-TDOA-ProvideAssistanceData | Present if UE supports target UE-assisted SL-TDOA. As defined in clause 8.5.1 |  | pc_Target_UEA_SL_TDOA |
| sl-TOA-ProvideAssistanceData | Present if UE supports target UE-assisted SL-TOA. As defined in clause 8.5.1 |  | pc_Target_UEA_SL_TOA |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.5.3.1.3.3-6: SLPP REQUEST LOCATION INFORMATION (step 6, Table 9.5.3.1.3.2-1)

| Derivation Path: Table 8.5-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 8.4-2 with the following exceptions: |  |  |  |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| sl-AoA-RequestLocationInformation | Present. | OCTET STRING containing SL-AoA-RequestLocationInformation as defined in Table 8.5-4 |  |
| sl-RTT-RequestLocationInformation | Present | OCTET STRING containing SL-RTT-RequestLocationInformation as defined in Table 8.5-5 |  |
| sl-TDOA-RequestLocationInformation | Present | OCTET STRING containing SL-TDOA-RequestLocationInformation as defined in Table 8.5-6 |  |
| sl-TOA-RequestLocationInformation | Present | OCTET STRING containing SL-TOA-RequestLocationInformation as defined in Table 8.5-7 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.5.3.1.3.3-7: ULInformationTransfer (steps 3 and 7, Table 9.5.3.1.3.2-1)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULInformationTransfer ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ulInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.5.3.1.3.3-8 | UL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.5.3.1.3.3-8: UL NAS TRANSPORT (steps 3 and 7, Table 9.5.3.1.3.2-1)

| Derivation Path: 24.501 Table 8.2.10.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 |  |  |
| UL NAS TRANSPORT message identity |  | 01100111 | UL NAS TRANSPORT |  |
| Payload container type |  | 1100 | SLPP message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 3: Set according to Table 9.5.1.1.3.3-6 | SLPP Provide Capabilities |  |
|  |  | Step 7: Set according to Table 9.5.3.1.3.3-9 | SLPP Provide Location Information |  |
| Additional information |  | Present | The UE includes the Routing Identifier received in the Additional Information IE of the DOWNLINK GENERIC NAS TRANSPORT message (step 2 or 6 Table 9.5.1.4.3.2-2) |  |

Table 9.5.3.1.3.3-9: SLPP PROVIDE LOCATION INFORMATION (step 7, Table 9.5.3.1.3.2-1)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Not present |  |  |
| transactionID | (0..255) |  |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| sessionID | Not present |  |  |
| acknowledgement | Not present |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| commonIEsProvideLocationInformation | Not checked |  |  |
| commonSL-PRS-MethodsIEsProvideLocationInformation | Not checked |  |  |
| sl-AoA-ProvideLocationInformation | Present if UE supports target UE-assisted SL-AoA | OCTET STRING containing SL-AoA-ProvideLocationInformation as defined in Table 9.5.3.1.3.3-10 | pc_Target_UEA_SL_AoA |
| sl-RTT-ProvideLocationInformation SEQUENCE { | Present if UE supports target UE-assisted SL-RTT | OCTET STRING containing SL-RTT-ProvideLocationInformation as defined in Table 9.5.3.1.3.3-11 | pc_Target_UEA_SL_RTT |
| sl-TDOA-ProvideLocationInformation SEQUENCE { | Present if UE supports target UE-assisted SL-TDOA | OCTET STRING containing SL-TDOA-ProvideLocationInformation as defined in Table 9.5.3.1.3.3-12 | pc_Target_UEA_SL_TDOA |
| sl-TOA-ProvideLocationInformation SEQUENCE { | Present if UE supports target UE-assisted SL-TOA | OCTET STRING containing SL-TOA-ProvideLocationInformation as defined in Table 9.5.3.1.3.3-13 | pc_Target_UEA_SL_TOA |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |

Table 9.5.3.1.3.3-10: SL-AoA-ProvideLocationInformation

| Derivation Path: 38.355 clause 6.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-AoA-ProvideLocationInformation ::= SEQUENCE { |  |  |  |
| sl-AoA-SignalMeasurementInformation | Present. Any value acceptable |  |  |
| sl-AoA-Error | May be present |  |  |
| } |  |  |  |

Table 9.5.3.1.3.3-11: SL-RTT-ProvideLocationInformation

| Derivation Path: 38.355 clause 6.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-RTT-ProvideLocationInformation ::= SEQUENCE { |  |  |  |
| sl-RTT-SignalMeasurementInformation | Present. Any value acceptable |  |  |
| sl-RTT-Error | May be present |  |  |
| } |  |  |  |

Table 9.5.3.1.3.3-12: SL-TDOA-ProvideLocationInformation

| Derivation Path: 38.355 clause 6.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-TDOA-ProvideLocationInformation ::= SEQUENCE { |  |  |  |
| sl-RSTD-ReferenceUE-Info | Present. Any value acceptable |  |  |
| sl-TDOA-SignalMeasurementInformation | Present. Any value acceptable |  |  |
| sl-TDOA-Error | May be present |  |  |
| } |  |  |  |

Table 9.5.3.1.3.3-13: SL-TOA-ProvideLocationInformation

| Derivation Path: 38.355 clause 6.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-TOA-ProvideLocationInformation ::= SEQUENCE { |  |  |  |
| sl-TOA-SignalMeasurementInformation | Present. Any value acceptable |  |  |
| sl-TOA-Error | May be present |  |  |
| } |  |  |  |

9.5.3.2	SLPP Requested Method not Supported between UEs (Target UE is in coverage) - UE-Assisted

9.5.3.2.1	Test Purpose (TP)

(1)

with { 	a UE supporting at least one of target UE-assisted SL-RTT, target UE-assisted SL-AOA, target UE-assisted SL-TDOA or target UE-assisted SL-TOA but not all of them }

and with { Target UE is in coverage and having established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {
  when {	UE receives a SLPP message requesting at least one positioning method not supported }

then { the UE provides location information for the supported methods}

}

9.5.3.2.2	Conformance requirements

As defined in clause 9.5.3.1.2.

9.5.3.2.3	Test description

9.5.3.2.3.1	Pre-test conditions

System Simulator:

SS-NW

-	NR Cell 1.

-	System information combination NR-36 as defined in TS 38.508-1 [30] clause 4.4.3.1.3 is used in NR cell.

NR-SS-UE

-	NR-SS-UE1, NR-SS-UE2 (if required) and NR-SS-UE3 (if required) operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	All NR-SS-UEs uses GNSS as the synchronization reference source.

GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	For V2X capable UE, the UE is equipped with a USIM containing default values as per TS 38.508-1 [30] clause 4.8.3.3.3.

-	For 5G ProSe capable UE, the UE is equipped with a USIM containing default values as per TS 38.508-1 [30] clause 4.8.3.3.4.

-	UE is synchronised on GNSS.

Preamble:

-	The UE is in state 1N-B RRC_IDLE_with_SL and Test Mode (On) with UE test loop mode E as defined in TS 38.508-1 [30], subclause 4.4A using generic procedure parameter Sidelink (On), test mode (On) and GNSS Sync (On) as defined in TS 38.508-1 [30], subclause 4.5.1, using the following procedures:

-	for V2X capable UE, using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22;

-	for 5G ProSe capable UE, using UE initiated unicast mode ProSe Direct communication procedure in subclause 4.9.39.

Related PICS/PIXIT Statements:

-

9.5.3.2.3.2	Test procedure sequence

Table 9.5.3.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The NR-SS-UE1 sends a RESET UE POSITIONING STORED INFORMATION message. | <-- | RESET UE POSITIONING STORED INFORMATION | - | - |
| 2 | The NR-SS-UE1 sends a SLPP message of type Request Capabilities. | <-- | SLPP REQUEST CAPABILITIES | - | - |
| 3 | The UE sends a SLPP message of type Provide Capabilities including the UE positioning capabilities. | --> | SLPP PROVIDE CAPABILITIES | - | - |
| 4 | IF the UE SLPP message at step 3 includes an acknowledgment request THEN NR-SS-UE1 sends a SLPP Acknowledgement response. | <-- | SLPP ACKNOWLEDGEMENT | - | - |
| 5 | The NR-SS-UE1 sends a SLPP message of type Provide Assistance Data containing the data for all supported positioning methods. | <-- | SLPP PROVIDE ASSISTANCE DATA | - | - |
| 6 | The NR-SS-UE1 sends a SLPP message of type Request Location Information including all specified positioning methods. | <-- | SLPP REQUEST LOCATION INFORMATION | - | - |
| 7 | The UE sends a SLPP message of type Provide Location Information including information for the supported method(s). | --> | SLPP PROVIDE LOCATION INFORMATION | 1 | P |
| 8 | IF the UE SLPP message at step 7 includes an acknowledgment request THEN NR-SS-UE1 sends a SLPP Acknowledgement response. | <-- | SLPP ACKNOWLEDGEMENT | - | - |

9.5.3.2.3.3	Specific message contents

Table 9.5.3.2.3.3-1: RESET UE POSITIONING STORED INFORMATION

| Derivation Path: 38.509 clause 6.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE Positioning Technology |  | FFS | FFS |  |

Table 9.5.3.2.3.3-2: SLPP REQUEST CAPABILITIES (step 2, Table 9.5.3.2.3.2-1)

Derivation Path: Table 8.5-1 with UE-only mode condition.

Table 9.5.3.2.3.3-3: SLPP PROVIDE CAPABILITIES (step 3, Table 9.5.3.2.3.2-1)

| Derivation Path: Table 9.5.1.1.3.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 9.5.1.1.3.3-6 with the following exception: |  |  |  |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| sessionID | Present | Contains the same value of the transactionID field in step 2, Table9.5.3.2.3.1-2. |  |
| } |  |  |  |

Table 9.5.3.2.3.3-4: SLPP Acknowledgement (steps 4 and 8, Table 9.5.3.2.3.2-1)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Not present |  |  |
| transactionID | Not present |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Not present |  |  |
| sessionID | Not present |  |  |
| acknowledgement SEQUENCE { |  |  |  |
| ackRequested | FALSE |  |  |
| ackIndicator | (0..255) | Contains the same value of the sequenceNumber field in 3 or 7, Table 9.5.1.4.3.2-2. |  |
| } |  |  |  |
| slpp-MessageBody | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 9.5.3.2.3.3-5: SLPP PROVIDE ASSISTANCE DATA (step 5, Table 9.5.3.2.3.2-1)

| Derivation Path: Table 8.5-2 with UE-only mode condition. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 8.5-2 with the following exceptions: |  |  |  |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| sl-AoA-ProvideAssistanceData | Present if UE supports target UE-assisted SL-AoA. As defined in clause 8.5.1 |  | pc_Target_UEA_SL_AoA |
| sl-RTT-ProvideAssistanceData | Present if UE supports target UE-assisted SL-RTT. As defined in clause 8.5.1 |  | pc_Target_UEA_SL_RTT |
| sl-TDOA-ProvideAssistanceData | Present if UE supports target UE-assisted SL-TDOA. As defined in clause 8.5.1 |  | pc_Target_UEA_SL_TDOA |
| sl-TOA-ProvideAssistanceData | Present if UE supports target UE-assisted SL-TOA. As defined in clause 8.5.1 |  | pc_Target_UEA_SL_TOA |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.5.3.2.3.3-6: SLPP REQUEST LOCATION INFORMATION (step 6, Table 9.5.3.2.3.2-1)

| Derivation Path: Table 8.5-3 with UE-only mode condition. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 8.4-2 with the following exceptions: |  |  |  |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| sl-AoA-RequestLocationInformation | Present. | OCTET STRING containing SL-AoA-RequestLocationInformation as defined in Table 8.5-4 |  |
| sl-RTT-RequestLocationInformation | Present | OCTET STRING containing SL-RTT-RequestLocationInformation as defined in Table 8.5-5 |  |
| sl-TDOA-RequestLocationInformation | Present | OCTET STRING containing SL-TDOA-RequestLocationInformation as defined in Table 8.5-6 |  |
| sl-TOA-RequestLocationInformation | Present | OCTET STRING containing SL-TOA-RequestLocationInformation as defined in Table 8.5-7 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.5.3.2.3.3-7: SLPP PROVIDE LOCATION INFORMATION (step 7, Table 9.5.3.2.3.2-1)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Present. Any value acceptable |  |  |
| transactionID | (0..255) | Contains the same value of the transactionID field in step 6, Table 9.5.3.2.3.2-1. |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| sessionID | Present | Contains the same value of the transactionID field in step 2, Table 9.5.3.2.3.2-1. |  |
| acknowledgement | Not present |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| commonIEsProvideLocationInformation | Not checked |  |  |
| commonSL-PRS-MethodsIEsProvideLocationInformation | Not checked |  |  |
| sl-AoA-ProvideLocationInformation | Present if UE supports target UE-assisted SL-AoA | OCTET STRING containing SL-AoA-ProvideLocationInformation as defined in Table 9.5.3.1.3.3-10 | pc_Target_UEA_SL_AoA |
| sl-RTT-ProvideLocationInformation | Present if UE supports target UE-assisted SL-RTT | OCTET STRING containing SL-RTT-ProvideLocationInformation as defined in Table 9.5.3.1.3.3-11 | pc_Target_UEA_SL_RTT |
| sl-TDOA-ProvideLocationInformation | Present if UE supports target UE-assisted SL-TDOA | OCTET STRING containing SL-TDOA-ProvideLocationInformation as defined in Table 9.5.3.1.3.3-12 | pc_Target_UEA_SL_TDOA |
| sl-TOA-ProvideLocationInformation | Present if UE supports target UE-assisted SL-TOA | OCTET STRING containing SL-TOA-ProvideLocationInformation as defined in Table 9.5.3.1.3.3-13 | pc_Target_UEA_SL_TOA |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |

9.5.3.3	SLPP Requested Method not Supported between UEs (Target UE is out of coverage) - UE-Assisted

9.5.3.3.1	Test Purpose (TP)

(1)

with { 	a UE supporting at least one of target UE-assisted SL-RTT, target UE-assisted SL-AOA, target UE-assisted SL-TDOA or target UE-assisted SL-TOA but not all of them }

and with { Target UE is out of coverage and having established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {
  when {	UE receives a SLPP message requesting at least one positioning method not supported }

then { the UE provides location information for the supported methods}

}

9.5.3.3.2	Conformance requirements

As defined in clause 9.5.3.1.2.

9.5.3.3.3	Test description

9.5.3.3.3.1	Pre-test conditions

System Simulator:

NR-SS-UE

-	NR-SS-UE1, NR-SS-UE2 (if required) and NR-SS-UE3 (if required) operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	All NR-SS-UEs uses GNSS as the synchronization reference source.

GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	For V2X capable UE, the UE is equipped with a USIM containing default values as per TS 38.508-1 [30] clause 4.8.3.3.3.

Table 9.5.3.3.3.1-1: UE/ USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°119 (V2X) supported |  |
| EFVST |  | As per TS 38.508-1 [30] clause 4.8.3.3.3 |  |
| EFV2XP_PC5 |  | SL-PreconfigurationNR field as defined in TS 38.508-1 [30], table 4.10.1-1 with SLPP condition |  |

-	For 5G ProSe capable UE, the UE is equipped with a USIM containing default values as per TS 38.508-1 [30] clause 4.8.3.3.4 except for SL-PreconfigurationNR with SLPP condition.

Table 9.5.3.3.3.1-2: UE / USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°139 5G ProSe is "available" |  |
| EF5G_PROSE_ST |  | Service n°1, n°2 and n°4 are "available" |  |
| EF5G_PROSE_DD |  | SL-PreconfigurationNR field as defined in TS 38.508-1 [30], table 4.10.1-1 with SLPP condition |  |
| EF5G_PROSE_DC |  | SL-PreconfigurationNR field as defined in TS 38.508-1 [30], table 4.10.1-1 with SLPP condition |  |
| EF5G_PROSE_RU |  | SL-PreconfigurationNR field as defined in TS 38.508-1 [30], table 4.10.1-1 with SLPP condition |  |

-	UE is synchronised on GNSS.

Preamble:

-	The UE is in state 4-A and Test Mode (On), Test Loop Function (Off) as defined in TS 38.508-1 [30], Table 4.5.7.2-1 using generic procedure parameter Sidelink (On),Cast Type (Unicast), UE initiating unicast mode NR sidelink communication, GNSS Sync (On), using the following procedures:

-	for V2X capable UE, using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22;

-	for 5G ProSe capable UE, using UE initiated unicast mode ProSe Direct communication procedure in subclause 4.9.39.

Related PICS/PIXIT Statements:

-

9.5.3.3.3.2	Test procedure sequence

As defined in clause 9.5.3.2.3.2.

9.5.3.3.3.3	Specific message contents

As defined in clause 9.5.3.2.3.3.