3GPP TSG-RAN5 Meeting #108                                                 	                 R5-254852r1

# Bengaluru, India, 25th August 2025 – 29th August 2025

Agenda item:	5.3.29.10

Source: 	QUALCOMM Europe Inc. - Spain

Title: 	Discussion on multi-Rx measurement uncertainty

Document for:	Discussion and Endorsement

## 1.	Introduction

This paper presents our views on the status of multi-Rx MU/MTSU analysis as well as attempts to initiate the discussion regarding the MU/MTSU derivation for multi-Rx test setups.

## 2.	Discussion

The main differential aspect of the r18 multi-Rx WI is the UE ability of simultaneously receiving payload in two different antenna panels, each served by beams incoming from different AoAs [1]. To enable testability of this feature, RAN4 has added two new test setups (setup 5 and setup 6) to 38.133 specification [2].

### 2.1. Multi-Rx setups 5 and 6

Briefly, both setups 5 and 6 are defined for simultaneous reception with QCL Type-D (spatial quasi-colocation relationship) within the QZ. Setup 5 defines the testing scenario when 2 simultaneous AoAs are used, and Setup 6 defines the testing scenario when 1AoA + 2 simultaneous AoAs (total 3 AoAs) are used:

### A.3.15.5 Setup 5: 2 AoAs for simultaneous reception with QCL Type-D There are 2 active probes in the test. The DL signals, and noise if applicable, are transmitted from the two active probes. The 2 AoAs (AoA1 and AoA2) for simultaneous reception with different QCL-typeD are from the set of AoA pairs, denoted by (AoA1, AoA2) that can support 2 AoA reception for UE declared AoA angular separation and declared orientation in the positioner of the test system according to the spherical coverage requirement for simultaneous reception from multiple directions as defined in clause 7.3K.3 of TS 38.101-2 [19] for UE power class 3 supporting simultaneous reception from multiple directions. The angular separation between the directions (AoA1 and AoA2) of the 2 active probes is declared from table 7.3K.3-1 in clause 7.3K.3 of TS 38.101-2 [19] and shall not be changed for each test iteration. AoA1 needs to satisfy the spherical coverage requirement in table 7.3.4.3-1 of TS 38.101-2 [19]. NOTE: The chosen AoA pair (AoA1, AoA2) is up to RAN5.

Source: 38.133 [2]

### A.3.15.6 Setup 6: 3 AoAs for simultaneous reception with different QCL Type-D There are 3 active probes in the test and the DL signals and noise are transmitted from the three active probes. Out of the three AoAs, one AoA (AoA1) is aligned to a direction which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for UE power class 3 and other 2 AoAs (AoA2 and AoA3) are from the set of AoA pairs, denoted by (AoA2, AoA3) that can support 2 AoA reception for UE declared AoA angular separation and declared orientation in the positioner of the test system according to the spherical coverage requirement for simultaneous reception from multiple directions as defined in clause 7.3K.3 of TS 38.101-2 [19] for power class 3 supporting simultaneous reception from multiple directions. The UE positioning shall be such that the UE passes both spherical coverage requirements. The angular offset between the directions of the AoA pair (AoA2, AoA3) is declared from table 7.3K.3-1 in clause 7.3K.3 of TS 38.101-2 [19] and shall not be changed for each test iteration. NOTE: The chosen AoA pair (AoA2, AoA3) is up to RAN5.

Source: 38.133 [2]

There are multiple observations we can extract from these setup definitions:

- The multi-Rx AoA pairs (setup5: (AoA1, AoA2); setup6: (AoA2, AoA3)) suitable for multi-Rx reception are chosen from the set declared by the UE for that purpose.

  - These AoA pairs must satisfy the spherical coverage requirements defined in 38.101-2 [3] clause 7.3K.3.

- In addition, in setup 6, there is another AoA, AoA1, which must satisfy the spherical coverage requirements defined in 38.101-2 [3] clause 7.3.4.

  - The UE is positioned such that it simultaneously passes all corresponding spherical coverage requirements mentioned for all AoAs.

- The chosen AoA pair for testing is up to RAN5 discretion

Observation1: Suitable AoA pairs for multi-Rx reception are chosen from the set declared by the UE for that purpose.

Observation2: In setup 6, where multi-Rx AoA pair (AoA2, AoA3) coexists with a third AoA (AoA1), the UE shall be positioned such that all AoAs pass their corresponding spherical coverage requirements.

Observation3: The chosen AoA pair for testing is up to RAN5 discretion.

From 38.101-2 [3] clause 7.3K.3 we read the following – the spherical coverage requirement for simultaneous reception is defined as the probability of successful rank2 PDSCH reception (clause 7.3K.0 of [3]), with the probability defined in Table 7.3K.3-1.

In addition, clause 7.3K.0 of [3] specifies that successful rank2 PDSCH reception for a given AoA is similar to that defined for a single AoA (clause 7.3.4).

Observation4: Successful rank2 PDSCH definition for a given AoA is similar to that defined for a single AoA.

### 7.3K.3 2AoA spherical coverage of power class 3 The requirements apply to the UE when tested in a test system as described in Annex L. The requirement is verified with the test metric of throughput (Link= 2AoA spherical coverage grid, Meas=Link Angle). The spherical coverage requirement for simultaneous reception from multiple directions is defined in terms of the probability to support simultaneous reception of rank 2 PDSCH defined in sub-clause 7.3K.0. The probability (see Annex L) is defined as the spatial average over the full sphere around the UE of the probability of any one direction to support 2 AoA reception. In the applicable test system (see Annex L), the probability of any one direction of the UE to support 2 AoA reception for any specific AoA separation is the ratio of the number of unique AoA pairs that include that direction and can support 2 AoA reception to the total number of verified unique AoA pairs that include that direction. The requirement applies only for the UE’s declared orientation in the positioner of the test system. The requirement for each AoA separation condition applies only for the UE’s declared orientation in the positioner of the test system for that AoA separation. The minimum required overall probability to support 2 AoA reception for power class 3 UEs for any channel bandwidth is specified by AoA separation in table 7.3K.3-1. The UE is only required to fulfil the requirement at any one of AoA separations declared from Table 7.3K.3-1. Table 7.3K.3-1: Requirement for power class 3 AoA separation (degrees); Probability (%). 30; 18.5. 60; 13.5. 90; 12.5. 120; 20.5. 150; 28.5.

Source: 38.101-2 [3]

### 7.3K.0 General For this release, the requirement applies only to FR2-1 UEs that support the following set of capabilities: 1. simultaneousReceptionDiffTypeD-r16 2. At least one of: a. singleDCI-SDM-scheme-r16 or b. multiDCI-MultiTRP-r16 and either of: i. overlapPDSCHsFullyFreqTime-r16. ii. overlapPDSCHsInTimePartiallyFreq-r16 The requirement applies for simultaneous reception of rank 2 PDSCH, where each layer uses overlapping RBs in both time and frequency and is associated with a unique TCI state and AoA. The scheduled TCI states for the rank 2 PDSCH shall be configured with different QCL type-D reference signals respectively. The DL power at the center of quiet zone from each AoA equals the EIS spherical coverage requirement from sub-clause 7.3.4. [ . . . ]

Source: 38.101-2 [3]

Furthermore, the requirements in clause 7.3.4 of [3] are the ones that apply to spherical coverage AoAs in general. In other words, we can conclude that the UE needs to simultaneously satisfy the spherical coverage requirements in clause 7.3.4 of [3] for each of the AoAs involved in the test (whether they are multi-Rx or not).

Conclusion1: The UE shall simultaneously satisfy the spherical coverage requirements for all AoAs involved in the test, defined in 7.3.4 of 38.101-2 [3].

Conclusion2: In addition, for the pairs of AoAs where multi-Rx is to be tested, the UE needs to declare specific angular support.

### 2.2. Measurement Uncertainty

Given what we have so far analyzed, it seems logical to take the existing MU/MTSU definition defined in 38.903 [4] for 2AoA as a starting point in this discussion.

From 38.871 [5], we note that RAN4 has done some initial work on this front, specifically on enhanced IFF setups, however we highlight the following note to Table A.2-1:

NOTE 1: The values from Enhanced IFF seem rather optimistic for MultiRX and should be further updated with the considerations of 2AoA impact

Source: 38.871 [5] Table A.2-1

Pasting below the uncertainty assessment table for 2AoA IFF setup for RRM scenarios from TR 38.903

Table E.3.1.4-2: Uncertainty assessment for fDL AWGN absolute power or wanted DL signal absolute power (f=23.45GHz, 32.125GHz, 40.8GHz, Quiet Zone size ≤ 30 cm) for PC3

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.00 | Normal | 2.00 | 0.00 |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | 0.00 |
| 3 | Quality of Quiet Zone (NOTE 4) | 0.7 | Actual | 1.00 | 0.7 |
| 4 | Mismatch | 1.30 | Actual | 1.00 | 1.30 |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 6 | gNB uncertainty on absolute level | 2.9 | Normal | 2.00 | 1.45 |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | 0.00 |
| 8 | Amplifier uncertainties | 2.1 | Normal | 2.00 | 1.05 |
| 9 | Random uncertainty | 0.50 | Normal | 2.00 | 0.25 |
| 10 | Influence of the XPD | 0.01 | U-shaped | 1.41 | 0.00 |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | 0.00 |
| 13 | Multiple measurement antenna uncertainty (NOTE 3) | 0.15 | Actual | 1.00 | 0.15 |
| 14 | DUT repositioning | 0.08 | Rectangular | 1.73 | 0.05 |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 17 | Mismatch | 0.00 | U-shaped | 1.41 | 0.00 |
| 18 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | 0.00 |
| 19 | Misalignment of positioning System | 0.00 | Normal | 2.00 | 0.00 |
| 20 | Uncertainty of the Network Analyzer | 0.73 | Normal | 2.00 | 0.37 |
| 21 | Uncertainty of the absolute gain of the calibration antenna | 0.60 | Normal | 2.00 | 0.30 |
| 22 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.01 | Rectangular | 1.73 | 0.00 |
| 23 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | 0.00 |
| 24 | Quality of quiet zone for calibration process (NOTE 4) | 0.4 | Actual | 1.00 | 0.4 |
| 25 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 26 | Influence of the calibration antenna feed cable | 0.14 | Normal | 2.00 | 0.07 |
| 27 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
|  | Systematic uncertainties (NOTE 2) |  |  |  | Value |
| 28 | Systematic error related to beam peak search |  |  |  | 0.5 |
| Total measurement uncertainty |  |  |  |  | Value |
| DL AWGN absolute power expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | 5.25 |
| NOTE 1: The analysis was done only for the case of operating in-band, non-CA. NOTE 2: In order to obtain the total measurement uncertainty, systematic uncertainties have to be added to the expanded root sum square of the standard deviations of the Stage 1 and Stage 2 contributors. NOTE 3: Applies to the system which has a structure of mechanical feed antenna positioning. NOTE 4: Value based on procedure defined in Annex M of TR 38.521-2 [7] for Quiet Zone size less or equal to 30 cm. NOTE 5: The values in this table have been derived for DL powers above and equal to REFSENS. The values might need to be revisited for power levels below REFSENS |  |  |  |  |  |

Pasting the RAN4 assessment of MU for MultiRx RRM scenarios for Enhanced IFF setup from TR 38.871

Table A.2-1: Uncertainty assessment for RRM testing with IFF

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.00 | Normal | 2.00 | [0.00] |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | [0.00] |
| 3 | Quality of Quiet Zone (NOTE 1) | 0.7 | Actual | 1.00 | [0.7] |
| 4 | Mismatch | 1.30 | Actual | 1.00 | [1.30] |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | [0.00] |
| 6 | gNB uncertainty on absolute level | 2.9 | Normal | 2.00 | [1.45] |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | [0.00] |
| 8 | Amplifier uncertainties | 2.1 | Normal | 2.00 | [1.05] |
| 9 | Random uncertainty | 0.50 | Normal | 2.00 | [0.25] |
| 10 | Influence of the XPD | 0.01 | U-shaped | 1.41 | [0.00] |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | [0.00] |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | [0.00] |
| 13 | Multiple measurement antenna uncertainty | 0.15 | Actual | 1.00 | [0.15] |
| 14 | DUT repositioning | 0.08 | Rectangular | 1.73 | [0.05] |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 15 | Mismatch | 0.00 | U-shaped | 1.41 | [0.00] |
| 16 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | [0.00] |
| 17 | Misalignment of positioning System | 0.00 | Normal | 2.00 | [0.00] |
| 18 | Uncertainty of the Network Analyzer | 0.73 | Normal | 2.00 | [0.37] |
| 19 | Uncertainty of the absolute gain of the calibration antenna | 0.60 | Normal | 2.00 | [0.30] |
| 20 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.01 | Rectangular | 1.73 | [0.00] |
| 21 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | [0.00] |
| 22 | Quality of quiet zone for calibration process (NOTE 1) | 0.4 | Actual | 1.00 | [0.4] |
| 23 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | [0.00] |
| 24 | Influence of the calibration antenna feed cable | 0.14 | Normal | 2.00 | [0.07] |
| 25 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | [0.00] |
|  | Systematic uncertainties (NOTE 2) |  |  |  | Value |
| Total measurement uncertainty |  |  |  |  | Value |
| DL AWGN absolute power expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [4.75] |
| NOTE 1: The values from Enhanced IFF seem rather optimistic for MultiRX and should be further updated with the considerations of 2AoA impact |  |  |  |  |  |

Comparing the 2 tables, we see that out of the total 29 MU terms, 25 of them are not changing.

Table 2.2-1:	Listing down the actual MU terms not changing

| 1 | Positioning misalignment |
| --- | --- |
| 2 | Measure distance uncertainty |
| 3 | Quality of Quiet Zone (NOTE 4) |
| 4 | Mismatch |
| 5 | Standing wave between the DUT and measurement antenna |
| 6 | gNB uncertainty on absolute level |
| 7 | Phase curvature |
| 8 | Amplifier uncertainties |
| 9 | Random uncertainty |
| 10 | Influence of the XPD |
| 11 | Insertion Loss Variation |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) |
| 13 | Multiple measurement antenna uncertainty (NOTE 3) |

| 15 | Mismatch |
| --- | --- |
| 16 | Amplifier Uncertainties |
| 17 | Misalignment of positioning System |
| 19 | Uncertainty of the absolute gain of the calibration antenna |
| 20 | Positioning and pointing misalignment between the reference antenna and the measurement antenna |
| 21 | Phase centre offset of calibration antenna |
| 22 | Quality of quiet zone for calibration process (NOTE 4) |
| 23 | Standing wave between reference calibration antenna and measurement antenna |
| 24 | Influence of the calibration antenna feed cable |
| 25 | Insertion Loss Variation |

Observation 6: The 25 MU values listed in table 2.2-1 can be re-used as working assumption for PC3 for FR2 MultiRx RRM

Below are the MU terms from RAN4 provided table that are either missing or changing w.r.t 2AoA IFF setup for FR2 RRM.

| 14 | DUT repositioning |
| --- | --- |
| 15 | Influence of spherical coverage grid |
| 19 | Uncertainty of the Network Analyzer |
| 28 | Systematic error related to beam peak search |

Since the FR2 MultiRx RRM do not require beam peak search, the MU term ‘Systematic error related to beam peak search’ is not relevant.

Observation 7: MU term ‘Systematic error related to beam peak search’ is not needed to be considered for the FR2 MultiRx RRM test cases

The 3 MU terms that requires RAN5 to further analyze the preliminary assessment done by RAN4 are ‘DUT repositioning’, ‘influence of spherical coverage grid’ and ‘uncertainty of the network analyzer’.

Proposal1: VOID.

Proposal 2a: reuse the 25 MU terms mentioned in table 2.2-1 and their associated values for FR2 MultiRx RRM scenarios as a working assumption for PC3.

Proposal 2b: The MU terms DUT repositioning’,‘influence of spherical coverage grid’ and ‘uncertainty of the network analyzer’ are identified as the ones pending further analysis from TE vendors. Additionally, conducted MU elements such as mismatch, amplifier uncertainty needs further analysis to determine its applicability to 3AoA FR2 MultiRx setup

## 3.	Conclusion

The following observations and proposals have been raised from this discussion:

Observation1: Suitable AoA pairs for multi-Rx reception are chosen from the set declared by the UE for that purpose.

Observation2: In setup 6, where multi-Rx AoA pair (AoA2, AoA3) coexists with a third AoA (AoA1), the UE shall be positioned such that all AoAs pass their corresponding spherical coverage requirements.

Observation3: The chosen AoA pair for testing is up to RAN5 discretion.

Observation4: Successful rank2 PDSCH definition for a given AoA is similar to that defined for a single AoA.

Observation 6: The 25 MU values listed in table 2.2-1 can be re-used for FR2 MultiRx RRM

Observation 7: MU term ‘Systematic error related to beam peak search’ is not needed to be considered as working assumption for PC3 for the FR2 MultiRx RRM test cases

Proposal 2a: reuse the 25 MU terms mentioned in table 2.2-1 and their associated values for FR2 MultiRx RRM scenarios as a working assumption for PC3

Proposal 2b: The MU terms DUT repositioning’, ‘influence of spherical coverage grid’ and ‘uncertainty of the network analyzer’ are identified as the ones pending further analysis from TE vendors. Additionally, conducted MU elements such as mismatch, amplifier uncertainty needs further analysis to determine its applicability to 3AoA FR2 MultiRx setup

## 4.		References

[1] RP-241653: “New WID on UE Conformance - Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception”, Qualcomm.

[2] 3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[3] 3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”

[4] 3GPP TR 38.903: “NR; Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”

[5] 3GPP TR 38.871: “Study on NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements”

[6] 3GPP TS 38.533: "NR; User Equipment (UE) conformance specification; Radio Resource Management (RRM)".

[7] 3GPP TS 38.521-4: “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Performance”

## 5.		Annex

### 5.1 Measurement uncertainty budget for RRM testing defined in TR 38.871 [5], enhanced IFF

Table A.2-1: Uncertainty assessment for RRM testing with IFF

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.00 | Normal | 2.00 | [0.00] |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | [0.00] |
| 3 | Quality of Quiet Zone (NOTE 1) | 0.7 | Actual | 1.00 | [0.7] |
| 4 | Mismatch | 1.30 | Actual | 1.00 | [1.30] |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | [0.00] |
| 6 | gNB uncertainty on absolute level | 2.9 | Normal | 2.00 | [1.45] |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | [0.00] |
| 8 | Amplifier uncertainties | 2.1 | Normal | 2.00 | [1.05] |
| 9 | Random uncertainty | 0.50 | Normal | 2.00 | [0.25] |
| 10 | Influence of the XPD | 0.01 | U-shaped | 1.41 | [0.00] |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | [0.00] |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | [0.00] |
| 13 | Multiple measurement antenna uncertainty | 0.15 | Actual | 1.00 | [0.15] |
| 14 | DUT repositioning | 0.08 | Rectangular | 1.73 | [0.05] |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 15 | Mismatch | 0.00 | U-shaped | 1.41 | [0.00] |
| 16 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | [0.00] |
| 17 | Misalignment of positioning System | 0.00 | Normal | 2.00 | [0.00] |
| 18 | Uncertainty of the Network Analyzer | 0.73 | Normal | 2.00 | [0.37] |
| 19 | Uncertainty of the absolute gain of the calibration antenna | 0.60 | Normal | 2.00 | [0.30] |
| 20 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.01 | Rectangular | 1.73 | [0.00] |
| 21 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | [0.00] |
| 22 | Quality of quiet zone for calibration process (NOTE 1) | 0.4 | Actual | 1.00 | [0.4] |
| 23 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | [0.00] |
| 24 | Influence of the calibration antenna feed cable | 0.14 | Normal | 2.00 | [0.07] |
| 25 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | [0.00] |
|  | Systematic uncertainties (NOTE 2) |  |  |  | Value |
| Total measurement uncertainty |  |  |  |  | Value |
| DL AWGN absolute power expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [4.75] |
| NOTE 1: The values from Enhanced IFF seem rather optimistic for MultiRX and should be further updated with the considerations of 2AoA impact |  |  |  |  |  |

### 5.2 Measurement uncertainty budget for RRM testing defined in TR 38.903 [4], enhanced IFF

Table E.3.1.4-2: Uncertainty assessment for fDL AWGN absolute power or wanted DL signal absolute power (f=23.45GHz, 32.125GHz, 40.8GHz, Quiet Zone size ≤ 30 cm) for PC3

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.00 | Normal | 2.00 | 0.00 |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | 0.00 |
| 3 | Quality of Quiet Zone (NOTE 4) | 0.7 | Actual | 1.00 | 0.7 |
| 4 | Mismatch | 1.30 | Actual | 1.00 | 1.30 |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 6 | gNB uncertainty on absolute level | 2.9 | Normal | 2.00 | 1.45 |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | 0.00 |
| 8 | Amplifier uncertainties | 2.1 | Normal | 2.00 | 1.05 |
| 9 | Random uncertainty | 0.50 | Normal | 2.00 | 0.25 |
| 10 | Influence of the XPD | 0.01 | U-shaped | 1.41 | 0.00 |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | 0.00 |
| 13 | Multiple measurement antenna uncertainty (NOTE 3) | 0.15 | Actual | 1.00 | 0.15 |
| 14 | DUT repositioning | 0.08 | Rectangular | 1.73 | 0.05 |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 17 | Mismatch | 0.00 | U-shaped | 1.41 | 0.00 |
| 18 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | 0.00 |
| 19 | Misalignment of positioning System | 0.00 | Normal | 2.00 | 0.00 |
| 20 | Uncertainty of the Network Analyzer | 0.73 | Normal | 2.00 | 0.37 |
| 21 | Uncertainty of the absolute gain of the calibration antenna | 0.60 | Normal | 2.00 | 0.30 |
| 22 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.01 | Rectangular | 1.73 | 0.00 |
| 23 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | 0.00 |
| 24 | Quality of quiet zone for calibration process (NOTE 4) | 0.4 | Actual | 1.00 | 0.4 |
| 25 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 26 | Influence of the calibration antenna feed cable | 0.14 | Normal | 2.00 | 0.07 |
| 27 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
|  | Systematic uncertainties (NOTE 2) |  |  |  | Value |
| 28 | Systematic error related to beam peak search |  |  |  | 0.5 |
| Total measurement uncertainty |  |  |  |  | Value |
| DL AWGN absolute power expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | 5.25 |
| NOTE 1: The analysis was done only for the case of operating in-band, non-CA. NOTE 2: In order to obtain the total measurement uncertainty, systematic uncertainties have to be added to the expanded root sum square of the standard deviations of the Stage 1 and Stage 2 contributors. NOTE 3: Applies to the system which has a structure of mechanical feed antenna positioning. NOTE 4: Value based on procedure defined in Annex M of TR 38.521-2 [7] for Quiet Zone size less or equal to 30 cm. NOTE 5: The values in this table have been derived for DL powers above and equal to REFSENS. The values might need to be revisited for power levels below REFSENS |  |  |  |  |  |