3GPP TSG-RAN5 Meeting #109	R5-255741

# Dallas, US, 17th – 21st November, 2025

Agenda item:	5.4.20

Source:	Keysight Technologies UK Ltd

Title:	EN-DC FR1 intra-band contiguous - REFSENS exceptions considerations for AP108.22

Document for:	Discussion and Endorsement

# Introduction

This contribution analyses the RAN4 recommendations and concerns made to RAN5 via LS [1] regarding testing REFSENS for EN-DC for intra-band contiguous case where exceptions apply. For convenience, the content of [1] is copied below.

RAN4 would like to share with RAN5 the agreements reached to address concerns that some E-UTRA uplink configurations specified as “N/A” for the intra-band contiguous EN-DC MSD requirements may be problematic for conformance testing. An example of such uplink configuration is illustrated in Table 1 for DC_(n)3AA. Table 1: Example of E-UTRA “N/A” uplink configuration for intra-band contiguous EN-DC MSD - Table 7.3B.2.1-1. EN-DC configuration; E-UTRA/NR band; FC (UL) (MHz); Channel bandwidth (MHz); UL allocation (LCRB); FC (DL) (MHz); MSD (dB); Duplex mode. DC_(n)3AA; 3; N/A; 15; N/A; 1842.5; 6.2; FDD. n3; 1770.0; 30; 10 (RBstart = 150); 1865.0; N/A. RAN4 has made the following agreements: - E-UTRA uplink carrier frequency “Fc (UL)”: RAN4 recommends configuring the E-UTRA Fc (UL) according to the E-UTRA band’s default Tx-Rx frequency separation specified in TS 36.101 Table 5.7.4-1; - E-UTRA uplink RB allocation “Lcrb”: RAN4 recommends configuring the E-UTRA UL RB allocation according to the UL configuration specified in TS 36.101 Table 7.3.1-2 of the corresponding E-UTRA DL CBW; - E-UTRA transmission power: RAN4 recommends configuring the E-UTRA transmit power as anchor agnostic, or preferably at minimum output power.

# Discussion

The RAN4 concerns are only for the intra-band contiguous cases where “E-UTRA uplink configurations specified as “N/A” for the intra-band contiguous EN-DC MSD requirements may be problematic for conformance testing”. If we look into the table in RAN4 core specs where those REFSENS cases are defined, Table 7.3B.2.1-1 in [2], copied below for convenience, it can be observed that, when N/A is defined for both E-UTRA UL frequency and E-UTRA UL allocation, no sensitivity degradation (MSD) is expected/specified in the NR DL carrier. What could be read in multiple ways:

- E-UTRA UL should not be configured at all to transmit PUSCH

- or

- No matter what happens/is configured in E-UTRA UL, REFSENS is NR carrier should not be degraded.

In the first interpretation it must be noted that to allow testing MSD on E-EUTRA carrier due to specific NR UL configuration, the E-EUTRA carrier will be transmitting PUCCH in UL.

The first interpretation is what is followed in RAN5 so PUCCH in E-UTRA UL is transmitted.  Assuming that what can be problematic is the E-UTRA UL power, it must be noted that by default, open loop power control applies. Considering the PUCCH power control formula defined in TS 36.213 [5], the default power control related message contents applicable for RF conformance testing in TS 36.508 [4], the default LTE DL power required at DUT connector in TS 38.521-3[3] and the open loop power control tolerance in extreme conditions of +/-12dB, the expected PUCCH UL power is about -117dBm+(18dBm-(-85dBm))+0dB+0dB +/-12dB = -13dBm+/-12dB, that is ≤ -1dBm. RAN4 should confirm whether in these conditions the current test implementation is considered as problematic for conformance testing.

Table 7.3B.2.1-1: Reference sensitivity (MSD) for intra-band contiguous EN-DC [2] EN-DC configuration / channel allocations /MSD. EN-DC configuration; E-UTRA/NR band; FC (UL) (MHz); Channel bandwidth (MHz); UL allocation (LCRB); FC (DL) (MHz); MSD (dB); Duplex mode. DC_(n)3AA; 3; N/A; 15; N/A; 1842.5; 6.2; FDD. n3; 1770.0; 30; 10 (RBstart = 150); 1865.0; N/A. DC_(n)5AA; 5; N/A; 5; N/A; 871.5; 5.2; FDD. n5; 839; 20; 20 (RBend = 105); 884; N/A. DC_(n)12AA; 12; N/A; 5; N/A; 733.5; 4.5; FDD. n12; 711; 10; 20 (RBend = 51); 741; N/A. DC_(n)71AA; 71; 668; 10; 10 (RBstart = 0); 622; 17.2; FDD. n71; 678; 10; 10 (RBend = 51); 632; 29.4. DC_(n)71AA4; 71; 668; 10; 10 (RBstart = 0); 622; 17.2. n71; 678; 101; 10 (RBend = 51); 634.5; 29.1. DC_(n)71AA3; 71; N/A; 10; N/A; 642.0; 21.3. n71; 673.0; 20; 5 (RBstart = 0); 627.0; N/A. DC_(n)71AA4; 71; N/A; 5; N/A; 639.5; 6.8. n71; 670.5; 151; 5 (RBstart = 2); 627.0; N/A. DC_(n)71AA5; 71; N/A; 5; N/A; 634.5; 6.4. n71; 670.5; 15; 5 (RBstart = 0); 624.5; N/A. NOTE 1: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. NOTE 3: Applicable only to BCS 2. NOTE 4: Applicable only to BCS 1. NOTE 5: Applicable only to BCS 0..

In addition, in previous table it can be observed that only FDD bands are impacted.

However, in the LS [1] RAN4 is recommending to use a specific allocation to the E-UTRA UL carrier. With such a recommendation from RAN4 to RAN5, RAN4 should be asked why that is not explicitly captured in RAN4 specs.

It is also unclear what RAN4 considers “may be problematic for conformance testing”. So that should be clarified as well by RAN4 so RAN5 can understand better how to proceed.

In the final bullet of [1], RAN4 is making 2 recommendations, with a preference on the second one:

- RAN4 recommends configuring the E-UTRA transmit power as anchor agnostic,

- or preferably at minimum output power.

Regarding first proposal, the only way to achieve E-UTRA transmit power as in anchor agnostic approach is by not scheduling UL nor DL in the E-UTRA carrier, as so is defined the anchor agnostic configuration of the EUTRA carrier in RAN5 specs [3], Table 4.6-1, row 6:

Table 4.6-1: E-UTRA configuration for EN-DC FR1 tests applying anchor agnostic approach Parameter; Value; Comments. Test Frequency during and after connection setup; Mid (See Table 4.6-2); As defined in TS 36.508 for the LTE band under test. Bandwidth during and after connection setup; 5 MHz (See Table 4.6-2); Supported by all LTE bands.. DL signal levels during connection setup; RS EPRE -85.0 dBm/15kHz; DL physical channels as defined in Annex C0, C.1, C.2 and Annex C.3 of TS 36.521-1 [10]. TS 36.521-1 [10] annex C.0 defines the default DL power level of RS EPRE to be -85dBm/15kHz.. UL Signal levels during connection setup; PUSCH Power; Attained by enabling open loop power control and setting up UL signal levels according to Annex H.0, H.2 and H.3 of TS 36.521-1 [10].. DL/UL RMC after connection setup; 0 RB allocation on both DL and UL (see Table 4.6-2); Once the LTE link is established, then LTE Tx can be restricted by configuring 0 RB allocation on DL and UL. TimeAlignmentTimerDedicated IE to be set to infinity to ensure UE doesn't look for TA adjustments (See Table 4.6-5). CQI Reports and SRS after connection setup; Disabled (See Table 4.6-3 and 4.6-4); Disable periodic and aperiodic CQI reports to ensure none of these transmissions occur on the LTE uplink. Since LTE transmissions could easily exceed spurious emissions limits, tests that are intended to measure RF parameters on the NR should simply avoid LTE transmit altogether.. Number of OFDM symbols for PDCCH; 3; The PCFICH carries information about the number of OFDM symbols used for transmission of PDCCHs in a subframe, as specified in TS 36.211 [8] clause 6.7.

However, as previously commented, that is not possible because DL allocation is required in the E-UTRA carrier in order to test the MSD that, as a result, will drive into PUCCH transmission in E-UTRA UL. In addition, this proposal contradicts with RAN4 second bullet recommendation of setting an UL PUSCH RB allocation other than 0.

Regarding the second and RAN4 preferred proposal, made in RAN4 LS bullet 3, of setting E-UTRA UL to minimum output power, that does may drive into test implementation issues. As captured in RAN4 core specs Table 7.3B.2.1-1 in [2], Note 2, by default UL should be set to Pmax. If NR carrier is set to Pmax while E-UTRA carrier is set to its minimum output power, as both carriers will be using the same DUT connector, that will drive into testability issues in the test equipment, high NR UL power will make E-UTRA UL low power signal undecodable because the test equipment will be saturated with the higher power.

Therefore, with all the concerns and inconsistences presented above and while it is not clear what exactly in the E-UTRA UL configuration “may be problematic for conformance testing” neither if current expected E-UTRA UL power in current test implementation is considered problematic, in this document 2 alternatives are proposed:

- Use close loop power control to fix E-UTRA UL power to a specific target power.

  - In such a case, RAN4 to confirm the E-UTRA target UL power and whether E-UTRA PUSCH must be scheduled (otherwise the power control will apply to PUCCH rather than PUSCH).

- Schedule no UL, so only PUCCH will be transmitted in E-UTRA UL, use an E-UTRA DL scheduler only scheduling alternate subframes and with this configuration the REFSENS in NR carrier will only be measured in slots not overlapping in time with the E-UTRA PUCCH transmission and then removing any possibility of impacting the test result.

  - Noting that the cases under analysis only apply to FDD bands, the impact in testing should be quite limited, as the resulting test time duration should be comparable to a TDD band case.

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1. The case under analysis affects only testing of certain REFSENS FDD bands operating in ENDC intra-band contiguous where sensitivity exceptions apply.

Observation 2. It is unclear what exactly RAN4 consider “may be problematic for conformance testing” when “E-UTRA uplink configurations specified as ‘N/A’”.

Observation 3. In current test implementation, there is no UL RB allocation in E-UTRA carrier, making the UE transmit only PUCCH on E-UTRA UL to send the HARQ feedback to the DL transmissions. The expected E-UTRA UL power is -1dBm.

Observation 4. Second recommendation in RAN4 LS about defining a specific non-zero UL allocation of the E-UTRA carrier contradicts with the third recommendation of using the anchor agnostic approach considering how anchor agnostic configuration of the E-UTRA carrier is defined in RAN5 specs.

Observation 5. Based on how anchor agnostic configuration of the E-UTRA carrier is defined in RAN5 specs, this is not an option in this test case. Such a definition implies zero RB allocation in DL, what would not allow testing MSD for REFSENS.

Observation 6. The preferred RAN4 recommendation in third bullet point of the LS, to set UL to minimum output power, will imply testability issues. NR UL and E-UTRA UL will share the DUT connector. NR UL power is required to be set to Pmax what will saturate the test equipment reception at that DUT connector making the E-UTRA UL low power signal undecodable.

Proposal 1. RAN4 to confirm if expected E-UTRA UL power (-1dBm) in current test implementation is problematic.

Proposal 2. If confirmed current test implementation and expected E-UTRA UL power has an issue, RAN4 to select between the following test implementation alternatives to reduce the potential issue due to E-UTRA UL power:

- Use close loop power control to fix E-UTRA UL power to a specific target power, PTarget_E-UTRA_UL.

  - In such a case, RAN4 to confirm the PTarget_E-UTRA_UL and whether E-UTRA PUSCH must be scheduled or not.

  - Based on Observation 6, this alternative would not be valid if required PTarget_E-UTRA_UL is close to the minimum output power.

- Schedule no UL, so only PUCCH will be transmitted in E-UTRA UL, use an E-UTRA DL scheduler only scheduling alternate subframes and with this configuration the REFSENS in NR carrier will only be measured in slots not overlapping in time with the E-UTRA PUCCH transmission.

Proposal 3. Send LS to RAN4 to share previous observations and proposals agreed in RAN5.

A draft of the LS response is available in the Annex.

# References

- R5-253716, “LS on E-UTRA uplink configurations for intra-band ENDC MSD”, RAN4, RAN5#108

- TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios", v19.3.0 (2025-09)

- TS 38.521-3, “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”, v19.1.0 (2025-09)

- TS 36.508, “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common test environments for User Equipment (UE) conformance testing”, v18.10.0 (2025-09)

- TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures", v18.3.0 (2024-12)

# Annex : Draft LS proposal to RAN4

3GPP TSG-RAN5 Meeting #109		R5-25XXXX

Dallas, US, 17th - 21th November, 2025

Title:	Reply LS on E-UTRA uplink configurations for intra-band ENDC MSD

Response to:	R5-253716 / R4-2507704

Source:	TSG RAN WG5

To:	TSG RAN WG4

Cc:

Contact Person:

#### Name:	Adan Toril

E-mail Address:	adan_toril@keysight.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

In LS [1], for testing REFSENS for EN-DC for intra-band contiguous case where MSD exceptions apply and E-UTRA UL configuration is set to “N/A”, RAN4 shared with RAN5 several recommendations on how to configure E-UTRA UL in order to avoid potential problematic conformance testing. RAN5 has evaluated those recommendations and conclude in [2] the following observations:

- In current test implementation, there is no UL RB allocation in E-UTRA carrier, making the UE transmit only PUCCH on E-UTRA UL to send the HARQ feedback to the DL transmissions. The expected E-UTRA UL power is ≤-1dBm.

- It is unclear if such an expected E-UTRA UL power can be problematic.

- Second recommendation in RAN4 LS [1] about defining a specific non-zero UL allocation of the E-UTRA carrier contradicts with the third recommendation of using the anchor agnostic approach considering how anchor agnostic configuration of the E-UTRA carrier is defined in RAN5 specs[4].

- Based on how anchor agnostic configuration of the E-UTRA carrier is defined in RAN5 specs [4], this is not an option in this test case. Such a definition implies zero RB allocation in DL, what would not allow testing MSD for REFSENS.

- The preferred RAN4 recommendation in third bullet point of the LS [1], to set UL to minimum output power, will imply testability issues. NR UL and E-UTRA UL will share the DUT connector. NR UL power is required to be set to Pmax what will saturate the test equipment reception at that DUT connector making the E-UTRA UL low power signal undecodable.

Based on previous observations, while it is unclear if current test implementation and expected E-UTRA UL power is or not problematic, RAN5 has evaluated 2 implementation alternatives to avoid any potential impact of the E-UTRA UL power in the test:

- Use close loop power control to fix E-UTRA UL power to a specific target power, PTarget_E-UTRA_UL.

  - In such a case, RAN4 to confirm the PTarget_E-UTRA_UL and whether E-UTRA PUSCH must be scheduled or not.

  - This alternative would not be valid if required PTarget_E-UTRA_UL is close to the minimum output power.

- Schedule no UL, so only PUCCH will be transmitted in E-UTRA UL, use an E-UTRA DL scheduler only scheduling alternate subframes and with this configuration the REFSENS in NR carrier will only be measured in slots not overlapping in time with the E-UTRA PUCCH transmission.

ACTION:

RAN5 respectfully asks RAN4 to take the above into consideration, responding to the following questions:

Q.1. RAN4 to confirm if current test implementation where PUCCH is transmitted in E-UTRA UL with expected power ≤-1dBm is problematic to verify the test requirements.

Q.2. If the answer to Q.1. is affirmative, RAN4 to indicate if there is any preferred option between the 2 alternatives A and B proposed by RAN5 above, providing also details on the E-UTRA UL channel and the required E-UTRA UL power in case of option A.

2. References:

- R5-253716, “LS on E-UTRA uplink configurations for intra-band ENDC MSD”, RAN4, RAN5#108

- R5-25xxxx, “EN-DC FR1 intra-band contiguous - REFSENS exceptions considerations for AP108.22”, Keysight, RAN5#109

- TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios", v19.3.0 (2025-09)

- TS 38.521-3, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”, v19.1.0 (2025-09)