3GPP TSG-RAN5 Meeting #109	R5-255956

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0105 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of band 252 into MOP TC for category M1 and NB-IoT NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_FDD_S_band-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The MOP requirement for category M1 and NB-IoT NTN band 252 is missing and needs to be added |  |  |  |  |  |  |  |  |
| Summary of change: |  | The MOP requirement for category M1 and NB-IoT NTN band 252 have been added into clause 6.2A.1 and 6.2B.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band 252 MOP requirement will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1, 6.2B.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | WIC belongs to Rel-19 that will trigger the specific upgrade to Rel-19 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

6.2A.1	UE maximum output power for category M1

6.2A.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2A.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 17 and forward of UE category M1 that support satellite access operation.

6.2A.1.3	Minimum conformance requirements

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2A.1-1: UE Power Class

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 |  |  | 23 | +/-2 | 20 | +/-2 |
| 255 |  |  | 23 | +/-2 | 20 | +/-2 |
| 254 |  |  | 23 | +/-2 | 20 | +/-2 |
| 253 |  |  | 23 | +/-2 | 20 | +/-2 |
| 252 |  |  | 23 | +/-2 | 20 | +/-2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance |  |  |  |  |  |  |

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

The normative reference for this requirement is TS 36.102 [11] clause 6.2.A.

==============Next change==============

6.2A.1.5	Test requirements

The maximum output power derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.1.5-1.

Table 6.2A.1.5-1: UE Power Class test requirements

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) | Class 6 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 256 |  |  | 23 | ±2.7 | 20 | ±2.7 |  |  |
| 255 |  |  | 23 | ±2.7 | 20 | ±2.7 |  |  |
| 254 |  |  | 23 | ±2.7 | 20 | ±2.7 |  |  |
| 253 |  |  | 23 | ±2.7 | 20 | ±2.7 |  |  |
| 252 |  |  | 23 | ±2.7 | 20 | ±2.7 |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |  |  |  |

==============Next change==============

6.2B	Transmit power for category NB1 and NB2

6.2B.1	UE maximum output power for category NB1 and NB2

6.2B.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2B.1.2	Test applicability

This test case applies to all types of NB-IoT UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

6.2B.1.3	Minimum conformance requirements

Category NB1 and NB2 UE Power Classes are specified in Table 6.2B.1.3-1 and define the maximum output power for any transmission bandwidth within the category NB1 and NB2 channel bandwidth. For 3.75kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one sub-frame (1ms).

Table 6.2B.1.3-1: UE Power Class

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 23 | +/-2 | 20 | +/-2 |
| 255 | 23 | +/-2 | 20 | +/-2 |
| 254 | 23 | +/-2 | 20 | +/-2 |
| 253 | 23 | +/-2 | 20 | +/-2 |
| 252 | 23 | +/-2 | 20 | +/-2 |

The normative reference for this requirement is TS 36.102 [11] clause 6.2B.1.

==============Next change==============

6.2B.1.5	Test requirements

The maximum output power derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2B.1.5-1.

Table 6.2B.1.5-1: UE Power Class test requirement

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 23 | +/-2.7 | 20 | +/-2.7 |
| 255 | 23 | +/-2.7 | 20 | +/-2.7 |
| 254 | 23 | +/-2.7 | 20 | +/-2.7 |
| 253 | 23 | +/-2.7 | 20 | +/-2.7 |
| 252 | 23 | +/-2.7 | 20 | +/-2.7 |

==============End of change==============