3GPP TSG-RAN5 Meeting #109	R5-256672

# Dallas, USA, 17th – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0709 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update annex A.29 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2025-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To update A.29 according the condition for loss of data protection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In step 1, the requirement on red/1000 has been updated to be conditionally mandatory. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RTT related condition for loss of data protection will not be considered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.29 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-2 CR 0403 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

# A.29	IMS Emergency Voice Call and RTT / 5GS

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | INVITE | UE sends INVITE with the first SDP offer. |
| 2 |  |  | 100 Trying | SS sends a 100 Trying provisional response. |
| 3 |  |  | 180 Ringing | SS sends a 180 Ringing. |
| 4 |  |  | 200 OK | SS responds INVITE with 200 OK. |
| 5 |  |  | ACK | UE acknowledges. |

Specific Message Contents

INVITE (Step 1)

Use the default message "INVITE for MO Call" in Annex A.2.1 of TS 34.229-1 [2] with condition A7 and with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Message-body | The following SDP types and values. Session description: v=0 o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) s=(session name) c=IN (addrtype) (connection-address for UE) [Note 1] Time description: t=(start-time) (stop-time) Media description: m=audio (transport port) [Note 2] c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) Media description: m=text (transport port) [Note 3, 4] c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) Note 1: At least one "c=" field shall be present. Note 2: AMR-WB/16000 codec shall be present in the media attributes, optionally including channel number "/1". Note 3: t140/1000 name and rate shall be present in the media attributes. Note 4: red/1000 name and rate is mandatory present in the media attributes if NOT A.12/70 [3]. It is optional present otherwise. |

180 Ringing for INVITE (Step 3)

Use the default message "180 Ringing for INVITE" in Annex A.2.6 of TS 34.229-1 [2] with conditions A4 and A8.

200 OK for INVITE (Step 4)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 of TS 34.229-1 [2] with condition A6 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length |  |
| value | length of message-body |
| Message-body | The following SDP types and values. Session description: v=0 o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) s=- c=IN (addrtype) (connection-address for SS) b=AS:39 Time description: t=0 0 Media description: m=audio (transport port) RTP/AVP (fmt) [Note 1] b=AS:37 b=RS:0 b=RR:0 Attributes for media: a=rtpmap: (payload type) AMR-WB/16000/1 [Note 1] a=fmtp: (format) mode-change-capability=2; max-red=220 a=ptime:20 a=maxptime:240 Media description: m=text (transport port) RTP/AVP (fmt) [Note 1] b=AS:2 b=RS:0 b=RR:0 Attributes for media: a=rtpmap: (payload type) t140/1000 [Note 1] a=rtpmap: (payload type) red/1000 [Note 2] a=fmtp: (format) (format specific parameters) [Note 2] Note 1: The values for fmt, payload type and format are copied from step 1. Note 2: If red/1000 is present in step 1 the values for fmt, payload type, format specific parameters and format are copied from step1, otherwise the red/1000 codec is not present. |

ACK (step 5)

Use the default message "ACK" in Annex A.2.7 of TS 34.229-1 [2].

<End of modified section>