3GPP TSG-RAN5 Meeting #109	R5-256876

# Dallas, US, 17th – 21st November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 1033 | rev | 1 | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of PDSCH dmrs-AdditionalPosition in PDCCH Demod test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | PDSCH dmrs-AdditionalPosition = pos1 is used as per default configuration defined in TS 38.508-1 Table 5.4.2.0-24. However, acccording to TS 38.214 section 5.1.6.2, when PDSCH is scheduled by DCI format 1-0, dmrs-AdditionalPosition = pos2 should be used. There was a dupplicated table number in Annex A.3.3.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new message exception to configure dmrs-AdditionalPosition = pos2 for those test cases using DCI format 1-0. Fixed table number Table A.3.3.1.1-3 to Table A.3.3.1.1-4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Signalling for PDCCH demod test cases using DCI format 1-0 will remain incorrect. Table number will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.1.1, 5.3.2.1.2, 5.3.2.2.1, 5.3.3.1.1, 5.3.3.1.2, 5.3.3.2.1, 7.3.2.2.1, 7.3.2.2.2, A.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-256876 is the final version of R5-256293 |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

##### 5.3.2.1.1	2Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA

5.3.2.1.1.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 2 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.2.1.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.2.1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.3.2.1.1.3	Minimum conformance requirements

For the parameters specified in Table 5.3.2.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.1.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.2.1.1.3-1: Minimum performance for 1 Tx PDCCH with 15 kHz SCS

| Test number | Bandwidth | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 10 MHz | 24 | 2 | 2 | R.PDCCH.1-2.1 FDD | TDLA30-10 | 1x2 Low | 1 | 8.1 |
| 1-2 | 10 MHz | 24 | 2 | 2 | R.PDCCH.1-2.3 FDD | TDLC300-100 | 1x2 Low | 1 | 8.2 |
| 1-3 | 10 MHz | 48 | 2 | 4 | R.PDCCH.1-2.4 FDD | TDLA30-10 | 1x2 Low | 1 | 5.5 |
| 1-4 | 10 MHz | 48 | 1 | 4 | R.PDCCH.1-1.1 FDD | TDLA30-10 | 1x2 Low | 1 | 4.4 |
| 1-5 | 10 MHz | 48 | 2 | 16 | R.PDCCH.1-2.6 FDD | TDLA30-10 | 1x2 Low | 1 | -2.1 |

The normative reference for this requirement is TS 38.101-4 [2] clause 5.3.

5.3.2.1.1.4	Test description

5.3.2.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: High Range with maxWgap for Intra-band non-contiguous EN-DC, otherwise, Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A in Figure A.3.1.7.2 for TE diagram and clause A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1 and Table 5.3.2.1.1-1 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.2.1.1.4.3.

5.3.2.1.1.4.2	Test procedure

1.	SS transmits PDCCH with DCI format as specified in PDCCH Reference Channel for C_RNTI to transmit the DL RMC according to Table 5.3.2.1.1.3-1. The details of PDCCH are specified in Table 5.3.1, Table 5.3.2.1-1, Table 5.3.2.1.1.3-1 respectively. The details of PDSCH are specified in Table A.3.3.1.1-3. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.2.1.1.4.4-1 as appropriate.

3. 	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.2.1.1.4.4-1, pass the UE. Otherwise fail the UE.

4.	Repeat steps from 1 to 3 for each subtest in Table 5.3.2.1.1.4.4-1 as appropriate.

5.3.2.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.3.2.1.1.4.3.1	Message exceptions for SA

Table 5.3.2.1.1.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP Test 3, 4, 5 |  |
|  | 11110000 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 24 RBs of the BWP Test 1, 2 |  |
| Duration | 2 | SearchSpace duration of 2 symbols Test 1, 2, 3, 5 |  |
|  | 1 | SearchSpace duration of 1 symbol Test 4 |  |
| } |  |  |  |

Table 5.3.2.1.1.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7 with condition USS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n1 | AL2 | Test 1, Test 2 |
| aggregationLevel4 | n1 | AL4 | Test 3, Test 4 |
| aggregationLevel8 | n0 |  |  |
| aggregationLevel16 | n1 | AL16 | Test 5 |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 for tests 2 and 3 | Long_DCI |
|  | formats0-0-And-1-0 | DCI Format 1_0 for tests 1, 4, 5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.1.1.4.3.1-3: Void

Table 5.3.2.1.1.4.3.1-3A: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType1 |  | Test 1, Test 4, Test 5 |
| } |  |  |  |

Table 5.3.2.1.1.4.3.1-4: DMRS-DownlinkConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | pos2 |  | Test 1-1, Test 1-4, Test 1-5 |
| } |  |  |  |

5.3.2.1.1.4.3.2	Message exceptions for NSA

Same as 5.3.2.1.1.4.3.1

5.3.2.1.1.4.4	Test requirement

Table 5.3.2.1.1.4.4-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.1.1.4.4-1.

Table 5.3.2.1.1.4.4-1: Test Requirement for 1Tx PDCCH with 15 kHz SCS

| Test number | Bandwidth | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 10 MHz | 24 | 2 | 2 | R.PDCCH.1-2.1 FDD | TDLA30-10 | 1x2 Low | 1 | 9.0 |
| 1-2 | 10 MHz | 24 | 2 | 2 | R.PDCCH.1-2.3 FDD | TDLC300-100 | 1x2 Low | 1 | 9.1 |
| 1-3 | 10 MHz | 48 | 2 | 4 | R.PDCCH.1-2.4 FDD | TDLA30-10 | 1x2 Low | 1 | 6.4 |
| 1-4 | 10 MHz | 48 | 1 | 4 | R.PDCCH.1-1.1 FDD | TDLA30-10 | 1x2 Low | 1 | 5.3 |
| 1-5 | 10MHz | 48 | 2 | 16 | R.PDCCH.1-2.6 FDD | TDLA30-10 | 1x2 Low | 1 | -1.2 |

## <<< Skip unchanged sections >>>

##### 5.3.2.1.2	2Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA

5.3.2.1.2.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 2 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.2.1.2.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.2.1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.3.2.1.2.3	Minimum conformance requirements

For the parameters specified in Table 5.3.2.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.1.2.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.2.1.2.3-1: Minimum performance for 2 Tx PDCCH with 15 kHz SCS

| Test number | Bandwidth | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 10 MHz | 24 | 2 | 4 | R.PDCCH. 1-2.2 FDD | TDLC300-100 | 2x2 Low | 1 | 2.0 |
| 1-2 | 10 MHz | 48 | 2 | 8 | R.PDCCH. 1-2.5 FDD | TDLC300-100 | 2x2 Low | 1 | -1.3 |
| 1-3 | 10 MHz | 48 | 1 | 8 | R.PDCCH.1-1.3 FDD | TDLA30-10 | 2x2 Low | 1 | -0.2 |

The normative reference for this requirement is TS 38.101-4 [2] clause 5.3.

5.3.2.1.2.4	Test description

5.3.2.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: High Range with maxWgap for Intra-band non-contiguous EN-DC, otherwise, Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and clause A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.2.1-1 and Table 5.3.2.1.2.3-1 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.2.1.2.4.3.

5.3.2.1.2.4.2	Test procedure

1.	SS transmits PDCCH with DCI format as specified in PDCCH Reference Channelfor C_RNTI to transmit the DL RMC according to Table 5.3.2.1.2.3-1. The details of PDCCH are specified in Table  5.3.1, Table 5.3.2.1-1, Table 5.3.2.1.2.3-1 respectively. The details of PDSCH are specified in Table A.3.3.1.1-3. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.2.1.2.4.4-1 as appropriate.

3.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.2.1.2.4.4-1, pass the UE. Otherwise fail the UE.

4.	Repeat steps from 1 to 3 for each subtest in Table 5.3.2.1.2.4.4-1 as appropriate.

5.3.2.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.3.2.1.2.4.3.1	Message exceptions for SA

Table 5.3.2.1.2.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP Test 2, 3 |  |
|  | 11110000 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 24 RBs of the BWP Test 1 |  |
| Duration | 2 | SearchSpace duration of 2 symbols Test 1, 2 |  |
|  | 1 | SearchSpace duration of 1 symbol Test 3 |  |
| } |  |  |  |

Table 5.3.2.1.2.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n1 | AL4 | Test 1 |
| aggregationLevel8 | n1 | AL8 | Test 2, 3 |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 for tests 2 and 3 | Long_DCI |
|  | formats0-0-And-1-0 | DCI Format 1_0 for test 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.1.2.4.3.1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType1 |  | Test 1 |
| } |  |  |  |

Table 5.3.2.1.2.4.3.1-4: DMRS-DownlinkConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | pos2 |  | Test 1-1 |
| } |  |  |  |

5.3.2.1.2.4.3.2	Message exceptions for NSA

Same as 5.3.2.1.2.4.3.1

5.3.2.1.2.4.4	Test requirement

Table 5.3.2.1.2.4.4-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.1.2.4.4-1.

Table 5.3.2.1.2.4.4-1: Test Requirements for 2 Tx PDCCH with 15 kHz SCS

| Test number | Bandwidth | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 10 MHz | 24 | 2 | 4 | R.PDCCH. 1-2.2 FDD | TDLC300-100 | 2x2 Low | 1 | 3.0 |
| 1-2 | 10 MHz | 48 | 2 | 8 | R.PDCCH. 1-2.5 FDD | TDLC300-100 | 2x2 Low | 1 | -0.3 |
| 1-3 | 10 MHz | 48 | 1 | 8 | R.PDCCH.1-1.3 FDD | TDLA30-10 | 2x2 Low | 1 | 0.8 |

## <<< Skip unchanged sections >>>

##### 5.3.2.2.1	2Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA

5.3.2.2.1.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 2 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.2.2.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.2.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.3.2.2.1.3	Minimum conformance requirements

For the parameters specified in Table 5.3.2.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.2.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.2.2.1.3-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 40 | 102 | 1 | 2 | R.PDCCH.2-1.1 TDD | TDLA30-10 | 1x2 Low | 1 | 7.0 |
| 1-2 | 40 | 102 | 1 | 4 | R.PDCCH.2-1.2 TDD | TDLC300- 100 | 1x2 Low | 1 | 3.0 |
| 1-3 | 40 | 48 | 2 | 16 | R.PDCCH.2-2.1 TDD | TDLC300- 100 | 1x2 Low | 1 | -3.8 |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.2.2.

5.3.2.2.1.4	Test description

5.3.2.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-2 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: High Range with maxWgap for Intra-band non-contiguous EN-DC, otherwise, Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.2 for TE diagram and section A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.2.2-1 and Table 5.3.2.2.1.3-1 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.2.2.1.4.3.

5.3.2.2.1.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH with DCI format as specified in PDCCH Reference Channel for C_RNTI to transmit the DL RMC according to Table 5.3.2.2.1.3-1. The details of PDCCH are specified in Table 5.3-1, Table 5.3.2.2-1 and Table 5.3.2.2.1.3-1 respectively. The details of PDSCH are specified in Table A.3.3.2.2-3. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.2.2.1.5-1 as appropriate.

3.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.2.2.1.5-1, pass the UE. Otherwise fail the UE.

4.	Repeat steps from 1 to 3 for each subtest in Table 5.3.2.2.1.5-1 as appropriate.

5.3.2.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1 and 5.4.2.

5.3.2.2.1.4.3.1	Message exceptions for SA

Table 5.3.2.2.1.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP Test 3 |  |
|  | 11111111 11111111 10000000 00000000 00000000 00000 | CORESET to use the least significant 102 RBs of the BWP Test 1, 2 |  |
| Duration | 2 | SearchSpace duration of 2 symbols Test 3 |  |
|  | 1 | SearchSpace duration of 1 symbol Test 1, 2 |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { | Null |  | Test 1, Test 2 |
| reg-BundleSize | n2 |  | 1 Tx |
| interleaverSize | n3 |  | TDD |
| } |  |  |  |
| nonInterleaved | null |  | Test 3 |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.1.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7 with condition USS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel2 | n1 | AL2 | Test 1 |
| aggregationLevel4 | n1 | AL4 | Test 2 |
| aggregationLevel16 | n1 | AL16 | Test 3 |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 for test 2 | Long_DCI |
|  | formats0-0-And-1-0 | DCI Format 1_0 for tests 1 and 3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.1.4.3.1-3: Void

Table 5.3.2.2.1.4.3.1-3A: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType1 |  | Test 1, Test 3 |
| } |  |  |  |

Table 5.3.2.2.1.4.3.1-4: DMRS-DownlinkConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | pos2 |  | Test 1-1, Test 1-3 |
| } |  |  |  |

5.3.2.2.1.4.3.2	Message exceptions for NSA

Same as 5.3.2.2.1.4.3.1.

5.3.2.2.1.5	Test requirement

Table 5.3.2.2.1.5-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.2.1.5-1.

Table 5.3.2.2.1.5-1: Test Requirement for 1Tx PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 40 | 102 | 1 | 2 | R.PDCCH.2-1.1 TDD | TDLA30-10 | 1x2 Low | 1 | 7.9 |
| 1-2 | 40 | 102 | 1 | 4 | R.PDCCH.2-1.2 TDD | TDLC300- 100 | 1x2 Low | 1 | 3.9 |
| 1-3 | 40 | 48 | 2 | 16 | R.PDCCH.2-2.1 TDD | TDLC300- 100 | 1x2 Low | 1 | -2.9 |

## <<< Skip unchanged sections >>>

##### 5.3.3.1.1	4Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA

5.3.3.1.1.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 4 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.3.1.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.3.1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 Rx antenna ports.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and 4 Rx antenna ports.

5.3.3.1.1.3	Minimum conformance requirements

For the parameters specified in Table 5.3.3.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.1.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.3.1.1.3-1: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.1 FDD | TDLA30-10 | 1x4 Low | 1 | 2.2 |
| 2 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.3 FDD | TDLC300- 100 | 1x4 Low | 1 | 2.7 |
| 3 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x4 Low | 1 | 0.2 |
| 4 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.1 FDD | TDLA30-10 | 1x4 Low | 1 | -0.4 |
| 5 | 10 | 48 | 2 | 16 | R.PDCCH. 1-2.6 FDD | TDLA30-10 | 1x4 Medium A | 1 | -3.2 |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.3.1.

5.3.3.1.1.4	Test description

5.3.3.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: High Range with maxWgap for Intra-band non-contiguous EN-DC, otherwise, Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.3 for TE diagram and section A.3.2.5 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.3.1-1 and Table 5.3.3.1.1.3-1 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.3.1.1.4.3.

5.3.3.1.1.4.2	Test procedure

1.	SS transmits PDCCH with DCI format as specified in PDCCH Reference Channel for C_RNTI to transmit the DL RMC according to Table 5.3.3.1.1.3-1. The details of PDCCH are specified in Table 5.3-1, Table 5.3.3.1-1 and Table 5.3.3.1.1.3-1 respectively. The details of PDSCH are specified in Table A.3.3.1.1-3. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.3.1.1.5-1 as appropriate.

3.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.3.1.1.5-1, pass the UE. Otherwise fail the UE.

4.	Repeat steps from 1 to 3 for each subtest in Table 5.3.3.1.1.5-1 as appropriate.

5.3.3.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.3.3.1.1.4.3.1	Message exceptions for SA

Table 5.3.3.1.1.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP Test 3, 4, 5 |  |
|  | 11110000 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 24 RBs of the BWP Test 1, 2 |  |
| Duration | 2 | SearchSpace duration of 2 symbols Test 1, 2, 3, 5 |  |
|  | 1 | SearchSpace duration of 1 symbol Test 4 |  |
| } |  |  |  |

Table 5.3.3.1.1.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7 with condition USS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel2 | n1 | AL2 | Test 1, Test 2 |
| aggregationLevel4 | n1 | AL4 | Test 3, Test 4 |
| aggregationLevel16 | n1 | AL16 | Test 5 |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 for tests 2 and 3 | Long_DCI |
|  | formats0-0-And-1-0 | DCI Format 1_0 for tests 1, 4, 5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.1.1.4.3.1-3: Void

Table 5.3.3.1.1.4.3.1-3A: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType1 |  | Test 1, Test 4, Test 5 |
| } |  |  |  |

Table 5.3.3.1.1.4.3.1-4: DMRS-DownlinkConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | pos2 |  | Test 1, Test 4, Test 5 |
| } |  |  |  |

5.3.3.1.1.4.3.2	Message exceptions for NSA

Same as 5.3.3.1.1.4.3.1.

5.3.3.1.1.5	Test requirement

Table 5.3.3.1.1.5-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.1.1.5-1.

Table 5.3.3.1.1.5-1: Test Requirement for 1Tx PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.1 FDD | TDLA30-10 | 1x4 Low | 1 | 3.1 |
| 2 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.3 FDD | TDLC300- 100 | 1x4 Low | 1 | 3.6 |
| 3 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x4 Low | 1 | 1.1 |
| 4 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.1 FDD | TDLA30-10 | 1x4 Low | 1 | 0.5 |
| 5 | 10 | 48 | 2 | 16 | R.PDCCH. 1-2.6 FDD | TDLA30-10 | 1x4 Medium A | 1 | -2.3 |

## <<< Skip unchanged sections >>>

##### 5.3.3.1.2	4Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA

5.3.3.1.2.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 4 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.3.1.2.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.3.1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 Rx antenna ports.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and 4 Rx antenna ports.

5.3.3.1.2.3	Minimum conformance requirements

For the parameters specified in Table 5.3.3.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.1.2.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.3.1.2.3-1: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 24 | 2 | 4 | R.PDCCH. 1-2.2 FDD | TDLC300-100 | 2x4 Low | 1 | -1.9 |
| 2 | 10 | 48 | 2 | 8 | R.PDCCH. 1-2.5 FDD | TDLC300-100 | 2x4 Low | 1 | -4.5 |
| 3 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.2 FDD | TDLA30-10 | 2x4 Low | 1 | -1.0 |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.3.1.

5.3.3.1.2.4	Test description

5.3.3.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: High Range with maxWgap for Intra-band non-contiguous EN-DC, otherwise, Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.4 for TE diagram and section A.3.2.5 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.3.1-1 and Table 5.3.3.1.2.3-1 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On) for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.3.1.2.4.3.

5.3.3.1.2.4.2	Test procedure

1.	SS transmits PDCCH with DCI format as specified in PDCCH Reference Channel for C_RNTI to transmit the DL RMC according to Table 5.3.3.1.2.3-1. The details of PDCCH are specified in Table 5.3-1, Table 5.3.3.1-1 and Table 5.3.3.1.2.3-1 respectively. The details of PDSCH are specified in Table A.3.3.1.1-3. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.3.1.2.5-1 as appropriate.

3.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.3.1.2.5-1, pass the UE. Otherwise fail the UE.

4.	Repeat steps from 1 to 3 for each subtest in Table 5.3.3.1.2.5-1 as appropriate.

5.3.3.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.3.3.1.2.4.3.1	Message exceptions for SA

Table 5.3.3.1.2.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP Test 2, 3 |  |
|  | 11110000 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 24 RBs of the BWP Test 1 |  |
| Duration | 2 | SearchSpace duration of 2 symbols Test 1, 2 |  |
|  | 1 | SearchSpace duration of 1 symbol Test3 |  |
| } |  |  |  |

Table 5.3.3.1.2.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7 with condition USS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel4 | n1 | AL4 | Test 1, Test 3 |
| aggregationLevel8 | n1 | AL8 | Test 2 |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 for tests 2 and 3 | Long_DCI |
|  | formats0-0-And-1-0 | DCI Format 1_0 for test 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.1.2.4.3.1-3: Void

Table 5.3.3.1.2.4.3.1-3A: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType1 |  | Test 1 |
| } |  |  |  |

Table 5.3.3.1.2.4.3.1-4: DMRS-DownlinkConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | pos2 |  | Test 1 |
| } |  |  |  |

5.3.3.1.2.4.3.2	Message exceptions for NSA

Same as 5.3.3.1.2.4.3.1.

5.3.3.1.2.5	Test requirement

Table 5.3.3.1.2.5-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.1.2.5-1.

Table 5.3.3.1.2.5-1: Test Requirement for 2Tx PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 24 | 2 | 4 | R.PDCCH. 1-2.2 FDD | TDLC300-100 | 2x4 Low | 1 | -0.9 |
| 2 | 10 | 48 | 2 | 8 | R.PDCCH. 1-2.5 FDD | TDLC300-100 | 2x4 Low | 1 | -3.5 |
| 3 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.2 FDD | TDLA30-10 | 2x4 Low | 1 | 0 |

## <<< Skip unchanged sections >>>

##### 5.3.3.2.1	4Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA

5.3.3.2.1.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 4 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.3.2.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.3.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 Rx antenna ports.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and 4 Rx antenna ports.

5.3.3.2.1.3	Minimum conformance requirements

For the parameters specified in Table 5.3.3.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.2.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.3.2.1.3-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 2 | R.PDCCH.2-1.1 TDD | TDLA30-10 | 1x4 Low | 1 | 2.1 |
| 2 | 40 | 102 | 1 | 4 | R.PDCCH.2-1.2 TDD | TDLC300-100 | 1x4 Low | 1 | -0.9 |
| 3 | 40 | 48 | 2 | 16 | R.PDCCH.2-2.1 TDD | TDLA30-10 | 1x4 Medium A | 1 | -3.6 |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.3.2.

5.3.3.2.1.4	Test description

5.3.3.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-2 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: High Range with maxWgap for Intra-band non-contiguous EN-DC, otherwise, Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.3 for TE diagram and section A.3.2.5 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.3.2-1 and Table 5.3.3.2.1.3-1 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On) for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.3.2.1.4.3.

5.3.3.2.1.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH with DCI format as specified in PDCCH Reference Channel for C_RNTI to transmit the DL RMC according to Table 5.3.3.2.1.3-1. The details of PDCCH are specified in Table 5.3-1, Table 5.3.3.2-1 and Table 5.3.3.2.1.3-1 respectively. The details of PDSCH are specified in Table A.3.3.2.2-3. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.3.2.1.5-1 as appropriate.

3.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.3.2.1.5-1, pass the UE. Otherwise fail the UE.

4.	Repeat steps from 1 to 3 for each subtest in Table 5.3.3.2.1.5-1 as appropriate.

5.3.3.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.3.3.2.1.4.3.1	Message exceptions for SA

Table 5.3.3.2.1.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP Test 3 |  |
|  | 11111111 11111111 10000000 00000000 00000000 00000 | CORESET to use the least significant 102 RBs of the BWP Test 1, 2 |  |
| Duration | 2 | SearchSpace duration of 2 symbols Test 3 |  |
|  | 1 | SearchSpace duration of 1 symbol Test 1, 2 |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { | Null |  | Test 1, Test 2 |
| reg-BundleSize | n2 |  | 1 Tx |
| interleaverSize | n3 |  | TDD |
| } |  |  |  |
| nonInterleaved | null |  | Test 3 |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.1.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7 with condition USS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel2 | n1 | AL2 | Test 1 |
| aggregationLevel4 | n1 | AL4 | Test 2 |
| aggregationLevel16 | n1 | AL16 | Test 3 |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 for test 2 | Long_DCI |
|  | formats0-0-And-1-0 | DCI Format 1_0 for test 1 and 3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.1.4.3.1-3: Void

Table 5.3.3.2.1.4.3.1-3A: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType1 |  | Test 1, Test 3 |
| } |  |  |  |

Table 5.3.3.2.1.4.3.1-4: DMRS-DownlinkConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | pos2 |  | Test 1, Test 3 |
| } |  |  |  |

5.3.3.2.1.4.3.2	Message exceptions for NSA

Same as 5.3.3.2.1.4.3.1.

5.3.3.2.1.5	Test requirement

Table 5.3.3.2.1.5-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.2.1.5-1.

Table 5.3.3.2.1.5-1: Test Requirement for 1Tx PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 2 | R.PDCCH.2-1.1 TDD | TDLA30-10 | 1x4 Low | 1 | 3 |
| 2 | 40 | 102 | 1 | 4 | R.PDCCH.2-1.2 TDD | TDLC300-100 | 1x4 Low | 1 | 0 |
| 3 | 40 | 48 | 2 | 16 | R.PDCCH.2-2.1 TDD | TDLA30-10 | 1x4 Medium A | 1 | -2.7 |

## <<< Skip unchanged sections >>>

##### 7.3.2.2.1	2Rx TDD FR2 PDCCH 1 Tx antenna performance for both SA and NSA

7.3.2.2.1.1	Test Purpose

This test verifies the demodulation performance of PDCCH for a single-antenna port with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 7.3.2.2.1.3-1.

7.3.2.2.1.2	Test Applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

7.3.2.2.1.3	Minimum conformance requirements

For the parameters specified in Table 7.3.2.2.1.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 7.3.2.2.1.3-2. The downlink physical setup is in accordance with Annex C.2.2.

Table 7.3.2.2.1.3-1: Test Parameters

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR2.120-1 |  |
| CCE to REG mapping type |  | Interleaved |  |
| REG bundle size |  | 2 for test 1-1 6 for test 1-2 | 2 |
| Interleaver size |  | 3 for test 1-1 2 for test 1-2 | 3 |
| Shift index |  | 0 |  |

Table 7.3.2.2.1.3-2: Minimum performance requirements with 120 kHz SCS for 1Tx antenna

| Test number | Bandwidth | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) |
| 1-1 | 100 MHz | 60 | 1 | 2 CCE | R.PDCCH.5-1.1 TDD | TDLA30-75 | 1x2 Low | 1 | 6.0 |
| 1-2 | 100 MHz | 60 | 1 | 4 CCE | R.PDCCH.5-1.2 TDD | TDLA30-300 | 1x2 Low | 1 | 2.6 |

The normative reference for this requirement is TS 38.101-4 [5] clause 7.3.2.2.1.

7.3.2.2.1.4	Test Description

7.3.2.2.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2-1 of TS 38.521-2 [8].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

Only sub tests shall be tested which are testable according to Table 7.1.1_1-2.

For EN-DC within FR2 operation, setup the LTE link according to Annex D:

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.3.2 for TE diagram and Figure A.3.4.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 7.3-1 and Table 7.3.2.2.1.3-1 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-2 [8].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 7.3.2.2.1.4.3.

7.3.2.2.1.4.2	Test procedure

1.	Set the UE in a direction found using one of  the test procedures defined in Annex H. If no direction found, mark the test as inconclusive.

2.	SS transmits PDCCH with DCI format as specified in PDCCH Reference Channel  for C_RNTI to transmit the DL RMC according to Table 7.3.2.2.1.4.4-1. The details of PDCCH are specified in Table 7.3.2.2.1.3-1 and Table 7.3.2.2.1.3-2. The details of PDSCH are specified in Table A.3.3.2.5-3. The SS sends downlink MAC padding bits on the DL RMC.

3.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 7.3.2.2.1.4.4-1 as appropriate.

4.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 7.3.2.2.1.4.4-1, pass the UE. Otherwise fail the UE.

5.	Repeat steps from 1 to 4 for each subtest in Table 7.3.2.2.1.4.4-1 as appropriate.

7.3.2.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] subclause 4.6.1 and 5.4.2 with the following exceptions:

7.3.2.2.1.4.3.1	Message exceptions for SA

Table 7.3.2.2.1.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| frequencyDomainResources | 11111111 11000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 60 RBs of the BWP |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { | Null |  |  |
| reg-BundleSize | n2 |  | 2 for test 1-1 |
|  | n6 |  | 6 for test 1-2 |
| interleaverSize | n3 |  | 3 for test 1-1 |
| } | n2 |  | 2 for test 1-2 |
| } |  |  |  |
| } |  |  |  |

Table 7.3.2.2.1.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel2 | n1 | AL2 | Test 1-1 |
| aggregationLevel4 | n1 | AL4 | Test 1-2 |
| } |  |  |  |
| } |  |  |  |

Table 7.3.2.2.1.4.3.1-3: DMRS-DownlinkConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | pos2 |  | Test 1-1 |
| } |  |  |  |

7.3.2.2.1.4.3.2	Message exceptions for NSA

Same as 7.3.2.2.1.4.3.1.

7.3.2.2.1.4.4	Test requirement

Table 7.3.2.2.1.4.4-1 defines the primary level settings.

For the parameters specified in Table 7.3.2.2.1.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 7.3.2.2.1.4.4-1.

Table 7.3.2.2.1.4.4-1: Test requirements with 120 kHz SCS for 1Tx antenna

| Test number | Bandwidth | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) |
| 1-1 | 100 MHz | 60 | 1 | 2 CCE | R.PDCCH.5-1.1 TDD | TDLA30-75 | 1x2 Low | 1 | 7.7 |
| 1-2 | 100 MHz | 60 | 1 | 4 CCE | R.PDCCH.5-1.2 TDD | TDLA30-300 | 1x2 Low | 1 | 4.3 |

##### 7.3.2.2.2	2Rx TDD FR2 PDCCH 2 Tx antenna performance for both SA and NSA

7.3.2.2.2.1	Test Purpose

This test verifies the demodulation performance of PDCCH for two-antenna port with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 7.3.2.2.2.3-1.

7.3.2.2.2.2	Test Applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

7.3.2.2.2.3	Minimum conformance requirements

For the parameters specified in Table 7.3.2.2.2.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 7.3.2.2.2.3-2. The downlink physical setup is in accordance with Annex C.2.2.

Table 7.3.2.2.2.3-1: Test Parameters

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR2.120-1 |  |
| CCE to REG mapping type |  | Interleaved |  |
| REG bundle size |  | 2 for test 1-1 6 for test 1-2 | 2 |
| Interleaver size |  | 3 for test 1-1 2 for test 1-2 | 3 |
| Shift index |  | 0 |  |

Table 7.3.2.2.2.3-2: Minimum performance requirements with 120 kHz SCS for 2Tx Antenna

| Test number | Bandwidth | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) |
| 2-1 | 100 MHz | 60 | 1 | 8 CCE | R.PDCCH.5-1.3 TDD | TDLA30-75 | 2x2 Low | 1 | 1.4 |
| 2-2 | 100 MHz | 60 | 2 | 16 CCE | R.PDCCH.5-2.1 TDD | TDLA30-75 | 2x2 Low | 1 | -1.6 |

The normative reference for this requirement is TS 38.101-4 [5] clause 7.3.2.2.2.

7.3.2.2.2.4	Test Description

7.3.2.2.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2-1 of TS 38.521-2 [8].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

Only sub tests shall be tested which are testable according to Table 7.1.1_1-2.

For EN-DC within FR2 operation, setup the LTE link according to Annex D:

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.3.2 for TE diagram and Figure A.3.4.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 7.3-1 and Table 7.3.2.2.2.3-1 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2, C.3.1 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-2 [8].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 7.3.2.2.1.4.3.

7.3.2.2.2.4.2	Test procedure

1.	Set the UE in a direction found using one of  the test procedures defined in Annex H If no direction found, mark the test as inconclusive.

2.	SS transmits PDCCH with DCI format as specified in PDCCH Reference Channel  for C_RNTI to transmit the DL RMC according to Table 7.3.2.2.2.4.4-1. The details of PDCCH are specified in Table 7.3.2.2.2.3-1 and Table 7.3.2.2.2.3-2. The details of PDSCH are specified in Table A.3.3.2.5-3. The SS sends downlink MAC padding bits on the DL RMC.

3.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 7.3.2.2.2.4.4-1 as appropriate.

4.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 7.3.2.2.2.4.4-1, pass the UE. Otherwise fail the UE.

5.	Repeat steps from 1 to 4 for each subtest in Table 7.3.2.2.2.4.4-1 as appropriate.

7.3.2.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] subclause 4.6.1 and 5.4.2 with the following exceptions:

7.3.2.2.2.4.3.1	Message exceptions for SA

Table 7.3.2.2.2.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| frequencyDomainResources | 11111111 11000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 60 RBs of the BWP |  |
| Duration | 2 | SearchSpace Duration of 2 symbols | Test 2-2 |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { | Null |  |  |
| reg-BundleSize | n2 |  |  |
| interleaverSize | n3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.3.2.2.2.4.3.1-2: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 2 entries |  | Test 2-2 |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 | Test 2-2 |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 100 | Start symbol(S)=2, Length(L)=8 | Test 2-2 |
| } |  |  |  |
| } |  |  |  |

Table 7.3.2.2.2.4.3.1-3: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel8 | n1 | AL8 | Test 2-1 |
| aggregationLevel16 | n1 | AL16 | Test 2-2 |
| } |  |  |  |
| } |  |  |  |

Table 7.3.2.2.2.4.3.1-4: DMRS-DownlinkConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | pos2 |  | Test 2-2 |
| } |  |  |  |

7.3.2.2.2.4.3.2	Message exceptions for NSA

Same as 7.3.2.2.2.4.3.1.

7.3.2.2.2.4.4	Test requirement

Table 7.3.2.2.2.4.4-1 defines the primary level settings.

For the parameters specified in Table 7.3.2.2.2.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 7.3.2.2.2.4.4-1.

Table 7.3.2.2.2.4.4-1: Test requirements with 120 kHz SCS

| Test number | Bandwidth | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) |
| 2-1 | 100 MHz | 60 | 1 | 8 CCE | R.PDCCH.5-1.3 TDD | TDLA30-75 | 2x2 Low | 1 | 3.2 |
| 2-2 | 100 MHz | 60 | 2 | 16 CCE | R.PDCCH.5-2.1 TDD | TDLA30-75 | 2x2 Low | 1 | 0.2 |

## <<< Skip unchanged sections >>>

## A.3.3	Reference measurement channels for PDCCH performance requirements

### A.3.3.1	FDD

#### A.3.3.1.1	Reference measurement channels for SCS 15 kHz FR1

Table A.3.3.1.1-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-1.1 FDD | R.PDCCH.1-1.2 FDD | R.PDCCH.1-1.3 FDD |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |  |
| CORESET frequency domain allocation |  | 48 | 48 | 48 |  |  |  |
| CORESET time domain allocation |  | 1 | 1 | 1 |  |  |  |
| Aggregation level |  | 4 | 4 | 8 |  |  |  |
| DCI Format |  | 1_0 | 1_1 | 1_1 |  |  |  |
| Payload (without CRC) | Bits | 39 | 52 | 52 |  |  |  |

Table A.3.3.1.1-2: PDCCH Reference Channel (Time domain allocation 2 symbols)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-2.1 FDD | R.PDCCH.1-2.2 FDD | R.PDCCH.1-2.3 FDD | R.PDCCH.1-2.4 FDD | R.PDCCH.1-2.5 FDD | R.PDCCH.1-2.6 FDD | R.PDCCH.1-2.7 FDD |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| CORESET frequency domain allocation |  | 24 | 24 | 24 | 48 | 48 | 48 | 48 |
| CORESET time domain allocation |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Aggregation level |  | 2 | 4 | 2 | 4 | 8 | 16 | 8 |
| DCI Format |  | 1_0 | 1_0 | 1_1 | 1_1 | 1_1 | 1_0 | 2_6 |
| Payload (without CRC) | Bits | 39 | 39 | 52 | 52 | 52 | 39 | 12 |

Table A.3.3.1.1-3: PDCCH Reference Channel (Time domain allocation 3 symbols)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-3.1 FDD |  |  |  |  |  |  |
| Subcarrier spacing | kHz | 15 |  |  |  |  |  |  |
| CORESET frequency domain allocation |  | 24 |  |  |  |  |  |  |
| CORESET time domain allocation |  | 3 |  |  |  |  |  |  |
| Aggregation level |  | 4 |  |  |  |  |  |  |
| DCI Format |  | 1_0 |  |  |  |  |  |  |
| Payload (without CRC) | Bits | 35Note1 |  |  |  |  |  |  |
| Note1: The payload is corresponding to BWP size of 15RB |  |  |  |  |  |  |  |  |

Table A.3.3.1.1-4: Additional PDSCH Reference Channel FDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| DCI Format |  | 1_0 | 1_1 |
| Channel bandwidth | MHz | 10 | 10 |
| Subcarrier spacing | kHz | 15 | 15 |
| Number of allocated resource blocks | PRBs | 52 | 52 |
| Number of consecutive PDSCH symbols |  | 12 | 12 |
| Allocated slots per 2 frames | Slots | 19 | 19 |
| MCS table |  | 64QAM | 64QAM |
| MCS index |  | 4 | 4 |
| Modulation |  | QPSK | QPSK |
| Target Coding Rate |  | 0.30 | 0.30 |
| Number of MIMO layers |  | 1 | 1 |
| Number of DMRS REs (Note 5) |  | 36 | 12 |
| Overhead for TBS determination |  | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 3368 | 4096 |
| Transport block CRC per Slot |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 16 | 24 |
| Number of Code Blocks per Slot |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A |
| For Slots i = 1,…, 19 | CBs | 1 | 1 |
| Binary Channel Bits Per Slot |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |
| For Slots i = 10, 11 | Bits | 9984 | 13104 |
| For Slots i = 1,…, 9, 12, …, 19 | Bits | 11232 | 13728 |
| Max. Throughput averaged over 2 frames | Mbps | 3.1996 | 3.8912 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. Note 2: Slot i is slot index per 2 frames. Note 3: For PDSCH & PDSCH DMRS precoding the following configuration shall be used: Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity. Note 4: For tests with DRX configured, PDCCH and PDSCH shall only be scheduled during the DRX on duration of the test. Note 5: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data. |  |  |  |

## <<< END OF CHANGES >>>