# 3GPP TSG-RAN5 Meeting #109	R5-256085r1

# Dallas, US, Nov 17 - 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3652 | rev | - | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of reference configuration for R18 IDC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IDC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of reference configurations for non-CA multiple frequencies IDC TCs and CA IDC TCs to enable the testing of R18 IDC related enhancements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new reference configurations for non-CA multiple frequencies IDC TCs and CA IDC TCs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA IDC cannot be tested, R18 FDM and TDM IDC TCs descriptions are not align with other IDC TCs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508-2 CR 0891, TS 38.523-1 CR 5202, 5203, 5204 and 5205 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | “Table 4.13-3” will be referenced in TS 38.508-2 CR 0891, and TS 38.523-1 CR 5204 and 5205. “Table 4.13-2” will be referenced in TS 38.523-1 CR 5202 and 5203. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Change the test cell from NR Cell 1 to NR Cell 10 to enable the inter-band non-CA multiple frequencies IDC configurations. |  |  |  |  |  |  |  |  |

<Start of modified section>

4.13	Reference Configuration for IDC

The present subclause provides configurations required for IDC mechanism testing and only applies when PICS “pc_IDC_Report” is TRUE.

Table 4.13-1 provides configurations for testing IDC mechanism.

For NR operating band n40, NR Cell 14/NRf4 (2390-2400MHz) is selected as it is the closest frequency available to 2401MHz. WLAN AP1 (Cell 27) with Channel 1 (2401-2423MHz) is configured.

For NR operating band n41, NR Cell 1/NRf1 (2496-2556MHz) is selected as it is the closest frequency available to 2483MHz or in inter-band scenario, NR Cell 10/NRf5 (2496-2556MHz) is selected as it is the closest frequency available to 2483MHz. WLAN AP2 (Cell 27b) with Channel 13 (2461-2483MHz) is configured.

For inter-band CA, the frequency combination CA_n3A-n8A is selected as this combination has inter-modulation order 3 interfers 2.4GHz WLAN.

When “C1” applies, UE does not transmit UEAssistanceInformation message including IDC-Assistance-r16 as UE cannot detect IDC problem with “C1” configuration.

When “C2” applies, UE transmits UEAssistanceInformation with IDC-Assistance-r16 as UE can detect IDC problem with “C2” configuration if it was configured with idc-AssistanceConfig-r16.

When “C3” applies, UE does not transmit UEAssistanceInformation with AffectedCarrierFreqRangeList-r18 as UE cannot detect IDC problem with “C3” configuration.

When “C4” applies, UE transmits UEAssistanceInformation with AffectedCarrierFreqRangeList-r18 in NR Cell 14 as UE can detect IDC problem with “C4” configuration if it was configured with idc-FDM-AssistanceConfig, or UE transmits UEAssistanceInformation with AffectedCarrierFreqRangeList-r18 and idc-TDM-Assistance-r18 in NR Cell 14 as UE can detect IDC problem with “C4” configuration if it was configured with idc-TDM-AssistanceConfig.

When “C5” applies, UE does not transmit UEAssistanceInformation with AffectedCarrierFreqRangeList-r18 as UE cannot detect IDC problem with “C5” configuration.

When “C6” applies, UE transmits UEAssistanceInformation with AffectedCarrierFreqRangeList-r18 in NR Cell 10 as UE can detect IDC problem with “C6” configuration if it was configured with idc-FDM-AssistanceConfig, or UE transmits UEAssistanceInformation with AffectedCarrierFreqRangeList-r18 and idc-TDM-Assistance-r18 in NR Cell 10 as UE can detect IDC problem with “C6” configuration if it was configured with idc-TDM-AssistanceConfig.

When “C7” applies, UE does not transmit UEAssistanceInformation with AffectedCarrierFreqRangeCombList-r18 as there is no inter-modulation with “C7” configuration.

When “C8” applies, UE does not transmit UEAssistanceInformation with AffectedCarrierFreqRangeCombList-r18 as there is no inter-modulation with “C8” configuration.

When “C9” applies, UE transmits UEAssistanceInformation with AffectedCarrierFreqRangeCombList-r18 as there is inter-modulation interfering 2.4GHz WLAN with “C9” configuration.

Table 4.13-1: IDC Configuration of cell power level for FR1

|  | Parameter | Unit | NR Cell 14/NR Cell 1 | WLAN AP 1 (Cell 27) /WLAN AP 2 (Cell 27b) | Remark |
| --- | --- | --- | --- | --- | --- |
| C1 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | Cell configuration for non-IDC. |
|  | BeaconRSSI | dBm |  | Off |  |
| C2 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | Cell configuration for IDC. |
|  | BeaconRSSI | dBm |  | -80 |  |

Table 4.13-2: Non-CA multiple frequencies IDC Configuration of cell power level for FR1

|  | Parameter | Unit | NR Cell 14 | WLAN AP 1 (Cell 27) | NR Cell 10 | WLAN AP 2 (Cell 27b) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C3 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | Off |  | Cell configuration for non-IDC. |
|  | BeaconRSSI | dBm |  | Off |  | Off |  |
| C4 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | Off |  | Cell configuration for IDC in NR Cell 14. |
|  | BeaconRSSI | dBm |  | -80 |  | Off |  |
| C5 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | -79 |  | Cell configuration for non-IDC in NR Cell 10. |
|  | BeaconRSSI | dBm |  | Off |  | Off |  |
| C6 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | -79 |  | Cell configuration for IDC in NR Cell 10. |
|  | BeaconRSSI | dBm |  | Off |  | -80 |  |

Table 4.13-3: CA IDC Configuration of cell power level for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | WLAN AP 1 (Cell 27) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| C7 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Off |  | Cell configuration for non-IDC. |
|  | BeaconRSSI | dBm |  |  | Off |  |
| C8 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 |  | Cell configuration for non-IDC. |
|  | BeaconRSSI | dBm |  |  | Off |  |
| C9 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 |  | Cell configuration for IDC of WLAN AP 1. |
|  | BeaconRSSI | dBm |  |  | -80 |  |

<End of modified section>