3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210416

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0136 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for common POS function f_POS_SelectTemplateReq_IonosphModel() |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Following the implementation of R5-211340, the check for pc_BDS_B1C is incorrect. With the current implementaion a fatal error will be thrown whenever pc_BDS_B1C is set to FALSE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Moved pc_BDS_B1C check inside pc_A_BDS condition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases may not execute correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All Assisted data POS test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | f_POS_SelectTemplateReq_IonosphModel() |
| --- | --- |
| Reason for change | Following the implementation of R5-211340, the check for pc_BDS_B1C is incorrect. With the current implementaion a fatal error will be thrown whenever pc_BDS_B1C is set to FALSE. |
| Summary of change | Moved pc_BDS_B1C check inside pc_A_BDS condition. |
| TTCN module | CommonPOS_Functions.ttcn |
| MCC160 Comment |  |

Before change

function f_POS_SelectTemplateReq_IonosphModel() return template (value) GNSS_IonosphericModelReq

{

var template (value) GNSS_IonosphericModelReq v_IonosphericModelReq;

if (pc_A_GPS_L1C_A or pc_A_GPS_L1C_A_MGPS or pc_A_GLONASS or pc_A_BDS){

v_IonosphericModelReq := cs_GNSS_IonosphericModelReq('00'B, omit); // Request klobuchar Model

if (pc_A_Galileo) {

v_IonosphericModelReq := cs_GNSS_IonosphericModelReq('00'B, NULL); // Request klobuchar and neQuick Models

}

}

else if (pc_A_Galileo) {

v_IonosphericModelReq := cs_GNSS_IonosphericModelReq(omit, NULL); // Request neQuick Model

}

if (pc_BDS_B1C) { //@sic R5-211340 sic@

v_IonosphericModelReq.klobucharModel2Req_r16 := NULL; // Add request for klobucharModel2

}

else {

FatalError (__FILE__, __LINE__, "No GNSS method supported");

}

return v_IonosphericModelReq;

}

After change

function f_POS_SelectTemplateReq_IonosphModel() return template (value) GNSS_IonosphericModelReq

{

var template (value) GNSS_IonosphericModelReq v_IonosphericModelReq;

if (pc_A_GPS_L1C_A or pc_A_GPS_L1C_A_MGPS or pc_A_GLONASS or pc_A_BDS){

v_IonosphericModelReq := cs_GNSS_IonosphericModelReq('00'B, omit); // Request klobuchar Model

if (pc_A_Galileo) {

v_IonosphericModelReq := cs_GNSS_IonosphericModelReq('00'B, NULL); // Request klobuchar and neQuick Models

}

if (pc_BDS_B1C and pc_A_BDS) { //@sic R5-211340 sic@

v_IonosphericModelReq.klobucharModel2Req_r16 := NULL; // Add request for klobucharModel2

}

}

else if (pc_A_Galileo) {

v_IonosphericModelReq := cs_GNSS_IonosphericModelReq(omit, NULL); // Request neQuick Model

}

/* MOVED if (pc_BDS_B1C) { //@sic R5-211340 sic@

v_IonosphericModelReq.klobucharModel2Req_r16 := NULL; // Add request for klobucharModel2

}*/

else {

FatalError (__FILE__, __LINE__, "No GNSS method supported");

}

return v_IonosphericModelReq;

}