3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211696

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0153 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new ASP definitions for NR test model extension in Rel-16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At RAN5#93e, R5-216658 was agreed to extend the positioning test model to cover new Rel-16 sub-tests in 37.571-2. This document contains the ASPs supporting that test model extension. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New Annex E is created with the ASP for Rel-16 test model extension. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test model for Rel-16 will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex E (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Annex E (informative)
NR5GC System Simulator Extension ASP Definitions in TTCN-3

# E.1	NR5GC_POS_ASP_TypeDefs

Type definitions for configuration of DL Positioning Reference Signal in the system simulator;
Common design principles: refer to 3GPP TS 38.523-3 [24] annex D.

## E.1.1	NR_System_Configuration_PosExt

Formal ASP Definitions for NR5GC DL-PRS system configuration

NR_SystemRequest_PosExt_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SystemRequest_PosExt_Type |  |
| Comment | NOTE: Same structure as NR_SystemRequest_Type in 3GPP TS 38.523-3 [24] annex D |  |
| Cell | NR_CellConfigRequest_PosExt_Type | configure DL-PRS in an NR cell; the cell shall be configured as usual beforehand and SS shall raise an error when the cell does not exist yet; there is no explicit release of the PRS but the whole cell needs to be released |

NR_SystemConfirm_PosExt_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SystemConfirm_PosExt_Type |  |
| Comment | confirmations for system configuration; in general to be sent after the configuration has been done |  |
| Cell | Null_Type | (no further parameters from SS) |

## E.1.2	NR_Cell_Configuration_PosExt

Specific Info for DL-PRS Cell Configuration Primitive

### E.1.2.1	NR_Cell_Configuration_Common_PosExt

NR_CellConfigRequest_PosExt_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_CellConfigRequest_PosExt_Type |  |
| Comment | NOTE: Same structure as NR_SystemRequest_Type in 3GPP TS 38.523-3 [24] annex D |  |
| AddOrReconfigure | NR_CellConfigInfo_PosExt_Type | for PRS configuration of given cell: CellId : identifier of the cell to be configured RoutingInfo : None RlcBearerRouting : 'None' MacBearerRouting : 'omit' TimingInfo : Now (for initial configuration and for reconfiguration in general) ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |

NR_CellConfigInfo_PosExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_CellConfigInfo_PosExt_Type |  |  |
| Comment | NOTE: Same structure as NR_SystemRequest_Type in 3GPP TS 38.523-3 [24] annex D common information for initial cell configuration or reconfiguration; in case of reconfiguration OMIT means 'keep configuration as it is' |  |  |
| PhysicalLayer | NR_CellConfigPhysicalLayer_PosExt_Type | opt | Extended DL-PRS information for a cell |

NR_CellConfigPhysicalLayer_PosExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_CellConfigPhysicalLayer_PosExt_Type |  |  |
| Comment | NOTE: Same structure as NR_SystemRequest_Type in 3GPP TS 38.523-3 [24] annex D Physical layer: |  |  |
| Downlink | NR_CellConfigPhysicalLayerDownlink_PosExt_Type | opt | Default settings regarding physical control DL-PRS signal |

### E.1.2.2	Downlink_Physical_Layer_Configuration_PosExt

Downlink physical layer configuration:
- DL-PRS Frequency Layer configuration.
- DL-PRS TRP configuration.

Downlink_Physical_Layer_Configuration_PosExt: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| DlPrs_CombSizeN_Type | NR_DL_PRS_PositioningFrequencyLayer_r16.dl_PRS_CombSizeN_r16 |  |

DlPrs_FreqLayerConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DlPrs_FreqLayerConfig_Type |  |  |
| Comment |  |  |  |
| DlPrs_ResourceBW | integer | opt |  |
| DlPrs_CombSizeN | DlPrs_CombSizeN_Type | opt |  |

DlPrs_TRPConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DlPrs_TRPConfig_Type |  |  |
| Comment |  |  |  |
| NrDlPrsSfn0Offset | NR_DL_PRS_SFN0_Offset_r16 | opt |  |
| NrDlPrsExpectedRstd | integer | opt |  |
| NrDlPrsExpectedRstdUncertainty | integer | opt |  |
| NrDlPrsInfo | NR_DL_PRS_Info_r16 | opt |  |

DL_PRSConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DL_PRSConfig_Type |  |  |
| Comment | to configure DL-PRS for positioning; all fields are declared as optional to allow single reconfigurations; in this case omit means "keep as it is" |  |  |
| DlPrs_FreqLayerConfig | DlPrs_FreqLayerConfig_Type | opt |  |
| DlPrs_TRPConfig | DlPrs_TRPConfig_Type | opt |  |

NR_CellConfigPhysicalLayerDownlink_PosExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| ame | NR_CellConfigPhysicalLayerDownlink_PosExt_Type |  |  |
| Comment | all fields are declared as optional to allow single reconfigurations; in this case omit means "keep as it is" |  |  |
| DL_PRSConfig | DL_PRSConfig_Type | opt |  |

## E.1.3	NR_System_Interface_PosExt

NR_SYSTEM_CTRL_POSEXT_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SYSTEM_CTRL_POSEXT_REQ |  |  |
| Comment |  |  |  |
| Common | NR_ReqAspCommonPart_Type |  | For configuration of DL-PRS TimingInfo is now |
| Request | NR_SystemRequest_PosExt_Type |  | configure a DL-PRS in a cell |

NR_SYSTEM_CTRL_POSEXT_CNF

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SYSTEM_CTRL_POSEXT_CNF |  |  |
| Comment |  |  |  |
| Common | NR_CnfAspCommonPart_Type |  | TimingInfo is ignored by TTCN => SS may set TimingInfo to "None" |
| Confirm | NR_SystemConfirm_PosExt_Type |  |  |

NR_SYSTEM_POSEXT_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NR_SYSTEM_POSEXT_PORT |  |
| Comment | NR PTC: Ext Port for system configuration of DL-PRS |  |
| out | NR_SYSTEM_CTRL_POSEXT_REQ |  |
| in | NR_SYSTEM_CTRL_POSEXT_CNF |  |

# E.2	References to TTCN-3

| References to TTCN-3 |  |  |
| --- | --- | --- |
| NR5GC_POS_ASP_TypeDefs | NR5GC/NR5GC_POS_ASP_TypeDefs.ttcn | Rev 20699 |

Annex F (informative):
Change history