3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210007

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4573 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NBIOT_SS_ConfigureActiveCell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TS 36.523-3 cl. 7A.1, defines a formula to calculate LTE PCI using NBIOT PCI when operating in ‘in-band different PCI’ mode, whereas ttcn is using that formula for ‘in-band same PCI’ operation mode |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed inband_samePCI to inband_differentPCI, in condition where operation mode is checked |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NBIOT TCs cannot be tested over in-band mode |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All NBIOT TCs in in-band mode |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_NBIOT_SS_ConfigureActiveCell |
| --- | --- |
| Reason for change | TS 36.523-3 cl. 7A.1, defines a formula to calculate LTE PCI using NBIOT PCI when operating in ‘in-band different PCI’ mode, whereas ttcn is using that formula for ‘in-band same PCI’ operation mode |
| Summary of change | Changed inband_samePCI to inband_differentPCI, in condition where operation mode is checked |
| TTCN module | NBIOT_ConfigurationSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NBIOT_SS_ConfigureActiveCell(NBIOT_CellId_Type p_CellId) runs on NBIOT_PTC { /* @sic R5s170258 - additional MCC160 changes: attribute DlCcchMsgInSeparateMacPdu removed for NBIOT sic@ */ var NBIOT_CellInfo_Type v_CellInfo := f_NBIOT_CellInfo_Get(p_CellId); var template (value) ContentionResolution_ContainedDlschSdu_Type v_ContentionResolution_ContainedRlcPdu; var template (value) DL_CCCH_Message_NB v_RrcConnSetup; var octetstring v_EncodedRrcConnSetup; var template (omit) NB_CommonStaticLteCellInfo_Type v_LteCellInfo; var EUTRA_ASN1_PhysCellId_Type v_NCellId := f_NBIOT_CellInfo_GetPhyCellId (p_CellId); var EUTRA_ASN1_PhysCellId_Type v_NId1; var EUTRA_ASN1_PhysCellId_Type v_NId2; var EUTRA_ASN1_PhysCellId_Type v_LTEId := v_NCellId; v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(); v_EncodedRrcConnSetup := f_RRC_DL_CCCH_NB_Encvalue(v_RrcConnSetup); v_ContentionResolution_ContainedRlcPdu := cs_ContentionResolution_DlschSdu(v_EncodedRrcConnSetup); // @sic R5-185180 sic@ if (f_NBIOT_OperationMode() == inband_samePCI) { v_NId1 := ((v_NCellId/3)+1) mod 168; v_NId2 := ((v_NCellId - 3 * v_NId1)+1) mod 3; v_LTEId := 3*v_NId1 + v_NId2; } if (f_NBIOT_OperationMode() == standalone) { // @sic R5w180205r1 sic@ v_LteCellInfo := omit; } else { v_LteCellInfo := cs_NB_CommonStaticLteCellInfo (f_NBIOT_CellInfo_GetEUTRA_ARFCN(p_CellId), px_ePrimaryBandChannelBandwidth, normal, v_LTEId, v_CellInfo); } f_NBIOT_SS_CommonCellConfig(p_CellId, cas_NB_CellConfig_Def_REQ(p_CellId, cs_TimingInfo_Now, v_CellInfo, v_ContentionResolution_ContainedRlcPdu, -, -, v_LteCellInfo)); // @sic R5w180205r1 sic@ }

After Change:

function f_NBIOT_SS_ConfigureActiveCell(NBIOT_CellId_Type p_CellId) runs on NBIOT_PTC { /* @sic R5s170258 - additional MCC160 changes: attribute DlCcchMsgInSeparateMacPdu removed for NBIOT sic@ */ var NBIOT_CellInfo_Type v_CellInfo := f_NBIOT_CellInfo_Get(p_CellId); var template (value) ContentionResolution_ContainedDlschSdu_Type v_ContentionResolution_ContainedRlcPdu; var template (value) DL_CCCH_Message_NB v_RrcConnSetup; var octetstring v_EncodedRrcConnSetup; var template (omit) NB_CommonStaticLteCellInfo_Type v_LteCellInfo; var EUTRA_ASN1_PhysCellId_Type v_NCellId := f_NBIOT_CellInfo_GetPhyCellId (p_CellId); var EUTRA_ASN1_PhysCellId_Type v_NId1; var EUTRA_ASN1_PhysCellId_Type v_NId2; var EUTRA_ASN1_PhysCellId_Type v_LTEId := v_NCellId; v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(); v_EncodedRrcConnSetup := f_RRC_DL_CCCH_NB_Encvalue(v_RrcConnSetup); v_ContentionResolution_ContainedRlcPdu := cs_ContentionResolution_DlschSdu(v_EncodedRrcConnSetup); // @sic R5-185180 sic@ if (f_NBIOT_OperationMode() == inband_differentPCI) { v_NId1 := ((v_NCellId/3)+1) mod 168; v_NId2 := ((v_NCellId - 3 * v_NId1)+1) mod 3; v_LTEId := 3*v_NId1 + v_NId2; } if (f_NBIOT_OperationMode() == standalone) { // @sic R5w180205r1 sic@ v_LteCellInfo := omit; } else { v_LteCellInfo := cs_NB_CommonStaticLteCellInfo (f_NBIOT_CellInfo_GetEUTRA_ARFCN(p_CellId), px_ePrimaryBandChannelBandwidth, normal, v_LTEId, v_CellInfo); } f_NBIOT_SS_CommonCellConfig(p_CellId, cas_NB_CellConfig_Def_REQ(p_CellId, cs_TimingInfo_Now, v_CellInfo, v_ContentionResolution_ContainedRlcPdu, -, -, v_LteCellInfo)); // @sic R5w180205r1 sic@ }