3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210074

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1508 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_BuildDefaultQoSRules |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Function f_BuildDefaultQoSRules is called in function f_NR5GC_ModifyPDUSession_AddVoice with arguments 7_Voice and ‘7A’O. However, for the option 7_Voice, the iel for the QoS rule 1 and 2 is not set correctly. This needs to be corrected. As a result of proposed change above, the iel within template cs_QoS_Rule needs to be parameterised. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Parameterized the cs_QoS_Rule, and passed iel '09'O for Qos rule "01" and iel '0f'O for Qos rule “02” when IMS voice call session setup over NR. Add the variable p_Len in the template cs_QoS_Rule and set its default value to ‘0006’O. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IMS voice call Testcases will fail a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | IMS voice call over NR testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	function f_BuildDefaultQoSRules	3

1.2	template cs_QoS_Rule	4

# Corrections required

## function f_BuildDefaultQoSRules

| Function name | f_BuildDefaultQoSRules |
| --- | --- |
| Reason for change | Function f_BuildDefaultQoSRules is called in function f_NR5GC_ModifyPDUSession_AddVoice with arguments 7_Voice and ‘7A’O. However, for the option 7_Voice, the iel for the QoS rule 1 and 2 is not set correctly. This needs to be corrected. |
| Summary of change | Parameterized the cs_QoS_Rule, and passed iel '09'O for Qos rule "01" and iel '0f'O for Qos rule “02” when IMS voice call session setup over NR. |
| TTCN module | Common/NR5GC/ NR5GC_IMSProcedures |
| MCC160 Comment |  |

Before Change:

function f_BuildDefaultQoSRules (charstring p_RuleNum, template (omit) IEI8_Type p_IEI := omit) return template (value) QoS_Rules { var template (value) QoS_Rules v_QoS_Rules; if (p_RuleNum == "7_Voice") { v_QoS_Rules := cs_QoS_Rules ({cs_QoS_Rule('01'O, int2bit(2, 6), 'FF'O), cs_QoS_Rule('02'O, int2bit(5, 6), '01'O, -, cs_PacketFilterList_MediaUDP('0110'B), '0'B)}, p_IEI) } else if (p_RuleNum == "7_Video") { v_QoS_Rules := cs_QoS_Rules ({cs_QoS_Rule('01'O, int2bit(2, 6), 'FF'O), cs_QoS_Rule('03'O, int2bit(6, 6), '01'O, -, cs_PacketFilterList_MediaUDP('0111'B), '0'B)}, p_IEI) } else { v_QoS_Rules := cs_QoS_Rules ({f_BuildDefaultQoSRule(p_RuleNum)}, p_IEI); } return v_QoS_Rules; }

After Change:

function f_BuildDefaultQoSRules (charstring p_RuleNum, template (omit) IEI8_Type p_IEI := omit) return template (value) QoS_Rules { var template (value) QoS_Rules v_QoS_Rules; if (p_RuleNum == "7_Voice") { v_QoS_Rules := cs_QoS_Rules ({cs_QoS_Rule('01'O, int2bit(2, 6), 'FF'O, -, -, -, '09'O), cs_QoS_Rule('02'O, int2bit(5, 6), '01'O, -, cs_PacketFilterList_MediaUDP('0110'B), '0'B, '0F'O)}, p_IEI) } else if (p_RuleNum == "7_Video") { v_QoS_Rules := cs_QoS_Rules ({cs_QoS_Rule('01'O, int2bit(2, 6), 'FF'O), cs_QoS_Rule('03'O, int2bit(6, 6), '01'O, -, cs_PacketFilterList_MediaUDP('0111'B), '0'B)}, p_IEI) } else { v_QoS_Rules := cs_QoS_Rules ({f_BuildDefaultQoSRule(p_RuleNum)}, p_IEI); } return v_QoS_Rules; }

## template cs_QoS_Rule

| Template name | cs_QoS_Rule |
| --- | --- |
| Reason for change | As a result of change 1.1 above, the iel within template cs_QoS_Rule needs to be parameterised. |
| Summary of change | Add the variable p_Len in the template cs_QoS_Rule and set its default value to ‘0006’O. |
| TTCN module | Common/NG_NAS/ NG_NAS_Templates |
| MCC160 Comment |  |

Before Change:

template (value) QoS_Rule cs_QoS_Rule (O1_Type p_Id, B6_Type p_QFI, O1_Type p_Precedence, B3_Type p_OperationCode := '001'B, template (value) NG_PacketFilterList p_FilterList := cs_PacketFilterList_1MatchAll, B1_Type p_DQR := '1'B) := { /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ identifier := p_Id, iel := '0006'O, ruleOperationCode := p_OperationCode, // @sic R5s190987 sic@ dqrBit := p_DQR, numOfPacketFilters := '0001'B, packetFilterList := p_FilterList, precedence := p_Precedence, spare := tsc_Spare1, segregation := tsc_Spare1, flowIdentifier := p_QFI };

After Change:

template (value) QoS_Rule cs_QoS_Rule (O1_Type p_Id, B6_Type p_QFI, O1_Type p_Precedence, B3_Type p_OperationCode := '001'B, template (value) NG_PacketFilterList p_FilterList := cs_PacketFilterList_1MatchAll, B1_Type p_DQR := '1'B, O1_Type p_Len := '0006'O ) := { /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ identifier := p_Id, iel := p_Len, ruleOperationCode := p_OperationCode, // @sic R5s190987 sic@ dqrBit := p_DQR, numOfPacketFilters := '0001'B, packetFilterList := p_FilterList, precedence := p_Precedence, spare := tsc_Spare1, segregation := tsc_Spare1, flowIdentifier := p_QFI };