3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210094

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1528 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR idle mode test case 6.4.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - From the conformance requirements for this test case (from 23.122 4.4.3.3): The attempts to access the HPLMN or an EHPLMN or higher priority PLMN shall be as specified below: ………………… b) The MS shall make the first attempt after a period of at least 2 minutes and at most T minutes: - only after switch on if Fast First Higher Priority PLMN search is disabled; or - after switch on or upon selecting a VPLMN if Fast First Higher Priority PLMN search is enabled. This means the TTCN should take into account an estimate of the elapsed time since UE switch on when starting the minimum (2 minute) timer at step 6. - Following deregistration of the UE, all stored information about DRBs and PDU sessions should be cleared since these no longer exist in the UE. For some test cases this is not being done via a call to f_NR5GC_MobileInfo_Init(), this can give execution issues later in the test case (e.g. AT commands for creating new PDU sessions are not issued) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Start timer on UE switch on and reduce SS timers at step 6 by estimated elapsed time since switch on - Clear stored mobile info before step 1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_6_4_1_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | From the conformance requirements for this test case (from 23.122 4.4.3.3): The attempts to access the HPLMN or an EHPLMN or higher priority PLMN shall be as specified below: ………………… b) The MS shall make the first attempt after a period of at least 2 minutes and at most T minutes: - only after switch on if Fast First Higher Priority PLMN search is disabled; or - after switch on or upon selecting a VPLMN if Fast First Higher Priority PLMN search is enabled. This means the TTCN should take into account an estimate of the elapsed time since UE switch on when starting the minimum (2 minute) timer at step 6. |
| Summary of change | Bring the functionality of f_NR5GC_Preamble_Steps1_13() into the test case function and start a timer immediately after UE switch on. Following preamble signaling, pass the elapsed time into test case body function. |
| TTCN module | Inactive_PLMNSelection_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_4_1_1_NR5GC() runs on NR5GC_PTC {// PLMN Selection/Higher priority/HPLMN in Automatic PLMN Selection Mode var ARFCN_ValueNR v_SSB_Freq_Cell1; var ARFCN_ValueNR v_SSB_Freq_Cell12; var ARFCN_ValueNR v_SSB_Freq_Cell13; var template (value) InterFreqCarrierFreqList v_InterFreqCarrierFreqList; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; f_NR5GC_Init(NR_4); v_SSB_Freq_Cell1 := f_NR_CellInfo_GetFrequencySSB(nr_Cell1); v_SSB_Freq_Cell12 := f_NR_CellInfo_GetFrequencySSB(nr_Cell12); v_SSB_Freq_Cell13 := f_NR_CellInfo_GetFrequencySSB(nr_Cell13); // Set Sys info and cell info for cell 12& 13 & 1 to be used // @sic R5-201349, R5s200657 sic@ f_NR_CellInfo_SetPLMN(nr_Cell12, tsc_NR_HPLMN_001_11);//PLMN15 f_NR_CellInfo_SetPLMN(nr_Cell13, tsc_NR_HPLMN_001_21);//PLMN16 // Table 6.4.1.1.3.3-1: SIB4 for NR Cell 12 v_InterFreqCarrierFreqList := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell3); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell12, v_InterFreqCarrierFreqList); // SIB4 on NR Cell 12 updated with frequency list from SIB4 of nr_Cell3 f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_dl_CarrierFreq(nr_Cell12, 0, v_SSB_Freq_Cell13); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_dl_CarrierFreq(nr_Cell12, 1, v_SSB_Freq_Cell1); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell12, 0, 4); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell12, 1, 4); // Table 6.4.1.1.3.3-2: SIB4 for NR Cell 13 v_InterFreqCarrierFreqList := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell6); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell13, v_InterFreqCarrierFreqList); // SIB4 on NR Cell 13 updated with frequency list from SIB4 of nr_Cell6 f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_dl_CarrierFreq(nr_Cell13, 0, v_SSB_Freq_Cell12); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_dl_CarrierFreq(nr_Cell13, 1, v_SSB_Freq_Cell1); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell13, 0, 4); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell13, 1, 4); // Set maximum cell power level for the Cells to be used // @sic R5s201306 sic@ f_NR_CellInfo_InitMaxReferencePower(nr_Cell13, -78, -78); // FFS values for FR2 // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell12); f_NR_CellConfig_Def(nr_Cell13); // USIM configuration as defined in TS 38.508-1 [4] Table 6.4.1-1 will be loaded f_UT_USIM_Insert(UT, "USIM configuration 1 specified in table 6.4.1-1 TS 38.508-1"); f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT); // The UE is registered on NR Cell 12 except that the REGISTRATION ACCEPT message indicates the PLMN of NR Cell 13 in the Equivalent PLMN list as described in Table 6.4.1.1.3.3-3 // The UE is in state 2N-A as defined in TS 38.508-1, Table 4.4A.2-2 on NR Cell 12 v_GMM_MobilityInfo := f_NR5GC_Preamble_Steps1_13 (nr_Cell12); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, // @sic R5-206426 sic@ f_NR5GC_GetMobileIdGUTI (nr_Cell12), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12)), {bit2oct(f_NR_CellInfo_GetTAC(nr_Cell12))}), cs_PLMN_List_1PLMN(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell13)))); // Table 6.4.1.1.3.3-3 SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); f_NR5GC_RRC_Idle_Steps15_20(nr_Cell12, TESTMode_OFF, noRrcConnectionRelease); // @sic R5s200657 sic@ f_NR_RRC_InactiveDef(nr_Cell12, -, false); // @sic R5s200657 sic@ f_NR_TestBody_Set(true); f_TC_6_4_1_1_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); }

After Change:

function f_TC_6_4_1_1_NR5GC() runs on NR5GC_PTC {// PLMN Selection/Higher priority/HPLMN in Automatic PLMN Selection Mode var ARFCN_ValueNR v_SSB_Freq_Cell1; var ARFCN_ValueNR v_SSB_Freq_Cell12; var ARFCN_ValueNR v_SSB_Freq_Cell13; var template (value) InterFreqCarrierFreqList v_InterFreqCarrierFreqList; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; timer t_WaitTime; var float v_ElaspedTime; f_NR5GC_Init(NR_4); v_SSB_Freq_Cell1 := f_NR_CellInfo_GetFrequencySSB(nr_Cell1); v_SSB_Freq_Cell12 := f_NR_CellInfo_GetFrequencySSB(nr_Cell12); v_SSB_Freq_Cell13 := f_NR_CellInfo_GetFrequencySSB(nr_Cell13); // Set Sys info and cell info for cell 12& 13 & 1 to be used // @sic R5-201349, R5s200657 sic@ f_NR_CellInfo_SetPLMN(nr_Cell12, tsc_NR_HPLMN_001_11);//PLMN15 f_NR_CellInfo_SetPLMN(nr_Cell13, tsc_NR_HPLMN_001_21);//PLMN16 // Table 6.4.1.1.3.3-1: SIB4 for NR Cell 12 v_InterFreqCarrierFreqList := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell3); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell12, v_InterFreqCarrierFreqList); // SIB4 on NR Cell 12 updated with frequency list from SIB4 of nr_Cell3 f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_dl_CarrierFreq(nr_Cell12, 0, v_SSB_Freq_Cell13); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_dl_CarrierFreq(nr_Cell12, 1, v_SSB_Freq_Cell1); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell12, 0, 4); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell12, 1, 4); // Table 6.4.1.1.3.3-2: SIB4 for NR Cell 13 v_InterFreqCarrierFreqList := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell6); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell13, v_InterFreqCarrierFreqList); // SIB4 on NR Cell 13 updated with frequency list from SIB4 of nr_Cell6 f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_dl_CarrierFreq(nr_Cell13, 0, v_SSB_Freq_Cell12); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_dl_CarrierFreq(nr_Cell13, 1, v_SSB_Freq_Cell1); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell13, 0, 4); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell13, 1, 4); // Set maximum cell power level for the Cells to be used // @sic R5s201306 sic@ f_NR_CellInfo_InitMaxReferencePower(nr_Cell13, -78, -78); // FFS values for FR2 // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell12); f_NR_CellConfig_Def(nr_Cell13); // USIM configuration as defined in TS 38.508-1 [4] Table 6.4.1-1 will be loaded f_UT_USIM_Insert(UT, "USIM configuration 1 specified in table 6.4.1-1 TS 38.508-1"); f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT); // The UE is registered on NR Cell 12 except that the REGISTRATION ACCEPT message indicates the PLMN of NR Cell 13 in the Equivalent PLMN list as described in Table 6.4.1.1.3.3-3 // The UE is in state 2N-A as defined in TS 38.508-1, Table 4.4A.2-2 on NR Cell 12 // turn on NR cell f_NR_SetCellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // Switch on UE f_NR5GC_SwitchOnAndResetIMS(); t_WaitTime.start(60.0); // Steps 2 - 4 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell12, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // Steps 5 - 13 f_NR5GC_RRC_Idle_Steps5_13(v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12)), TESTMode_OFF, Initial_NoSecurity, NOT_SUPPORTED); // @sic R5-202564 sic@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, // @sic R5-206426 sic@ f_NR5GC_GetMobileIdGUTI (nr_Cell12), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12)), {bit2oct(f_NR_CellInfo_GetTAC(nr_Cell12))}), cs_PLMN_List_1PLMN(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell13)))); // Table 6.4.1.1.3.3-3 SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); f_NR5GC_RRC_Idle_Steps15_20(nr_Cell12, TESTMode_OFF, noRrcConnectionRelease); // @sic R5s200657 sic@ f_NR_RRC_InactiveDef(nr_Cell12, -, false); // @sic R5s200657 sic@ f_NR_TestBody_Set(true); v_ElaspedTime := t_WaitTime.read(); t_WaitTime.stop(); f_TC_6_4_1_1_NR5GC_TestBody(v_ElaspedTime); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); }

## Change 2

| Function name | f_TC_6_4_1_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | - Following deregistration of the UE, all stored information about DRBs and PDU sessions should be cleared since these no longer exist in the UE. For some test cases this is not being done via a call to f_NR5GC_MobileInfo_Init(), this can give execution issues later in the test case (e.g. AT commands for creating new PDU sessions are not issued) - From the conformance requirements for this test case (from 23.122 4.4.3.3): The attempts to access the HPLMN or an EHPLMN or higher priority PLMN shall be as specified below: ………………… b) The MS shall make the first attempt after a period of at least 2 minutes and at most T minutes: - only after switch on if Fast First Higher Priority PLMN search is disabled; or - after switch on or upon selecting a VPLMN if Fast First Higher Priority PLMN search is enabled. This means the TTCN should take into account an estimate of the elapsed time since UE switch on when starting the minimum (2 minute) timer at step 6. |
| Summary of change | - Before step 1, add call to f_NR5GC_MobileInfo_Init() - Reduce t_Tmin and t_Tmax by estimated elapsed time from SWITCH ON, see change 1 |
| TTCN module | Inactive_PLMNSelection_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_4_1_1_NR5GC_TestBody() runs on NR5GC_PTC { // Power levels on FR2 are FFS // @sic R5s201047, R5-203382 sic@ var template (value) NR_CellPowerList_Type v_CellPowerList_T0; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (omit) NG_NAS_MSG_Indication_Type v_ReceivedMsgTemplate; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; timer t_Tmin; timer t_Tmax; v_CellPowerList_T0 := { cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2), // FR2 FFS cs_NR_CellPower (nr_Cell13, -78, -78), cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell13, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; f_NR_SS_AS_ActivateSecurity_Current(nr_Cell13 ,tsc_NR_38508_NextHopChainingCount); // @sic R5s200657 sic@ // @siclog "Step 1" siclog@ // SS adjusts cell levels according to row T0 of table 6.4.1.1.3.2-1/2 f_NR_SetCellPowerList(v_CellPowerList_T0); // @siclog "Step 2" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on NR Cell 13? f_NR_RRC_ResumeRequest_Def(nr_Cell13, -, mo_Signalling); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Step 3" siclog@ // The SS transmits an RRCResume message f_NR_SendRRCResume_Def(nr_Cell13); // @siclog "Step 4" siclog@ // The UE transmits an RRCResumeComplete message // @sic R5s200657 sic@ v_ReceivedMsgTemplate := f_NR_RRCResumeComplete_Def(nr_Cell13); v_ReceivedMsg := valueof(v_ReceivedMsgTemplate); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // @siclog "Step 4A-4B" siclog@ // Steps 4 to 5 of the generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 with condition MOBILITY are performed. f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell13, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease, -, tsc_NR_RbId_SRB2); // @sic R5s200657 sic@ // @siclog "Step 5" siclog@ // The SS transmits an an RRCRelease message with suspend f_NR_RRC_InactiveDef(nr_Cell13); // @siclog "Step 6" siclog@ // SS starts timers of tmin = 2min and tmax = (6 minutes + 5min) t_Tmin.start(f_NR_SetTimerToleranceMin(nr_Cell13, nonProtocolTimer, 120.0)); t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell13, nonProtocolTimer, 660.0)); // @siclog "Step 7" siclog@ // The SS transmits a Paging message including a matched identity (correct fullI-RNTI) on NR Cell 13 f_NR_Paging_Def(nr_Cell13, RRC_INACTIVE); // @siclog "Step 8" siclog@ // Check: Does the UE transmit an RRCResumeRequest message to resume RRC Connection by setting resumeIdentity to the stored shortI-RNTI value on NR Cell 13? f_NR_RRC_ResumeRequest_Def(nr_Cell13, tsc_NR_ShortI_RNTI_Value1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); // @siclog "Step 9" siclog@ // The SS transmits an RRCResume message f_NR_SendRRCResume_Def(nr_Cell13); // @siclog "Step 10" siclog@ // The UE transmits an RRCResumeComplete message f_NR_RRCResumeComplete_Def(nr_Cell13); // @siclog "Step 11" siclog@ // The SS transmits an RRCRelease message with suspend f_NR_RRC_InactiveDef(nr_Cell13); // @siclog "Step 12" siclog@ // SS adjusts cell levels according to row T1 of table 6.4.1.1.3.2-1/2 f_NR_SetCellPowerList(v_CellPowerList_T1); // @siclog "Step 13" siclog@ // Check: Does the UE send a RRCSetupRequest on NR Cell 1 before tmax expires? alt { [] SRB.check(receive( car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { //verdict fail RRCSetupRequest received on PLMNN4 /NR Cell 1 within tmin f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.4.1.1 Step 13"); } [] t_Tmin.timeout { alt { [] SRB.check(receive( car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Tmax.stop; // verdict pass RRCSetupRequest received on PLMN4 /NR Cell 1 after t_Tmin time but before tmax f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.4.1.1 Step 13"); } [] t_Tmax.timeout { // verdict fail No RRCSetupRequest received f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.4.1.1 Step 13"); } } } } // @siclog "Step 14-14b5a1" siclog@ f_NR5GC_InitialOrMobilityRegistration(nr_Cell1); }

After Change:

function f_TC_6_4_1_1_NR5GC_TestBody(float p_ElaspedTime) runs on NR5GC_PTC { // Power levels on FR2 are FFS // @sic R5s201047, R5-203382 sic@ var template (value) NR_CellPowerList_Type v_CellPowerList_T0; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (omit) NG_NAS_MSG_Indication_Type v_ReceivedMsgTemplate; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; timer t_Tmin; timer t_Tmax; // SS starts timers of tmin = 2min and tmax = (6 minutes + 5min) t_Tmin.start(f_NR_SetTimerToleranceMin(nr_Cell13, nonProtocolTimer, 120.0) - p_ElaspedTime); t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell13, nonProtocolTimer, 660.0) - p_ElaspedTime); v_CellPowerList_T0 := { cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2), // FR2 FFS cs_NR_CellPower (nr_Cell13, -78, -78), cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell13, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; f_NR_SS_AS_ActivateSecurity_Current(nr_Cell13 ,tsc_NR_38508_NextHopChainingCount); // @sic R5s200657 sic@ f_NR5GC_MobileInfo_Init(); // @siclog "Step 1" siclog@ // SS adjusts cell levels according to row T0 of table 6.4.1.1.3.2-1/2 f_NR_SetCellPowerList(v_CellPowerList_T0); // @siclog "Step 2" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on NR Cell 13? f_NR_RRC_ResumeRequest_Def(nr_Cell13, -, mo_Signalling); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Step 3" siclog@ // The SS transmits an RRCResume message f_NR_SendRRCResume_Def(nr_Cell13); // @siclog "Step 4" siclog@ // The UE transmits an RRCResumeComplete message // @sic R5s200657 sic@ v_ReceivedMsgTemplate := f_NR_RRCResumeComplete_Def(nr_Cell13); v_ReceivedMsg := valueof(v_ReceivedMsgTemplate); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // @siclog "Step 4A-4B" siclog@ // Steps 4 to 5 of the generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 with condition MOBILITY are performed. f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell13, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease, -, tsc_NR_RbId_SRB2); // @sic R5s200657 sic@ // @siclog "Step 5" siclog@ // The SS transmits an an RRCRelease message with suspend f_NR_RRC_InactiveDef(nr_Cell13); // @siclog "Step 6" siclog@ // @siclog "Step 7" siclog@ // The SS transmits a Paging message including a matched identity (correct fullI-RNTI) on NR Cell 13 f_NR_Paging_Def(nr_Cell13, RRC_INACTIVE); // @siclog "Step 8" siclog@ // Check: Does the UE transmit an RRCResumeRequest message to resume RRC Connection by setting resumeIdentity to the stored shortI-RNTI value on NR Cell 13? f_NR_RRC_ResumeRequest_Def(nr_Cell13, tsc_NR_ShortI_RNTI_Value1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); // @siclog "Step 9" siclog@ // The SS transmits an RRCResume message f_NR_SendRRCResume_Def(nr_Cell13); // @siclog "Step 10" siclog@ // The UE transmits an RRCResumeComplete message f_NR_RRCResumeComplete_Def(nr_Cell13); // @siclog "Step 11" siclog@ // The SS transmits an RRCRelease message with suspend f_NR_RRC_InactiveDef(nr_Cell13); // @siclog "Step 12" siclog@ // SS adjusts cell levels according to row T1 of table 6.4.1.1.3.2-1/2 f_NR_SetCellPowerList(v_CellPowerList_T1); // @siclog "Step 13" siclog@ // Check: Does the UE send a RRCSetupRequest on NR Cell 1 before tmax expires? alt { [] SRB.check(receive( car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { //verdict fail RRCSetupRequest received on PLMNN4 /NR Cell 1 within tmin f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.4.1.1 Step 13"); } [] t_Tmin.timeout { alt { [] SRB.check(receive( car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Tmax.stop; // verdict pass RRCSetupRequest received on PLMN4 /NR Cell 1 after t_Tmin time but before tmax f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.4.1.1 Step 13"); } [] t_Tmax.timeout { // verdict fail No RRCSetupRequest received f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.4.1.1 Step 13"); } } } } // @siclog "Step 14-14b5a1" siclog@ f_NR5GC_InitialOrMobilityRegistration(nr_Cell1); }