3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210153

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1560 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EAP authentication calculation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The function f_NG_EAP_SuccessfulAuthenticationResponse decodes the EAP response, replaces the MAC attribute with an empty value, then re-encodes the message in order to match this value against the received AT_MAC in the received EAP response. As the EAP attributes are defined as a set, the order of the attributes included in the decoded EAP response sent to the TTCN will be listed in the order defined in the set, not in the order as they were received in the un-decoded message. This can result in the MAC attribute being re-encoded in a different position in the EAP response. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Create a local function to find and clear the MAC value in the un-decoded message, rather than decoding and re-encoding the message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail EAP authentication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All NR5GC test cases using EAP authentication |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

| Function name | f_NG_EAP_SuccessfulAuthenticationResponse |
| --- | --- |
| Reason for change | As the EAP attributes are defined as a set, the order of the attributes included in the decoded EAP response sent to the TTCN will be listed in the order defined in the set, not in the order as they were received in the un-decoded message. This can result in the MAC attribute being re-encoded in a different position in the EAP response. |
| Summary of change | Create a local function to find and clear the MAC value in the un-decoded message, rather than decoding and re-encoding the message |
| TTCN module | NG_NAS_SecurityFunctions |

Before Change

function f_NG_EAP_SuccessfulAuthenticationResponse(inout NG_NAS_SecurityParams_Type p_Auth_Params, octetstring p_ReceivedEAP, template EAP_AT_RESULT_IND p_ResultInd := *, O1_Type p_Id := '00'O) return boolean { var boolean v_Result := false; var bitstring v_ReceivedEAP := oct2bit(p_ReceivedEAP); var O16_Type v_RxdMAC; var O16_Type v_MACValue := '00000000000000000000000000000000'O; var bitstring v_K_aut; var EAP_Message_Type v_EAP_Message; var B32_128_Type v_RESValue; var bitstring v_EncodedMsg; var integer v_Len; var integer v_Paddinglen; var integer i; if (decvalue(v_ReceivedEAP, v_EAP_Message) != 0) { FatalError(__FILE__, __LINE__, "EAP message cannot be decoded"); } //get length in bits of received RES (as it's a variable length, need to compare the correct length) v_Len := oct2int(v_EAP_Message.akaChallenge_Rsp.attributes.res.reslen); // store this in the global info p_Auth_Params.AuthParams.XRESLength := v_Len / 8; // Get the RES value from the global info v_RESValue := substr(p_Auth_Params.AuthParams.XRES, 0, v_Len); // Now pad it so that it's (hopefully) the same length of the received value if (v_Len mod 8 > 0 or (v_Len/8) mod 4 > 0) { v_Paddinglen := 8 - v_Len mod 8; // pad to end of octet, in bits v_Paddinglen := (4 - ((v_Len + v_Paddinglen)/8) mod 4)*8; // pad to multiple of 4 octets, in bits for (i:=0; i < v_Paddinglen; i:=i+1) { v_RESValue := v_RESValue & '0'B; } } // Check message is AKAChallenge, length and AT_RES is correct if (match (v_EAP_Message, cr_EAP_Response_AKAChallenge(int2oct(lengthof(p_ReceivedEAP), 2), cr_EAP_AT_RES (bit2oct(v_RESValue), int2oct(v_Len, 2)), ?, p_ResultInd, p_Id))) { // @sic R5-201142, R5s201483 sic@ // Then test MAC v_RxdMAC := v_EAP_Message.akaChallenge_Rsp.attributes.mac.mac; // First build the message with MAC=0 v_EAP_Message.akaChallenge_Rsp.attributes.mac.mac := v_MACValue; // Compute MAC according to RFC 5448 cl. 3.4.2 v_EncodedMsg := encvalue(v_EAP_Message); v_K_aut := substr(p_Auth_Params.MK, 128, 256); // RFC 5448 cl. 3.3 K_aut to be used for computing MAC @sic R5s200400 sic@ v_MACValue := bit2oct(substr(f_HMAC_SHA_256(v_K_aut, bit2oct(v_EncodedMsg)), 0, 128)); // @sic R5s200400 sic@ // Then test it against the received value if (match(v_RxdMAC, v_MACValue)) { // if the MAC also matches then the whole message is correct v_Result := true; } } return v_Result; }

After Change

function fl_NG_EAP_Response_ClearMACValue(octetstring p_ReceivedEAP) return octetstring { var octetstring v_ModifiedEAP := p_ReceivedEAP; var integer i := 8; /* according to RFC 4187 clause 8.1 the EAP-AKA header is 8 octets */ var integer k; while (p_ReceivedEAP[i] != '0B'O) { /* MAC attribute type is 11 according to RFC 4187 clause 11 */ i := i + (oct2int(p_ReceivedEAP[i+1]) * 4); if (i > lengthof(p_ReceivedEAP)) { FatalError(__FILE__, __LINE__, "no MAC attribute found"); } } i := i + 4; /* first 4 octets are type, length, reserved (RFC 4187 clause 10.15) */ for (k := 0; k < 16; k := k + 1) { v_ModifiedEAP[i+k] := '00'O; } return v_ModifiedEAP; } function f_NG_EAP_SuccessfulAuthenticationResponse(inout NG_NAS_SecurityParams_Type p_Auth_Params, octetstring p_ReceivedEAP, template EAP_AT_RESULT_IND p_ResultInd := *, O1_Type p_Id := '00'O) return boolean { var boolean v_Result := false; var bitstring v_ReceivedEAP := oct2bit(p_ReceivedEAP); var O16_Type v_RxdMAC; var O16_Type v_MACValue := '00000000000000000000000000000000'O; var bitstring v_K_aut; var EAP_Message_Type v_EAP_Message; var B32_128_Type v_RESValue; var octetstring v_ModifiedEAP; var integer v_Len; var integer v_Paddinglen; var integer i; if (decvalue(v_ReceivedEAP, v_EAP_Message) != 0) { FatalError(__FILE__, __LINE__, "EAP message cannot be decoded"); } //get length in bits of received RES (as it's a variable length, need to compare the correct length) v_Len := oct2int(v_EAP_Message.akaChallenge_Rsp.attributes.res.reslen); // store this in the global info p_Auth_Params.AuthParams.XRESLength := v_Len / 8; // Get the RES value from the global info v_RESValue := substr(p_Auth_Params.AuthParams.XRES, 0, v_Len); // Now pad it so that it's (hopefully) the same length of the received value if (v_Len mod 8 > 0 or (v_Len/8) mod 4 > 0) { v_Paddinglen := 8 - v_Len mod 8; // pad to end of octet, in bits v_Paddinglen := (4 - ((v_Len + v_Paddinglen)/8) mod 4)*8; // pad to multiple of 4 octets, in bits for (i:=0; i < v_Paddinglen; i:=i+1) { v_RESValue := v_RESValue & '0'B; } } // Check message is AKAChallenge, length and AT_RES is correct if (match (v_EAP_Message, cr_EAP_Response_AKAChallenge(int2oct(lengthof(p_ReceivedEAP), 2), cr_EAP_AT_RES (bit2oct(v_RESValue), int2oct(v_Len, 2)), ?, p_ResultInd, p_Id))) { // @sic R5-201142, R5s201483 sic@ // Then test MAC v_RxdMAC := v_EAP_Message.akaChallenge_Rsp.attributes.mac.mac; // First build the message with MAC=0 v_ModifiedEAP := fl_NG_EAP_Response_ClearMACValue(p_ReceivedEAP); // Compute MAC according to RFC 5448 cl. 3.4.2 v_K_aut := substr(p_Auth_Params.MK, 128, 256); // RFC 5448 cl. 3.3 K_aut to be used for computing MAC @sic R5s200400 sic@ v_MACValue := bit2oct(substr(f_HMAC_SHA_256(v_K_aut, bit2oct(v_ModifiedEAP)), 0, 128)); // @sic R5s200400 sic@ // Then test it against the received value if (match(v_RxdMAC, v_MACValue)) { // if the MAC also matches then the whole message is correct v_Result := true; } } return v_Result; }