3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210180

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1375 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC IRAT test case 6.2.3.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-11-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current implementation of step 17 uses f_EUTRA_RRC_RRCConnectionRequest_Check() with p_JustCheck=false. This doesn’t work because RRCConnectionRequest is also received in f_EUTRA38_TAUReqFromN1(). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implement the check of step 17 using a wait timer. Note : This is reinstating the implementation that was in IWD 20wk25. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.9.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_6_2_3_9_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | The current implementation of step 17 uses f_EUTRA_RRC_RRCConnectionRequest_Check() with p_JustCheck=false. This doesn’t work because RRCConnectionRequest is also received in f_EUTRA38_TAUReqFromN1(). |
| Summary of change | Implement the check of step 17 using a wait timer. Note : This is reinstating the implementation that was in IWD 20wk25. |
| TTCN module | Idle_CellReSelection_NR5GC_EUTRA |
| MCC160 Comment |  |

Before Change

function fl_TC_6_2_3_9_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { // @sic R5s200607 R5-201344 R5s201004 sic@ …… //@siclog "Step 17" siclog@ // Check: Does the UE send an RRCConnectionRequest on E-UTRA Cell 1 within 33s after step 16? (Note 7) v_WaitTime_Cell_Reselection := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 33.0); if (f_EUTRA_RRC_RRCConnectionRequest_Check(eutra_Cell1, v_WaitTime_Cell_Reselection)){ f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "NR5GC IRAT Test Case 6.2.3.9 Step 17 - UE sent RRCConnectionRequest message on Cell 1 within expected interval on Table 6.2.3.9.3.2-3 Note 9"); } else { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "NR5GC IRAT Test Case 6.2.3.9 Step 17 - UE did not sent RRCConnectionRequest message on Cell 1"); } //@siclog "Step 18" siclog@ // Steps 2 to 6 of the generic test procedure in TS 38.508-1 Table 4.9.7.2.2-1 are performed on E-UTRA cell 1. f_EUTRA38_TrackingAreaUpdateFromN1 (eutra_Cell1, tsc_RRC_TI_Def,- ,-, noRrcConnectionRelease, -, -, -, false); // @sic R5-202563 sic@ }

After Change

function fl_TC_6_2_3_9_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { // @sic R5s200607 R5-201344 R5s201004 sic@ ……………….. //@siclog "Step 17" siclog@ // Check: Does the UE send an RRCConnectionRequest on E-UTRA Cell 1 within 33s after step 16? (Note 7) v_WaitTime_Cell_Reselection := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 33.0); /* REMOVE if (f_EUTRA_RRC_RRCConnectionRequest_Check(eutra_Cell1, v_WaitTime_Cell_Reselection)){ f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "NR5GC IRAT Test Case 6.2.3.9 Step 17 - UE sent RRCConnectionRequest message on Cell 1 within expected interval on Table 6.2.3.9.3.2-3 Note 9"); } else { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "NR5GC IRAT Test Case 6.2.3.9 Step 17 - UE did not sent RRCConnectionRequest message on Cell 1"); } */ t_Wait.start(v_WaitTime_Cell_Reselection); //@siclog "Step 18" siclog@ // Steps 2 to 6 of the generic test procedure in TS 38.508-1 Table 4.9.7.2.2-1 are performed on E-UTRA cell 1. f_EUTRA38_TrackingAreaUpdateFromN1 (eutra_Cell1, tsc_RRC_TI_Def,- ,-, noRrcConnectionRelease, -, -, -, false); // @sic R5-202563 sic@ //KS R5sxxx change_6239 t_Wait.stop; // If this times out it will be caught in the default behaviour f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "NR5GC IRAT Test Case 6.2.3.9 Step 17 - UE sent RRCConnectionRequest message on Cell 1 within expected interval on Table 6.2.3.9.3.2-3 Note 9"); }