3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210225

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4566 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to template cs_NB_SiSchedulList |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Current implementation of template cs_NB_SiSchedulList sets 2nd input parameter, p_SiRadioFrameOffset, as a mandatory parameter. According to ASN.1 definition of SystemInformationBlockType1-NB: si-RadioFrameOffset-r13 INTEGER (1..15) OPTIONAL, That means that p_SiRadioFrameOffset should be an integer value 1-15 or omit. Right now, “omit” value will cause a runtime error as p_SiRadioFrameOffset, according to definition, must be present. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added “template (omit)” to 2nd input paramater in template definition to allow “omit” value. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may FAIL the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All NBIOT testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	1

1	Corrections	1

1.1	Correction to template cs_NB_SiSchedulList	1

# Corrections

## Correction to template cs_NB_SiSchedulList

| Template name | Correction to template cs_NB_SiSchedulList |
| --- | --- |
| Reason for change | “omit” value will cause a runtime error as p_SiRadioFrameOffset, according to definition, must be present. |
| Summary of change | Added “template (omit)” to 2nd input paramater in template definition to allow “omit” value. |
| TTCN module | NBIOT/common/NBIOT_CellCfg_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> template (value) NB_AllSiSchedul_Type cs_NB_SiSchedulList(NB_SiWindowLength_Type p_SiWindowLength, /* as in Sib1 */ NB_SiRadioFrameOffset_Type p_SiRadioFrameOffset, /* as in Sib1 */ template (value) NB_SiSchedulList_Type p_SiSchedulList) := { /* @status APPROVED (NBIOT) */ /* common template for one or more SIs */ WindowLength := p_SiWindowLength, RadioFrameOffset := p_SiRadioFrameOffset, SiList := p_SiSchedulList }; <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) NB_AllSiSchedul_Type cs_NB_SiSchedulList(NB_SiWindowLength_Type p_SiWindowLength, /* as in Sib1 */ template (omit) NB_SiRadioFrameOffset_Type p_SiRadioFrameOffset, /* as in Sib1 */ template (value) NB_SiSchedulList_Type p_SiSchedulList) := { /* @status APPROVED (NBIOT) */ /* common template for one or more SIs */ WindowLength := p_SiWindowLength, RadioFrameOffset := p_SiRadioFrameOffset, SiList := p_SiSchedulList }; <<SKIPPED CODE>>