3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210234

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1580 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_Get_NG_DLNASTransport |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of 6.3.1.8.NR5GC, fl_TC_6_3_1_8_TestBodyf_Get_NG_DLNASTransport (cs_PayloadContainerType(tsc_PayloadContainerSORMsg), omit, cs_PayloadContainer(omit, v_NAS_TxMsgContainer_SOR)); is called. The function f_Get_NG_DLNASTransport returns the v_DL_Pdu based on assignment : v_DL_Pdu := cs_NG_NAS_DL_PduWithPiggybacking (v_GMMMsg, p_PiggyBackedGSMMsg); This results in the highlighted IE being set to OMIT in template cs_NG_NAS_DL_PduWithPiggybacking : template (value) NG_NAS_DL_Pdu_Type cs_NG_NAS_DL_PduWithPiggybacking(template (value) NG_NAS_DL_Message_Type p_GmmMsg, template (omit) NG_NAS_DL_Message_Type p_PiggyBackedMsg) := { /* @status APPROVED (IMS, NR5GC, NR5GC_IRAT, POS) */ Msg := p_GmmMsg, PiggybackedPduList := { { Msg := p_PiggyBackedMsg, PiggybackedPduList := omit } } }; This causes a runtime error, because parameter p_PiggyBackedGSMMsg is set to OMIT whereas according to ASN.1 this is not an optional IE. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to use a different template in case of Piggybacking in PDU is not included. Created a new template cs_NG_NAS_DL_PduWithuotPiggybacking and depending on value of parameter p_PiggyBackedGSMMsg either used this new template cs_NG_NAS_DL_PduWithuotPiggybacking, or used existing template cs_NG_NAS_DL_PduWithPiggybacking. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case(s) may end with a runtime error. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.8.NR5GC, 6.3.1.9.NR5GC and 9.1.8.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This issue also affects testcases 6.3.1.9.NR5GC and 9.1.8.1.NR5GC |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	4

1.1	Function f_Get_NG_DLNASTransport	4

1.2	Template cs_NG_NAS_DL_PduWithoutPiggybacking	6

# Corrections required

## Function f_Get_NG_DLNASTransport

| Function name | f_Get_NG_DLNASTransport |
| --- | --- |
| Reason for change | In the current TTCN implementation of 6.3.1.8.NR5GC, fl_TC_6_3_1_8_TestBodyf_Get_NG_DLNASTransport (cs_PayloadContainerType(tsc_PayloadContainerSORMsg), omit, cs_PayloadContainer(omit, v_NAS_TxMsgContainer_SOR)); is called. The function f_Get_NG_DLNASTransport returns the v_DL_Pdu based on assignment : v_DL_Pdu := cs_NG_NAS_DL_PduWithPiggybacking (v_GMMMsg, p_PiggyBackedGSMMsg); This results in the highlighted IE being set to OMIT in template cs_NG_NAS_DL_PduWithPiggybacking : template (value) NG_NAS_DL_Pdu_Type cs_NG_NAS_DL_PduWithPiggybacking(template (value) NG_NAS_DL_Message_Type p_GmmMsg, template (omit) NG_NAS_DL_Message_Type p_PiggyBackedMsg) := { /* @status APPROVED (IMS, NR5GC, NR5GC_IRAT, POS) */ Msg := p_GmmMsg, PiggybackedPduList := { { Msg := p_PiggyBackedMsg, PiggybackedPduList := omit } } }; This causes a runtime error, because parameter p_PiggyBackedGSMMsg is set to OMIT whereas according to ASN.1 this is not an optional IE. This needs to be corrected. |
| Summary of change | It is proposed to use a different template in case of Piggybacking in PDU is not included. Created a new template cs_NG_NAS_DL_PduWithuotPiggybacking and depending on value of parameter p_PiggyBackedGSMMsg either used this new template cs_NG_NAS_DL_PduWithuotPiggybacking, or used existing template cs_NG_NAS_DL_PduWithPiggybacking. |
| TTCN module | Common/NG_NAS/NG_NASTemplateFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_Get_NG_DLNASTransport(template (value) PayloadContainerType p_PayloadContainerType, template (omit) NG_NAS_DL_Message_Type p_PiggyBackedGSMMsg, template (value) PayloadContainer p_Payload := cs_DummyPayloadContainer, // dummy value for p_PiggyBackedGSMMsg is encoded in emulator template (omit) O1_Type p_PDU_SessionId := omit, template (omit) AdditionalInformation p_AdditionalInfo := omit, template (omit) GMM_GSM_Cause p_Cause := omit, template (omit) GPRS_Timer3 p_BackOff := omit) return template (value) NG_NAS_DL_Pdu_Type { var template (omit) NG_PDU_SessionId v_PDU_SessionId := omit; var template (value) NG_NAS_DL_Message_Type v_GMMMsg; if (match(tsc_PayloadContainerESMMsg, p_PayloadContainerType.container) and isvalue(p_PDU_SessionId)) { // @sic R5s190626 sic@ v_PDU_SessionId := cs_NG_PDU_SessionId(p_PDU_SessionId); } v_GMMMsg := cs_NG_DL_NAS_TRANSPORT (p_PayloadContainerType, p_Payload, v_PDU_SessionId, p_AdditionalInfo, p_Cause, p_BackOff); return cs_NG_NAS_DL_PduWithPiggybacking (v_GMMMsg, p_PiggyBackedGSMMsg); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_Get_NG_DLNASTransport(template (value) PayloadContainerType p_PayloadContainerType, template (omit) NG_NAS_DL_Message_Type p_PiggyBackedGSMMsg, template (value) PayloadContainer p_Payload := cs_DummyPayloadContainer, // dummy value for p_PiggyBackedGSMMsg is encoded in emulator template (omit) O1_Type p_PDU_SessionId := omit, template (omit) AdditionalInformation p_AdditionalInfo := omit, template (omit) GMM_GSM_Cause p_Cause := omit, template (omit) GPRS_Timer3 p_BackOff := omit) return template (value) NG_NAS_DL_Pdu_Type { var template (omit) NG_PDU_SessionId v_PDU_SessionId := omit; var template (value) NG_NAS_DL_Message_Type v_GMMMsg; var template (value) NG_NAS_DL_Pdu_Type v_DL_Pdu; if (match(tsc_PayloadContainerESMMsg, p_PayloadContainerType.container) and isvalue(p_PDU_SessionId)) { // @sic R5s190626 sic@ v_PDU_SessionId := cs_NG_PDU_SessionId(p_PDU_SessionId); } v_GMMMsg := cs_NG_DL_NAS_TRANSPORT (p_PayloadContainerType, p_Payload, v_PDU_SessionId, p_AdditionalInfo, p_Cause, p_BackOff); if(isvalue(p_PiggyBackedGSMMsg)){ v_DL_Pdu := cs_NG_NAS_DL_PduWithPiggybacking (v_GMMMsg, p_PiggyBackedGSMMsg); }else{ v_DL_Pdu := cs_NG_NAS_DL_PduWithoutPiggybacking (v_GMMMsg); } return v_DL_Pdu; } <<SKIPPED CODE>>

## Template cs_NG_NAS_DL_PduWithoutPiggybacking

| Template name | cs_NG_NAS_DL_PduWithoutPiggybacking |
| --- | --- |
| Reason for change | Please see change 1.1 |
| Summary of change | New template |
| TTCN module | Common/NG_NAS/NGNASTemplates.ttcn |
| MCC160 Comment |  |

template (value) NG_NAS_DL_Pdu_Type cs_NG_NAS_DL_PduWithuotPiggybacking(template (value) NG_NAS_DL_Message_Type p_GmmMsg) := { /* @status APPROVED (IMS, NR5GC, NR5GC_IRAT, POS) */ Msg := p_GmmMsg, PiggybackedPduList := omit };