3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210344

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1623 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC test case 10.2.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In case of IMS single PDN during initial registration, linked EPS bearer ID at Steps 1, 4, and 7 should be ‘5’H instead of ‘C’H. #associated proseCR shall be raised for next RAN5 meeting |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add condition to set linked EPS bearer ID to ‘5’H if test is single PDN and IMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.2.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_10_2_2_1_ENDC_EUTRA_TestBody |
| --- | --- |
| Reason for change | In case of IMS single PDN during initial registration, linked EPS bearer ID at Steps 1, 4, and 7 should be ‘5’H instead of ‘C’H. |
| Summary of change | Add condition to set linked EPS bearer ID to ‘5’H if test is single PDN and IMS. |
| TTCN module | ESM_ENDC_EUTRA |
| MCC160 Comment |  |

Before change:

function fl_TC_10_2_2_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{

var ProcedureTransactionIdentifier v_EpsPti;

var EUTRA38_SecurityParamsExtd_Type v_Security_Params;

var template (value) NAS_DL_Message_Type v_ModifyEPSBearerCxtReq;

var template (value)  NAS_MSG_Request_Type v_ActivateBearer;

var SRB_COMMON_IND v_ReceivedAsp;

var template (omit) RadioResourceConfigDedicated v_RadioResourceConfigDedicated; //@sic R5s190323 sic@

// Cause the UE to request bearer resource allocation of dedicated EPS bearer associated with first PDN connectivity

//@siclog "Step 1" siclog@

f_UT_RequestActivateDedicatedBearer(UT,

hex2int(tsc_EpsDedicatedBearerId),

hex2int(tsc_EpsDefaultBearerId2ndPDN),

f_ConvertEPSQoS_ForATCommand(cs_508_EPS_QoS_Dedicated_1, cs_Extd_EPS_QOS),  // @sic R5-192186 sic@   //Anritsu TTCN CR R5s210238

f_EUTRA_Get_508_TrafficFlowTemplate(2, tsc_EpsDedicatedBearerId));

// The UE transmits a SERVICE REQUEST

// The SS establishes SRB2 and DRB associated with default EPS bearer context obtained during the attach procedure.

//@siclog "Step 2 - 3" siclog@

v_RadioResourceConfigDedicated := f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated(eutra_Cell1, MCG_SCG, RRC_IDLE);  //@sic R5s190323 sic@

f_EUTRA38_508_ENDC_RbEst_Steps3_10 (eutra_Cell1, omit, omit, omit, omit, omit, v_RadioResourceConfigDedicated); //@sic R5s190323 sic@

// Check : Does the UE transmits a Bearer Resource Allocation REQ ?

//@siclog "Step 4" siclog@

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_BearerResourceAllocReq(?,

tsc_EpsDefaultBearerId2ndPDN, //@sic R5s190323 sic@

cds_508_EPS_QoS(-, '00000001'B),// @sic R5-197006 sic@ @sic R5s200559 sic@

cr_Extd_EPS_QOS)))) -> value v_ReceivedAsp; // @sic R5-197006 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4");

// store PTI received

v_EpsPti := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.bEARER_RESOURCE_ALLOCATION_REQUEST.procedureTransactionIdentifier;

//Create the Activate Dedicated EPS Bearer Context REQ according to Table 10.2.2.1.3.3-2 in 38.523-1

v_ActivateBearer := f_EPS_GetActivateDedicatedBearer(tsc_EpsDedicatedBearerId,

cds_508_EPS_QoS(-, '00000001'B),// @sic R5-197006 sic@ @sic R5s200559 sic@

f_EUTRA_Get_508_TrafficFlowTemplate(6, tsc_EpsDedicatedBearerId),cs_Extd_EPS_QOS);

v_ActivateBearer.Pdu.Msg.aCTIVATE_DEDICATED_EPS_BEARER_CONTEXT_REQUEST.procedureTransactionIdentifier := v_EpsPti;

// Update SgNB Keys:

v_Security_Params := f_EUTRA38_Security_Get (); // @sic R5s200045 sic@

// @siclog "Steps 5 - 6" siclog@

f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, MCG_SCG, v_Security_Params.SK_Counter, RRC_CONNECTED, {v_ActivateBearer}); //@sic R5s190323 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6");

// @siclog "Steps 7" siclog@

// As per NOTE 2 CGCMOD is used

f_UT_RequestModifyDedicatedBearer(UT,

hex2int(tsc_EpsDedicatedBearerId),

hex2int(tsc_EpsDefaultBearerId2ndPDN),

f_ConvertEPSQoS_ForATCommand(cs_508_EPS_QoS_Dedicated_1, cs_Extd_EPS_QOS ('00010000'B)),  // @sic R5-192186 sic@       //Anritsu TTCN CR R5s210238

f_EUTRA_Get_508_TrafficFlowTemplate_2_Modify(tsc_EpsDedicatedBearerId, -, '2'H));

//@siclog "Step 8" siclog@

// Check : Does the UE transmit a BEARER RESOURCE MODIFICATION REQUEST message ?

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cdr_BearerResourceModReq(?,

tsc_EpsDedicatedBearerId,

omit,

cr_Tft(-),

cds_508_EPS_QoS('5B'O, '00000001'B),// @sic R5-197006 sic@ //@sic R5s190323 sic@ @sic R5s190920 sic@

cr_Extd_EPS_QOS(-,-,-))))) -> value v_ReceivedAsp; //@sic R5s190323 sic@ //@sic R5s190702 sic@ // @sic R5-197006 sic@ // @sic R5s191073 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 8");

v_EpsPti := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.bEARER_RESOURCE_MODIFICATION_REQUEST.procedureTransactionIdentifier;

//Create the Modify EPS Bearer Context REQ to be sent at Step 9 according to Table 10.2.2.1.3.3-4 in 38.523-1

v_ModifyEPSBearerCxtReq := cs_ModifyEPSBearerCxtReq_AllParams(tsc_EpsDedicatedBearerId,

v_EpsPti,

cds_508_EPS_QoS('5B'O, '00000001'B),// @sic R5-197006 sic@ // @sic R5s191073 sic@

f_EUTRA_Get_508_TrafficFlowTemplate(6, tsc_EpsDedicatedBearerId),

omit,

omit,

omit,

f_Get_508_PFI(),

cs_APN_AMBR,

omit,

omit,

omit,

omit,

cs_Extd_APN_AMBR,

cs_Extd_EPS_QOS ('00010000'B)); //@sic R5s190323 sic@ //@sic R5s190702 sic@

// @siclog "Steps 9" siclog@

SRB.send(cas_SRB2_NasPdu_REQ(eutra_Cell1,

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

v_ModifyEPSBearerCxtReq)));

//  Check:  Does the UE transmit a MODIFY EPS BEARER CONTEXT ACCEPT message as specified?

//@siclog "Step 10" siclog@

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_ModifyEPSBearerCxtAccept(tsc_EpsDedicatedBearerId))));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 10");

} // End of function f_TC_10_2_2_1_ENDC_EUTRA_TestBody

After change:

function fl_TC_10_2_2_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{

var ProcedureTransactionIdentifier v_EpsPti;

var EUTRA38_SecurityParamsExtd_Type v_Security_Params;

var template (value) NAS_DL_Message_Type v_ModifyEPSBearerCxtReq;

var template (value)  NAS_MSG_Request_Type v_ActivateBearer;

var SRB_COMMON_IND v_ReceivedAsp;

var template (omit) RadioResourceConfigDedicated v_RadioResourceConfigDedicated; //@sic R5s190323 sic@

var HalfOctet_Type v_LinkedId := tsc_EpsDefaultBearerId2ndPDN;

if (not tsc_EUTRA_Registration_MultiplePDN and tsc_EUTRA_Registration_PDN1isIMS) {

v_LinkedId := tsc_EpsDefaultBearerId;

}

// Cause the UE to request bearer resource allocation of dedicated EPS bearer associated with first PDN connectivity

//@siclog "Step 1" siclog@

f_UT_RequestActivateDedicatedBearer(UT,

hex2int(tsc_EpsDedicatedBearerId),

hex2int(v_LinkedId),

f_ConvertEPSQoS_ForATCommand(cs_508_EPS_QoS_Dedicated_1, cs_Extd_EPS_QOS),  // @sic R5-192186 sic@   //Anritsu TTCN CR R5s210238

f_EUTRA_Get_508_TrafficFlowTemplate(2, tsc_EpsDedicatedBearerId));

// The UE transmits a SERVICE REQUEST

// The SS establishes SRB2 and DRB associated with default EPS bearer context obtained during the attach procedure.

//@siclog "Step 2 - 3" siclog@

v_RadioResourceConfigDedicated := f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated(eutra_Cell1, MCG_SCG, RRC_IDLE);  //@sic R5s190323 sic@

f_EUTRA38_508_ENDC_RbEst_Steps3_10 (eutra_Cell1, omit, omit, omit, omit, omit, v_RadioResourceConfigDedicated); //@sic R5s190323 sic@

// Check : Does the UE transmits a Bearer Resource Allocation REQ ?

//@siclog "Step 4" siclog@

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_BearerResourceAllocReq(?,

v_LinkedId, //@sic R5s190323 sic@

cds_508_EPS_QoS(-, '00000001'B),// @sic R5-197006 sic@ @sic R5s200559 sic@

cr_Extd_EPS_QOS)))) -> value v_ReceivedAsp; // @sic R5-197006 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4");

// store PTI received

v_EpsPti := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.bEARER_RESOURCE_ALLOCATION_REQUEST.procedureTransactionIdentifier;

//Create the Activate Dedicated EPS Bearer Context REQ according to Table 10.2.2.1.3.3-2 in 38.523-1

v_ActivateBearer := f_EPS_GetActivateDedicatedBearer(tsc_EpsDedicatedBearerId,

cds_508_EPS_QoS(-, '00000001'B),// @sic R5-197006 sic@ @sic R5s200559 sic@

f_EUTRA_Get_508_TrafficFlowTemplate(6, tsc_EpsDedicatedBearerId),cs_Extd_EPS_QOS);

v_ActivateBearer.Pdu.Msg.aCTIVATE_DEDICATED_EPS_BEARER_CONTEXT_REQUEST.procedureTransactionIdentifier := v_EpsPti;

// Update SgNB Keys:

v_Security_Params := f_EUTRA38_Security_Get (); // @sic R5s200045 sic@

// @siclog "Steps 5 - 6" siclog@

f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, MCG_SCG, v_Security_Params.SK_Counter, RRC_CONNECTED, {v_ActivateBearer}); //@sic R5s190323 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6");

// @siclog "Steps 7" siclog@

// As per NOTE 2 CGCMOD is used

f_UT_RequestModifyDedicatedBearer(UT,

hex2int(tsc_EpsDedicatedBearerId),

hex2int(v_LinkedId),

f_ConvertEPSQoS_ForATCommand(cs_508_EPS_QoS_Dedicated_1, cs_Extd_EPS_QOS ('00010000'B)),  // @sic R5-192186 sic@       //Anritsu TTCN CR R5s210238

f_EUTRA_Get_508_TrafficFlowTemplate_2_Modify(tsc_EpsDedicatedBearerId, -, '2'H));

//@siclog "Step 8" siclog@

// Check : Does the UE transmit a BEARER RESOURCE MODIFICATION REQUEST message ?

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cdr_BearerResourceModReq(?,

tsc_EpsDedicatedBearerId,

omit,

cr_Tft(-),

cds_508_EPS_QoS('5B'O, '00000001'B),// @sic R5-197006 sic@ //@sic R5s190323 sic@ @sic R5s190920 sic@

cr_Extd_EPS_QOS(-,-,-))))) -> value v_ReceivedAsp; //@sic R5s190323 sic@ //@sic R5s190702 sic@ // @sic R5-197006 sic@ // @sic R5s191073 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 8");

v_EpsPti := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.bEARER_RESOURCE_MODIFICATION_REQUEST.procedureTransactionIdentifier;

//Create the Modify EPS Bearer Context REQ to be sent at Step 9 according to Table 10.2.2.1.3.3-4 in 38.523-1

v_ModifyEPSBearerCxtReq := cs_ModifyEPSBearerCxtReq_AllParams(tsc_EpsDedicatedBearerId,

v_EpsPti,

cds_508_EPS_QoS('5B'O, '00000001'B),// @sic R5-197006 sic@ // @sic R5s191073 sic@

f_EUTRA_Get_508_TrafficFlowTemplate(6, tsc_EpsDedicatedBearerId),

omit,

omit,

omit,

f_Get_508_PFI(),

cs_APN_AMBR,

omit,

omit,

omit,

omit,

cs_Extd_APN_AMBR,

cs_Extd_EPS_QOS ('00010000'B)); //@sic R5s190323 sic@ //@sic R5s190702 sic@

// @siclog "Steps 9" siclog@

SRB.send(cas_SRB2_NasPdu_REQ(eutra_Cell1,

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

v_ModifyEPSBearerCxtReq)));

//  Check:  Does the UE transmit a MODIFY EPS BEARER CONTEXT ACCEPT message as specified?

//@siclog "Step 10" siclog@

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_ModifyEPSBearerCxtAccept(tsc_EpsDedicatedBearerId))));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 10");

} // End of function f_TC_10_2_2_1_ENDC_EUTRA_TestBody