3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210521

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1716 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_RRC_Idle_Extension |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per 38.508-1 Table 4.5.2.2-4: NR RRC_IDLE Extension, Steps 2-9a1 should be skipped if there is already an existing PDU connection established. As per ETSI ES 201 873-1 TTCN Specification: 19.10 The Return statement The return statement terminates execution of a function or altstep and returns control to the point from which the function or altstep was called. When used in functions, a return statement may be optionally associated with a return value or template. TTCN-3 allows optional statement blocks that may follow altstep calls within alt statements. If there is a statement block, the return statement returns control to the beginning of this statement block and the statement block is executed before the alt statement is left. If there is no statement block, test execution continues with the first statement following the alt statement. Simply using the return statement is not enough to skip Steps 2-9a1 in f_NR5GC_RRC_Idle_Extension, as it would only exit the alt statement instead of exiting the function. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an additional condition v_NoOfExistingSessions == 0 before executing Steps 2-9a1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.2.1.NR5GC and others |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_NR5GC_RRC_Idle_Extension |
| --- | --- |
| Reason for change | As per 38.508-1 Table 4.5.2.2-4: NR RRC_IDLE Extension, Steps 2-9a1 should be skipped if there is already an existing PDU connection established. As per ETSI ES 201 873-1 TTCN Specification: 19.10 The Return statement The return statement terminates execution of a function or altstep and returns control to the point from which the function or altstep was called. When used in functions, a return statement may be optionally associated with a return value or template. TTCN-3 allows optional statement blocks that may follow altstep calls within alt statements. If there is a statement block, the return statement returns control to the beginning of this statement block and the statement block is executed before the alt statement is left. If there is no statement block, test execution continues with the first statement following the alt statement. Simply using the return statement is not enough to skip Steps 2-9a1 in f_NR5GC_RRC_Idle_Extension, as it would only exit the alt statement instead of exiting the function. |
| Summary of change | Add an additional condition v_NoOfExistingSessions == 0 before executing Steps 2-9a1 |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before change:

function f_NR5GC_RRC_Idle_Extension(NR_CellId_Type p_NR_CellId,

NR_TESTMODE_STATE_Type p_TestMode,

RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease,

template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling,

UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O,

template (omit) SDAP_Config p_SDAP_Config := omit,

template S_NSSAI_Type p_ExpectedS_NSSAI := *,

template DNN p_ExpectedDNN := *,

template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B),

template (omit) DNN p_DNN := omit,

template (omit) QoS_Rules p_QoS_Rules := omit,

template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O),

template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@

template (omit) QoSFlowDescr p_QoSFlowDescr := omit,

template (present) B4_Type p_ServiceType := '0000'B)  runs on NR5GC_PTC

{

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId);

var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams);

var integer v_NoOfExistingSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); // @sic R5-206291 sic@

var PDU_PDN_DNN_Type v_PDN_Type;

var charstring v_DNN := "";

timer t_Wait := 10.0;

// Step 0 @sic R5-204399 sic@

t_Wait.start;

alt {

[] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) {

t_Wait.stop;

}

[] t_Wait.timeout {

// Step 1

if (v_NoOfExistingSessions == 0) { // @sic R5-206291 sic@

/* Only trigger if there's no sessions established automatically

to ensure there's (at least) one session established for the test body */

v_PDN_Type := f_GetDefaultAPN_DNNType_FromPICS(); // @sic R5-211372 sic@

select (v_PDN_Type) {

case (Internet_DNN, None) {

v_DNN := pc_APN_ID_Internet;

}

case (IMS_DNN) {

v_DNN := pc_APN_ID_IMS;

}

}

f_UT_RequestPDUSession(UT, 1, v_DNN); // @sic R5s210084, R5-211372 sic@

} else { // @sic R5s210084 sic@

// if an establishment request message hasn't been sent, or triggered, then don't need to carry on

return;

}

}

}

// Steps 2 - 4

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?); // @sic R5s200489 sic@

if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu,  cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (),

tsc_NasKsi_NativeSecurityContext),

v_S_TMSI, p_ServiceType)) {   // @sic R5s210189 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed");

}

// Steps 5-6

// RRC Security Mode procedure

f_NR_RRC_ActivateSecurity (p_NR_CellId, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); // Use NasCount returned from Service Req

// FFS - Prose DOES NOT specify PDU Session Status to be reflected back from Req message

// Step 7

SRB.send (cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus)))); // @sic R5s201387 Baseline Moving sic@

// Step 8

f_NR5GC_PDUSessionEstablishment (p_NR_CellId,

pc_noOf_PDUsNewConnection,

tsc_NR_RbId_SRB1, // @sic R5-206291 sic@

p_SDAP_Config, // @sic R5s210134 sic@

p_ExpectedS_NSSAI,

p_ExpectedDNN,

p_SSC_Mode,

p_DNN,

p_QoS_Rules,

p_Session_AMBR,

p_S_NSSAI,

p_QoSFlowDescr);

//Step 9a1

if (p_ReleaseConnection == rrcConnectionRelease) {

// Release RRC to enter RRC_IDLE

f_NR_RRCRelease(p_NR_CellId);

} else {

// Step 8Aa1 - 8Aa2

select (p_TestMode) {

case (TEST_LOOPModeA_ON) {

f_NR5GC_CloseUE_TestLoopModeA (p_NR_CellId, p_UE_TestLoopModeA_NR_LB_Setup);

}

case (TEST_LOOPModeB_ON) {

f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime);

}

case (PING_Or_TEST_LOOPModeB_ON) {

if (not pc_IP_Ping) {

f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime);

}

}

}

}

}

After change:

function f_NR5GC_RRC_Idle_Extension(NR_CellId_Type p_NR_CellId,

NR_TESTMODE_STATE_Type p_TestMode,

RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease,

template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling,

UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O,

template (omit) SDAP_Config p_SDAP_Config := omit,

template S_NSSAI_Type p_ExpectedS_NSSAI := *,

template DNN p_ExpectedDNN := *,

template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B),

template (omit) DNN p_DNN := omit,

template (omit) QoS_Rules p_QoS_Rules := omit,

template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O),

template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@

template (omit) QoSFlowDescr p_QoSFlowDescr := omit,

template (present) B4_Type p_ServiceType := '0000'B)  runs on NR5GC_PTC

{

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId);

var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams);

var integer v_NoOfExistingSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); // @sic R5-206291 sic@

var PDU_PDN_DNN_Type v_PDN_Type;

var charstring v_DNN := "";

timer t_Wait := 10.0;

// Step 0 @sic R5-204399 sic@

t_Wait.start;

alt {

[] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) {

t_Wait.stop;

}

[] t_Wait.timeout {

// Step 1

if (v_NoOfExistingSessions == 0) { // @sic R5-206291 sic@

/* Only trigger if there's no sessions established automatically

to ensure there's (at least) one session established for the test body */

v_PDN_Type := f_GetDefaultAPN_DNNType_FromPICS(); // @sic R5-211372 sic@

select (v_PDN_Type) {

case (Internet_DNN, None) {

v_DNN := pc_APN_ID_Internet;

}

case (IMS_DNN) {

v_DNN := pc_APN_ID_IMS;

}

}

f_UT_RequestPDUSession(UT, 1, v_DNN); // @sic R5s210084, R5-211372 sic@

} else { // @sic R5s210084 sic@

// if an establishment request message hasn't been sent, or triggered, then don't need to carry on

return;

}

}

}

if (v_NoOfExistingSessions == 0) {

// Steps 2 - 4

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?); // @sic R5s200489 sic@

if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu,  cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (),

tsc_NasKsi_NativeSecurityContext),

v_S_TMSI, p_ServiceType)) {   // @sic R5s210189 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed");

}

// Steps 5-6

// RRC Security Mode procedure

f_NR_RRC_ActivateSecurity (p_NR_CellId, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); // Use NasCount returned from Service Req

// FFS - Prose DOES NOT specify PDU Session Status to be reflected back from Req message

// Step 7

SRB.send (cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus)))); // @sic R5s201387 Baseline Moving sic@

// Step 8

f_NR5GC_PDUSessionEstablishment (p_NR_CellId,

pc_noOf_PDUsNewConnection,

tsc_NR_RbId_SRB1, // @sic R5-206291 sic@

p_SDAP_Config, // @sic R5s210134 sic@

p_ExpectedS_NSSAI,

p_ExpectedDNN,

p_SSC_Mode,

p_DNN,

p_QoS_Rules,

p_Session_AMBR,

p_S_NSSAI,

p_QoSFlowDescr);

//Step 9a1

if (p_ReleaseConnection == rrcConnectionRelease) {

// Release RRC to enter RRC_IDLE

f_NR_RRCRelease(p_NR_CellId);

} else {

// Step 8Aa1 - 8Aa2

select (p_TestMode) {

case (TEST_LOOPModeA_ON) {

f_NR5GC_CloseUE_TestLoopModeA (p_NR_CellId, p_UE_TestLoopModeA_NR_LB_Setup);

}

case (TEST_LOOPModeB_ON) {

f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime);

}

case (PING_Or_TEST_LOOPModeB_ON) {

if (not pc_IP_Ping) {

f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime);

}

}

}

}

}

}