3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210571

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1639 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Multiple ENDC Measurement testcases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation, function f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); is commented out for the test cases 8.2.3.4.1,8.2.3.5.1,8.2.3.6.1/1a/1b, 8.2.3.7.1/1a/1b and 8.2.3.8.1/1a/1b. This change is introduced due to the proposed implementation for R5s210089. Because of this change , test cases are failing as the EUTRA PTC is waiting for this coordination signal from NR PTC |  |  |  |  |  |  |  |  |
| Summary of change: |  | f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}) is enabled in the test cases 8.2.3.4.1,8.2.3.5.1,8.2.3.6.1/1a/1b, 8.2.3.7.1/1a/1b and 8.2.3.8.1/1a/1b |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.4.1.ENDC,8.2.3.5.1.ENDC,8.2.3.6.1.ENDC, 8.2.3.6.1a.ENDC ,8.2.3.6.1b.ENDC, 8.2.3.7.1.ENDC, 8.2.3.7.1a.ENDC, 8.2.3.7.1b.ENDC , 8.2.3.8.1.ENDC, 8.2.3.8.1a.ENDC and 8.2.3.8.1b.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	Correction to function f_TC_8_2_3_4_1_ENDC_NR ()	3

1.2	Correction to function f_TC_8_2_3_5_1_ENDC_NR ()	4

1.3	Correction to function f_TC_8_2_3_6_1_Common ()	5

1.4	Correction to the function f_TC_8_2_3_7_1_Common	6

1.5	Correction to the function f_TC_8_2_3_8_1_Common	8

# Corrections required

## Correction to function f_TC_8_2_3_4_1_ENDC_NR ()

| Function name | Function f_TC_8_2_3_4_1_ENDC_NR |
| --- | --- |
| Reason for change | In the current TTCN implementation, function f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); is commented out for the test case 8.2.3.4.1. This change is introduced due to the proposed implementation for R5s210089 Because of this change , test case is failing as the EUTRA PTC is waiting for this coordination signal from NR PTC |
| Summary of change | f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}) is enabled in the test cases 8.2.3.4.1 |
| TTCN module | common\dev\ENDC_working\RRCMeasurement_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_2_3_4_1_ENDC_NR() runs on ENDC_NR_PTC { //Measurement configuration control and reporting / Event A1 / Measurement of NR PSCell / EN-DC f_ENDC_NR_Init(); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -79, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5s190062, R5-195929, R5-195369 sic@ //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_SCG); //f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); @sic R5-195369, R5s@210089 sic@ f_NR_TestBody_Set(true); fl_TC_8_2_3_Common_TestBody(triggerEvent_A1); fl_TC_8_2_3_4_1_Additional_TestBody(); f_NR_TestBody_Set(false); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); } // End of f_TC_8_2_3_4_1_ENDC_NR()

After Change:

function f_TC_8_2_3_4_1_ENDC_NR() runs on ENDC_NR_PTC { //Measurement configuration control and reporting / Event A1 / Measurement of NR PSCell / EN-DC f_ENDC_NR_Init(); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -79, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5s190062, R5-195929, R5-195369 sic@ //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_SCG); f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); //WA_21wk12 @sic R5-195369, R5s@210089 sic@ f_NR_TestBody_Set(true); fl_TC_8_2_3_Common_TestBody(triggerEvent_A1); fl_TC_8_2_3_4_1_Additional_TestBody(); f_NR_TestBody_Set(false); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); } // End of f_TC_8_2_3_4_1_ENDC_NR()

## Correction to function f_TC_8_2_3_5_1_ENDC_NR ()

| Function name | Function f_TC_8_2_3_5_1_ENDC_NR |
| --- | --- |
| Reason for change | In the current TTCN implementation, function f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); is commented out for the test case 8.2.3.5.1. This change is introduced due to the proposed implementation for R5s210089 Because of this change , test case is failing as the EUTRA PTC is waiting for this coordination signal from NR PTC |
| Summary of change | f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}) is enabled in the test cases 8.2.3.5.1 |
| TTCN module | common\dev\ENDC_working\RRCMeasurement_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_2_3_5_1_ENDC_NR() runs on ENDC_NR_PTC { //Measurement configuration control and reporting / Event A2 / Measurement of NR PSCell / EN-DC f_ENDC_NR_Init(); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -79, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5s190060, R5-195929, R5-195369 sic@ //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_SCG); //f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); @sic R5-195369, R5s210089 sic@ f_NR_TestBody_Set(true); fl_TC_8_2_3_Common_TestBody(triggerEvent_A2); f_NR_TestBody_Set(false); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

After Change:

function f_TC_8_2_3_5_1_ENDC_NR() runs on ENDC_NR_PTC { //Measurement configuration control and reporting / Event A2 / Measurement of NR PSCell / EN-DC f_ENDC_NR_Init(); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -79, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5s190060, R5-195929, R5-195369 sic@ //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_SCG); f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); //WA_21wk12 @sic R5-195369, R5s210089 sic@ f_NR_TestBody_Set(true); fl_TC_8_2_3_Common_TestBody(triggerEvent_A2); f_NR_TestBody_Set(false); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

## Correction to function f_TC_8_2_3_6_1_Common ()

| Function name | function f_TC_8_2_3_6_1_Common |
| --- | --- |
| Reason for change | In the current TTCN implementation, function f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); is commented out for the test case 8.2.3.6.1. This change is introduced due to the proposed implementation for R5s210089 Because of this change , test case is failing as the EUTRA PTC is waiting for this coordination signal from NR PTC |
| Summary of change | f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}) is enabled in the test cases 8.2.3.6.1/1a/1b |
| TTCN module | common\dev\ENDC_working\RRCMeasurement_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_2_3_6_1_Common(NR_CellId_Type p_NR_CellId) runs on ENDC_NR_PTC { //Measurement configuration control and reporting / Event A3 /Measurement of Neighbour NR cells / Intra-frequency measurements / EN-DC f_ENDC_NR_Init(); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -85, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5s190155, R5-195929, R5-195369 sic@ f_NR_CellInfo_InitMaxReferencePower(p_NR_CellId, -79, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5-195929 sic@ //Create and configure NR cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(p_NR_CellId); f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_SCG); //f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); @sic R5-195369, R5s210089 sic@ f_NR_TestBody_Set(true); fl_TC_8_2_3_6_1x_TestBody(p_NR_CellId); //@sic R5-195365, R5-195366 sic@ f_NR_TestBody_Set(false); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

After Change:

function f_TC_8_2_3_6_1_Common(NR_CellId_Type p_NR_CellId) runs on ENDC_NR_PTC { //Measurement configuration control and reporting / Event A3 /Measurement of Neighbour NR cells / Intra-frequency measurements / EN-DC f_ENDC_NR_Init(); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -85, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5s190155, R5-195929, R5-195369 sic@ f_NR_CellInfo_InitMaxReferencePower(p_NR_CellId, -79, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5-195929 sic@ //Create and configure NR cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(p_NR_CellId); f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_SCG); f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); //WA_21wk12 @sic R5-195369, R5s210089 sic@ f_NR_TestBody_Set(true); fl_TC_8_2_3_6_1x_TestBody(p_NR_CellId); //@sic R5-195365, R5-195366 sic@ f_NR_TestBody_Set(false); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

## Correction to the function f_TC_8_2_3_7_1_Common

| Function name | function f_TC_8_2_3_7_1_Common |
| --- | --- |
| Reason for change | In the current TTCN implementation, function f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); is commented out for the test case 8.2.3.7.1/1a/1b. This change is introduced due to the proposed implementation for R5s210089 Because of this change , test case is failing as the EUTRA PTC is waiting for this coordination signal from NR PTC |
| Summary of change | f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}) is enabled in the test cases 8.2.3.7.1/1a/1b |
| TTCN module | common\dev\ENDC_working\RRCMeasurement_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_2_3_7_1_Common(NR_CellId_Type p_NR_CellId) runs on ENDC_NR_PTC { //@sic R5-196118, R5-196119 sic@ f_ENDC_NR_Init(); f_NR_CellInfo_InitMaxReferencePower(p_NR_CellId, -79, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5-195929, R5-195369 sic@ //Create and configure NR cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(p_NR_CellId); //The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (EN-DC) and DC Bearers (MCG and SCG) on E-UTRA Cell 1 according to TS 38.508-1 [4], clause 4.5.4. f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_SCG); //f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); @sic R5-195369, R5s210089 sic@ f_NR_TestBody_Set(true); fl_TC_8_2_3_7_1x_TestBody(p_NR_CellId); //@sic R5-195365, R5-195366 sic@ f_NR_TestBody_Set(false); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

After Change:

function f_TC_8_2_3_7_1_Common(NR_CellId_Type p_NR_CellId) runs on ENDC_NR_PTC { //@sic R5-196118, R5-196119 sic@ f_ENDC_NR_Init(); f_NR_CellInfo_InitMaxReferencePower(p_NR_CellId, -79, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5-195929, R5-195369 sic@ //Create and configure NR cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(p_NR_CellId); //The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (EN-DC) and DC Bearers (MCG and SCG) on E-UTRA Cell 1 according to TS 38.508-1 [4], clause 4.5.4. f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_SCG); f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); //WA_21wk12 @sic R5-195369, R5s210089 sic@ f_NR_TestBody_Set(true); fl_TC_8_2_3_7_1x_TestBody(p_NR_CellId); //@sic R5-195365, R5-195366 sic@ f_NR_TestBody_Set(false); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

## Correction to the function f_TC_8_2_3_8_1_Common

| Function name | function f_TC_8_2_3_8_1_Common |
| --- | --- |
| Reason for change | In the current TTCN implementation, function f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); is commented out for the test case 8.2.3.8.1/1a/1b. This change is introduced due to the proposed implementation for R5s210089 Because of this change , test case is failing as the EUTRA PTC is waiting for this coordination signal from NR PTC |
| Summary of change | f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}) is enabled in the test cases 8.2.3.8.1/1a/1b |
| TTCN module | common\dev\ENDC_working\RRCMeasurement_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_2_3_8_1_Common(NR_CellId_Type p_NR_CellId) runs on ENDC_NR_PTC { //Measurement configuration control and reporting / Event A5 / Measurement of Neighbour NR cell / Intra-frequency measurements / EN-DC f_ENDC_NR_Init(); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -78, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5-194851, R5s190416, R5-195929, R5-204455 sic@ f_NR_CellInfo_InitMaxReferencePower(p_NR_CellId, -84, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5s190416, R5-195712, R5-195929, R5s200426, R5-204455 sic@ //Create and configure NR cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(p_NR_CellId); //The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (EN-DC) and DC Bearers (MCG and SCG) on E-UTRA Cell 1 according to TS 38.508-1 [4], clause 4.5.4. f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_SCG); //f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); @sic R5210089 sic@ f_NR_TestBody_Set(true); fl_TC_8_2_3_8_1x_TestBody(p_NR_CellId); //@sic R5-195367, R5-195368 sic@ f_NR_TestBody_Set(false); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

After Change:

function f_TC_8_2_3_8_1_Common(NR_CellId_Type p_NR_CellId) runs on ENDC_NR_PTC { //Measurement configuration control and reporting / Event A5 / Measurement of Neighbour NR cell / Intra-frequency measurements / EN-DC f_ENDC_NR_Init(); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -78, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5-194851, R5s190416, R5-195929, R5-204455 sic@ f_NR_CellInfo_InitMaxReferencePower(p_NR_CellId, -84, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5s190416, R5-195712, R5-195929, R5s200426, R5-204455 sic@ //Create and configure NR cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(p_NR_CellId); //The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (EN-DC) and DC Bearers (MCG and SCG) on E-UTRA Cell 1 according to TS 38.508-1 [4], clause 4.5.4. f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_SCG); f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); //WA_21wk12 @sic R5210089 sic@ f_NR_TestBody_Set(true); fl_TC_8_2_3_8_1x_TestBody(p_NR_CellId); //@sic R5-195367, R5-195368 sic@ f_NR_TestBody_Set(false); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }