3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210594

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1738 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC MAC DRX test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In all NR5GC MAC DRX test cases, before the start of the test procedure f_NR_ULGrantConfiguration_StartPeriodicGrant is called with cs_TimingInfo_SubFrame(0), the expectation is that UL grant is sent out on SF 0, slot 0 (first slot). However, for SCS 30KHz, based on the UL-DL pattern configuration and K2=4, the UL data will be expected in slot 4 which is not an UL slot. The grant scheduling should go on slot 4, to expect data on slot 8. See the scheduling table below: DL; DL; DL; DL; DL; DL; DL; DL/F/UL; UL; UL. 0; 1; 2; 3; 4; 5; 6; 7; 8; 9. <---------------------n+k2 = 4+4 ----------- ->. DCI_0_1; PUSCH. Slot 0; Slot 1; Slot 0; Slot 1; Slot 0; Slot 1; Slot 0; Slot 1; Slot 0; Slot 1. SF 0 (1msec); SF 1; SF 2; SF 3; SF 4. <<-------------------------------dl_UL_TransmissionPeriodicity = ms5 -------------------------------------------------------------------->>. |
| Summary of change: |  | Changed the subframe to 2 when SCS 30 is configured. Editorial correction to the comment as the UL grant is configured to be sent out every 10msec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC implementation will FAIL conformant UEs when testing with SCS 30KHz. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.5.1.NR5GC, 7.1.1.5.2.NR5GC, 7.1.1.5.3.NR5GC, 7.1.1.5.4.NR5GC, 7.1.1.5.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required for NR5GC DRX test cases	3

1.1	Correction to f_TC_7_1_1_5_1_NR5GC	3

1.2	Other DRX test cases	5

# Corrections required for NR5GC DRX test cases

## Correction to f_TC_7_1_1_5_1_NR5GC

| Function name | f_TC_7_1_1_5_1_NR5GC |
| --- | --- |
| Reason for change | In all NR5GC MAC DRX test cases, before the start of the test procedure f_NR_ULGrantConfiguration_StartPeriodicGrant is called with cs_TimingInfo_SubFrame(0), the expectation is that UL grant is sent out on SF 0, slot 0 (first slot). However, for SCS 30KHz, based on the UL-DL pattern configuration and K2=4, the UL data will be expected in slot 4 which is not an UL slot. The grant scheduling should go on slot 4, to expect data on slot 8. See the scheduling table below: DL; DL; DL; DL; DL; DL; DL; DL/F/UL; UL; UL. 0; 1; 2; 3; 4; 5; 6; 7; 8; 9. <---------------------n+k2 = 4+4 ----------- ->. DCI_0_1; PUSCH. Slot 0; Slot 1; Slot 0; Slot 1; Slot 0; Slot 1; Slot 0; Slot 1; Slot 0; Slot 1. SF 0 (1msec); SF 1; SF 2; SF 3; SF 4. <<-------------------------------dl_UL_TransmissionPeriodicity = ms5 -------------------------------------------------------------------->>. |
| Summary of change | Changed the subframe to 2 when SCS 30 is configured. Editorial correction to the comment as the UL grant is configured to be sent out every 10msec. |
| TTCN module | DRX_MAC_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_5_1_NR5GC() runs on NR5GC_PTC { // DRX operation / Short cycle not configured / Parameters configured by RRC const integer tsc_LongDRXCycle := 640; const integer tsc_DrxStartOffset := 7; //@sic R5-202609 R5s200711 sic@ var DRB_Identity v_DRBId; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var template (value) CellGroupConfig v_CellGroupConfig; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var SubFrameTiming_Type v_Timing1; //Init Cell parameters f_NR5GC_Init(NR_1); … … … //@sic step 1-2 sic@ f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_SubFrame(0), valueof (cs_NR_ResourceAllocationDef), 10); // periodic grant every 5ms (80% of 12ms the Inactivity Timer) v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1);

After Change:

function f_TC_7_1_1_5_1_NR5GC() runs on NR5GC_PTC { // DRX operation / Short cycle not configured / Parameters configured by RRC const integer tsc_LongDRXCycle := 640; const integer tsc_DrxStartOffset := 7; //@sic R5-202609 R5s200711 sic@ var DRB_Identity v_DRBId; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var template (value) CellGroupConfig v_CellGroupConfig; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var SubFrameTiming_Type v_Timing1; var SubcarrierSpacing v_SCS; var SubFrameNumber_Type v_SF := 0; //Init Cell parameters f_NR5GC_Init(NR_1); … … … //@sic step 1-2 sic@ v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); if (v_SCS ==kHz30) {v_SF:= 2;} f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_SubFrame(v_SF), valueof (cs_NR_ResourceAllocationDef), 10); // periodic grant every 10ms (80% of 12ms the Inactivity Timer) v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1);

## Other DRX test cases

Same changes are required for

- f_TC_7_1_1_5_2_NR5GC

- f_TC_7_1_1_5_3_NR5GC

- f_TC_7_1_1_5_4_NR5GC

- f_TC_7_1_1_5_5_NR5GC