3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210767

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4627 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EUTRA RRC MDT test case 8.6.4.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2021-07-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per test spec 36.523-1 section 8.6.4.5.3.2-2 timer duration is 5 seconds to ensure that the UE detects T310 which is more than required. As per 36.331 section 6.3.6, T310 itself can be defined for a maximum of 2s, so there is no need to turn off cell for 5Sec for UE to detect the T310 expiry. Hence timer duration has been reduced to 3 seconds |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reduced wait timer to 3 seconds to ensure that cell is turned on before UE performs cell scan Note : An associated prose CR on 36.523-1 will be raised at RAN5#92 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.523-1 CR xxxx |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_8_6_4_5_EUTRA() |
| --- | --- |
| Reason for change | As per test spec 36.523-1 section 8.6.4.5.3.2-2 timer duration is 5 seconds to ensure that the UE detects T310 which is more than required. As per 36.331 section 6.3.6, T310 itself can be defined for a maximum of 2s, so there is no need to turn off cell for 5Sec for UE to detect the T310 expiry. Hence timer duration has been reduced to 3 seconds |
| Summary of change | Reduced wait timer to 3 seconds to ensure that cell is turned on before UE performs cell scan Note : An associated prose CR on 36.523-1 will be raised at RAN5#92 |
| TTCN module | RRC_LoggedRLF.ttcn |
| MCC160 Comment |  |

Before change

function f_TC_8_6_4_5_EUTRA() runs on EUTRA_PTC

{ //Radio Link Failure logging / Reporting of ECGI of the PCell

var template (value) CellPowerList_Type v_CellPowerList_AtT0;

var template (value) CellPowerList_Type v_CellPowerList_AtT1;

var template (value) CellPowerList_Type v_CellPowerList_AtT2;

var template (value) CellPowerList_Type v_CellPowerList_AtT3;

………………..

v_Dl_CarrierFreq_Cell2 := v_Frequency_IE_Cell2.UL_DL_Earfcn.dl_CarrierFreq;

v_TimerValue := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 5.0);

//Set EPRE for RF Initial

f_EUTRA_CellInfo_InitMaxReferencePower(eutra_Cell2, -79); //@sic R5s160638 sic@

// Create and configure Cells

f_EUTRA_CellConfig_Def(eutra_Cell1);

f_EUTRA_CellConfig_Def(eutra_Cell2);

f_EUTRA_SetCellPowerList(v_CellPowerList_AtT0);

// Preamble

f_EUTRA_Preamble(eutra_Cell1, STATE2_IDLEUPDATE);

f_EUTRA_RbEst_Def(eutra_Cell1);

f_EUTRA_TestBody_Set (true);

…………….

f_EUTRA_SS_SRBs_DRBs_Reset (eutra_Cell2); //@sic R5s150662, R5s150402, R5s140067, R5s140069 sic@

//@siclog "Step 7" siclog@

//The SS changes Cell 1 and Cell 2 parameter according to the row "T2" in Table 8.6.4.5.3.2-1.

f_EUTRA_SetCellPowerList(v_CellPowerList_AtT2);

//@siclog "Step 8" siclog@

//Wait for 5s to ensure that the UE detects T310 expiry.

f_Delay(v_TimerValue);

//@siclog "Step 9" siclog@

//The SS changes Cell 2 parameter according to the row "T3" in Table 8.6.4.5.3.2-1.

f_EUTRA_SetCellPowerList(v_CellPowerList_AtT3);

………………

// Postamble

f_EUTRA_Postamble(eutra_Cell2, E2_CONNECTED);

} // end of f_TC_8_6_4_5_EUTRA

After change

function f_TC_8_6_4_5_EUTRA() runs on EUTRA_PTC

{ //Radio Link Failure logging / Reporting of ECGI of the PCell

var template (value) CellPowerList_Type v_CellPowerList_AtT0;

var template (value) CellPowerList_Type v_CellPowerList_AtT1;

var template (value) CellPowerList_Type v_CellPowerList_AtT2;

var template (value) CellPowerList_Type v_CellPowerList_AtT3;

……………..

v_Dl_CarrierFreq_Cell2 := v_Frequency_IE_Cell2.UL_DL_Earfcn.dl_CarrierFreq;

v_TimerValue := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 3.0);

//Set EPRE for RF Initial

f_EUTRA_CellInfo_InitMaxReferencePower(eutra_Cell2, -79); //@sic R5s160638 sic@

// Create and configure Cells

f_EUTRA_CellConfig_Def(eutra_Cell1);

f_EUTRA_CellConfig_Def(eutra_Cell2);

f_EUTRA_SetCellPowerList(v_CellPowerList_AtT0);

// Preamble

f_EUTRA_Preamble(eutra_Cell1, STATE2_IDLEUPDATE);

f_EUTRA_RbEst_Def(eutra_Cell1);

f_EUTRA_TestBody_Set (true);

………………

f_EUTRA_SS_SRBs_DRBs_Reset (eutra_Cell2); //@sic R5s150662, R5s150402, R5s140067, R5s140069 sic@

//@siclog "Step 7" siclog@

//The SS changes Cell 1 and Cell 2 parameter according to the row "T2" in Table 8.6.4.5.3.2-1.

f_EUTRA_SetCellPowerList(v_CellPowerList_AtT2);

//@siclog "Step 8" siclog@

//Wait for 3s to ensure that the UE detects T310 expiry.

f_Delay(v_TimerValue);

//@siclog "Step 9" siclog@

//The SS changes Cell 2 parameter according to the row "T3" in Table 8.6.4.5.3.2-1.

f_EUTRA_SetCellPowerList(v_CellPowerList_AtT3);

…………………….

// Postamble

f_EUTRA_Postamble(eutra_Cell2, E2_CONNECTED);