3GPP TSG-RAN5 Meeting #2020-TTCN email	R5s210882

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4641 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test case 22.3.3.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | By default, in Test case 22.3.3.5 TTCN is only supporting ROHC_profile 0x0004 i.e IP data. In the scenario when UE supports only ROHC profile 0x0002, the test case fail as UDP data was not generated by SS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Populating SDU list based on ROHC profile supported and configuring UDP packet in SDU list when UE supports ROHC profile 0x0002 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail this test case when using ROHC profile 0x0002 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

# Change 1

| Function name | f_TC_22_3_3_5_NBIOT |
| --- | --- |
| Reason for change | By default, in Test case 22.3.3.5 TTCN is only supporting ROHC_profile 0x0004 i.e IP data. In the scenario when UE supports only ROHC profile 0x0002, the test case fail as UDP data was not generated by SS |
| Summary of change | Populating SDU list based on ROHC profile supported and configuring UDP packet in SDU list when UE supports ROHC profile 0x0002 |
| TTCN module | NBIOT\source\NBIOT\22_3\NBIOT_PDCP_Testcases.ttcn |
| MCC160 Comment |  |

Before change:

…

module NBIOT_PDCP_Testcases

{

import from NBIOT_RRC_ASN1_Definitions language "ASN.1:2002" all with {encode "UNALIGNED_PER_OctetAligned"};

import from CommonDefs all;

import from CommonAspDefs all;

import from Common4G5G_Templates all;

import from LoopbackIP all;

import from EUTRA_NB_CommonDefs all;

import from EUTRA_NB_CommonL2 all;

import from EUTRA_NB_ASP_L2DataDefs all;

import from NBIOT_Imported_EUTRA_ASN1_Types all;

import from NBIOT_CommonDefs all;

import from NBIOT_ASP_TypeDefs all;

import from NBIOT_AspCommon_Templates all;

import from NBIOT_Component all;

import from NBIOT_CellInfo all;

import from NBIOT_CellCfg_Templates all;

import from NBIOT_ConfigurationSteps all;

import from NBIOT_CommonProcedures all;

import from NBIOT_Paging all;

import from NBIOT_RRC_Templates all;

import from NBIOT_SRB_DRB_Templates all;

import from NBIOT_Security_Templates all;

import from NBIOT_RRCSteps all;

import from NBIOT_NASSteps all;

import from NBIOT_Timing all;

import from NBIOT_LoopBack all;

import from NAS_CommonTypeDefs all;

import from NBIOT_SecurityFunctions all;

import from NBIOT_SecuritySteps all;

import from NAS_CommonTemplates all;

import from CIOT_NASTemplates all;

import from EPS_NAS_Templates all;

import from EPS_NAS_MsgContainers all;

……

function f_TC_22_3_3_5_NBIOT() runs on NBIOT_PTC

{

var NBIOT_CellId_Type v_CellId := nbiot_Cell1;

var RRCConnectionResume_NB_r13_IEs.drb_ContinueROHC_r13 v_ContinueROHC := true_;

var integer v_SequenceNumberTX;

var integer v_SequenceNumberRX;

var charstring v_SourceAddr := f_LoopbackModeB_IP_Address_UE();

var UE_Capability_NB_r13 v_NBIOT_Cap;

var template (value) DL_DCCH_Message_NB v_RRCConnectionResume;

var template (omit) PDCP_Config_NB_r13 v_PDCP_Config_NB;

var template (value) DRB_ToAddMod_NB_r13 v_DRB_ToAddMod;

var template (value) DRB_ToAddModList_NB_r13 v_DRB_ToAddModList;

var template (value) NB_RadioBearer_Type v_NB_DRB_ConfigRoHC;

var template (value) PDCP_SDU_Type v_PDCP_SDU;

var template (value) PDCP_SDUList_Type v_PDCP_SDUList := {

f_IPv4IPv6_IcmpEchoReply(v_SourceAddr, -, 1),

f_IPv4IPv6_IcmpEchoReply(v_SourceAddr, -, 2),

f_IPv4IPv6_IcmpEchoReply(v_SourceAddr, -, 3)

};

f_NBIOT_Init(c1, USER_PLANE);      /* @sic R5s170772 Change 1.1: USER_PLANE sic@ */

// Create and configure all cells

f_NBIOT_CellConfig_Def(v_CellId, USER_PLANE);

…

After change:

…

module NBIOT_PDCP_Testcases

{

import from NBIOT_RRC_ASN1_Definitions language "ASN.1:2002" all with {encode "UNALIGNED_PER_OctetAligned"};

import from CommonDefs all;

import from CommonAspDefs all;

import from Common4G5G_Templates all;

import from LoopbackIP all;

import from EUTRA_NB_CommonDefs all;

import from EUTRA_NB_CommonL2 all;

import from EUTRA_NB_ASP_L2DataDefs all;

import from NBIOT_Imported_EUTRA_ASN1_Types all;

import from NBIOT_CommonDefs all;

import from NBIOT_ASP_TypeDefs all;

import from NBIOT_AspCommon_Templates all;

import from NBIOT_Component all;

import from NBIOT_CellInfo all;

import from NBIOT_CellCfg_Templates all;

import from NBIOT_ConfigurationSteps all;

import from NBIOT_CommonProcedures all;

import from NBIOT_Paging all;

import from NBIOT_RRC_Templates all;

import from NBIOT_SRB_DRB_Templates all;

import from NBIOT_Security_Templates all;

import from NBIOT_RRCSteps all;

import from NBIOT_NASSteps all;

import from NBIOT_Timing all;

import from NBIOT_LoopBack all;

import from NAS_CommonTypeDefs all;

import from NBIOT_SecurityFunctions all;

import from NBIOT_SecuritySteps all;

import from NAS_CommonTemplates all;

import from CIOT_NASTemplates all;

import from EPS_NAS_Templates all;

import from EPS_NAS_MsgContainers all;

import from Parameters all;

……

function f_TC_22_3_3_5_NBIOT() runs on NBIOT_PTC

{

var NBIOT_CellId_Type v_CellId := nbiot_Cell1;

var RRCConnectionResume_NB_r13_IEs.drb_ContinueROHC_r13 v_ContinueROHC := true_;

var integer v_SequenceNumberTX;

var integer v_SequenceNumberRX;

var charstring v_SourceAddr := f_LoopbackModeB_IP_Address_UE();

var UE_Capability_NB_r13 v_NBIOT_Cap;

var template (value) DL_DCCH_Message_NB v_RRCConnectionResume;

var template (omit) PDCP_Config_NB_r13 v_PDCP_Config_NB;

var template (value) DRB_ToAddMod_NB_r13 v_DRB_ToAddMod;

var template (value) DRB_ToAddModList_NB_r13 v_DRB_ToAddModList;

var template (value) NB_RadioBearer_Type v_NB_DRB_ConfigRoHC;

var template (value) PDCP_SDU_Type v_PDCP_SDU;

var template (value) PDCP_SDUList_Type v_PDCP_SDUList;

var charstring v_IPv6_SourceAddr := "fe80::1:1";

var charstring v_IPv6_DestAddr   := "2001:0ba0::0001:0001";

if(pc_ROHC_profile0x0002 == true)

{

v_PDCP_SDUList := {

f_IPv4IPv6_AnyUdpPacket(v_IPv6_SourceAddr, v_IPv6_DestAddr, 31160, 61000),

f_IPv4IPv6_AnyUdpPacket(v_IPv6_SourceAddr, v_IPv6_DestAddr, 31160, 61000),

f_IPv4IPv6_AnyUdpPacket(v_IPv6_SourceAddr, v_IPv6_DestAddr, 31160, 61000)

};

}

else if(pc_ROHC_profile0x0004==true)

{

v_PDCP_SDUList := {

f_IPv4IPv6_IcmpEchoReply(v_SourceAddr, -, 1),

f_IPv4IPv6_IcmpEchoReply(v_SourceAddr, -, 2),

f_IPv4IPv6_IcmpEchoReply(v_SourceAddr, -, 3)

};

}

f_NBIOT_Init(c1, USER_PLANE);      /* @sic R5s170772 Change 1.1: USER_PLANE sic@ */

// Create and configure all cells

f_NBIOT_CellConfig_Def(v_CellId, USER_PLANE);

…