3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210902

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1721 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR handover function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to 38.523-3 c7.3.5.3.1, step 11-12 of the sequence of intra-NR inter-cell handover should be applied to target cell. 11. Target Cell: Start periodic TA. 12. Target Cell: Inform the SS about completion of the HO (e.g. to trigger PDCP STATUS REPORT PDU). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use p_TargetCellId for f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_12() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | e.g. 8.1.4.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR5GC_508RRC_IntraNR_HO_InterCell_CommonSteps1_12 |
| --- | --- |
| Reason for change | According to 38.523-3 c7.3.5.3.1, step 11-12 of the sequence of intra-NR inter-cell handover should be applied to target cell. 11. Target Cell: Start periodic TA. 12. Target Cell: Inform the SS about completion of the HO (e.g. to trigger PDCP STATUS REPORT PDU). |
| Summary of change | Use p_TargetCellId for f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_12() |
| TTCN module | NR5GC_Handover |
| MCC160 Comment |  |

Before Change

function f_NR5GC_508RRC_IntraNR_HO_InterCell_CommonSteps1_12(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, boolean p_RBConfig_KeyChange := true, template (value) DL_DCCH_Message p_RRCReconfiguration, template (value) NR_RachProcedureConfig_Type p_RachProcedureConfig, template (value) NR_DRBInfoList_Type p_DrbInfoList, template (value) UL_GrantConfig_Type p_UL_GrantConfig_SourceCell := cs_UL_GrantConfig_OnSR, template (value) UL_GrantConfig_Type p_UL_GrantConfig_TargetCell := cs_UL_GrantConfig_OnSR, template (present) UL_DCCH_Message p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete, NextHopChainingCount p_NCC := tsc_NR_38508_NextHopChainingCount, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit ) runs on NR_BASE_PTC { //@sic R5s200819 sic@ f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps1_3(p_SourceCellId, p_TargetCellId, p_RBConfig_KeyChange, p_DrbInfoList ); f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps4_9( p_SourceCellId, p_TargetCellId, p_RNTI_Value, p_RBConfig_KeyChange, p_RRCReconfiguration, p_RachProcedureConfig, p_UL_GrantConfig_SourceCell, p_UL_GrantConfig_TargetCell, p_NCC, p_RACH_ConfigDedicated); // Step 10: Target Cell: Receive RRCReconfigurationComplete SRB.receive(car_NR_SRB1_RrcPdu_IND(p_TargetCellId, p_RRCConnReconfigComplete)); f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_12(p_SourceCellId); //@sic R5s210135 sic@ }

After Change

function f_NR5GC_508RRC_IntraNR_HO_InterCell_CommonSteps1_12(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, boolean p_RBConfig_KeyChange := true, template (value) DL_DCCH_Message p_RRCReconfiguration, template (value) NR_RachProcedureConfig_Type p_RachProcedureConfig, template (value) NR_DRBInfoList_Type p_DrbInfoList, template (value) UL_GrantConfig_Type p_UL_GrantConfig_SourceCell := cs_UL_GrantConfig_OnSR, template (value) UL_GrantConfig_Type p_UL_GrantConfig_TargetCell := cs_UL_GrantConfig_OnSR, template (present) UL_DCCH_Message p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete, NextHopChainingCount p_NCC := tsc_NR_38508_NextHopChainingCount, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit ) runs on NR_BASE_PTC { //@sic R5s200819 sic@ f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps1_3(p_SourceCellId, p_TargetCellId, p_RBConfig_KeyChange, p_DrbInfoList ); f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps4_9( p_SourceCellId, p_TargetCellId, p_RNTI_Value, p_RBConfig_KeyChange, p_RRCReconfiguration, p_RachProcedureConfig, p_UL_GrantConfig_SourceCell, p_UL_GrantConfig_TargetCell, p_NCC, p_RACH_ConfigDedicated); // Step 10: Target Cell: Receive RRCReconfigurationComplete SRB.receive(car_NR_SRB1_RrcPdu_IND(p_TargetCellId, p_RRCConnReconfigComplete)); f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_12(p_TargetCellId); //@sic R5s210135 sic@ }