3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211464

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2009 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 7.1.1.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | LCID value ‘000101’ is fixed for RLC SDUs always on this case. For internet PDN TTCN is assigning DRB id 2 and LCID 5. For ims PDN, TTCN is assigning DRB id 1 and LCID 4. But, LCID 5(tsc_LCID_SCG_Bearer) is only used in construction of MAC PDU in case of DRBid=1 or DRBid=2. Hence, it will fail the TC if this TC will be executed with IMS as default dnn. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead constant LCID, added conditional LCID value as per DRB id. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will fail conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_2_1_NR_TestBody1() and f_TC_7_1_1_2_1_NR_TestBody2() |
| --- | --- |
| Reason for change | LCID value ‘000101’ is fixed for RLC SDUs always on this case. For internet PDN TTCN is assigning DRB id 2 and LCID 5. For ims PDN, TTCN is assigning DRB id 1 and LCID 4. But, LCID 5(tsc_LCID_SCG_Bearer) is only used in construction of MAC PDU in case of DRBid=1 or DRBid=2. Hence, it will fail the TC if this TC will be executed with IMS as default dnn. |
| Summary of change | Instead constant LCID, added conditional LCID value as per DRB id. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_2_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { var RNTI_Value_Type v_C_RNTI; var integer j; var integer v_Nack_Count; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_MAC_SDU_Type v_EncodedRlcPdu2; var NR_MAC_SDU_Type v_EncodedRlcPdu3; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL; //Anritsu TTCN WA for 7.1.1.2.1.ENDC_FR2 timer t_Watchdog := 0.6; //configure SS for no Auto Time alignement //@sic R5s200009 sic@ f_NR_ULGrantConfiguration_Common(nr_Cell1, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Stop); // Configure for indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // @siclog "Steps 1 - 2" siclog@ // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); t_Watchdog.start; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); // @siclog "Step 3" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); t_Watchdog.stop; // store cell info parameters for C-RNTI v_C_RNTI := f_NR_CellInfo_GetRNTI(nr_Cell1); // @siclog "Steps 4-5" siclog@ // Configure SS with a different RNTI than allocated to UE f_NR_SS_C_RNTI_Config(nr_Cell1, tsc_RNTI_Value_Diff); // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B))); t_Watchdog.start; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); // @siclog "Step 6" siclog@ alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, (ack, nack))) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6: UE is sending HARQ ACK/NACK when sent addressed to different CRNTI"); } [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } } // Configure SS with a same RNTI allocated to UE f_NR_SS_C_RNTI_Config(nr_Cell1, v_C_RNTI); for (j :=0; j < 16; j:= j+1) { v_Nack_Count := 0; // Configure SS to corrupt CRC in first transmission of DL MAC PDU //@sic R5s190360 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 400); //Anritsu TTCN WA for 7.1.1.2.1.ENDC_FR2 v_TimingDL.Subframe.Number := 2; //Anritsu TTCN WA for 7.1.1.2.1.NR5GC if (v_TimingDL.SFN.Number mod 2 == 1){ //Anritsu TTCN WA for 7.1.1.2.1.NR5GC v_TimingDL.SFN.Number := (v_TimingDL.SFN.Number + 1) mod 1024; } f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Error1AndNormal, cs_TimingInfo_NR(v_TimingDL))); //Anritsu TTCN WA for 7.1.1.2.1.ENDC_FR2 //@siclog "Step 7" siclog@ v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(j+1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(j+1, crs_NR_PDCP_SDU_36B))); // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) t_Watchdog.start; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment(j))); //@sic R5s190360 sic@ //Anritsu TTCN WA for 7.1.1.2.1.ENDC_FR2 //@siclog "Step 8" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, nack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); //@siclog "Step 9" siclog@ // MAC PDU retransmision automatically performed by SS //@siclog "Step 10" siclog@ alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); t_Watchdog.stop; } [v_Nack_Count<3] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, nack)) { v_Nack_Count := v_Nack_Count +1; repeat; } [] t_Watchdog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10: UE did not send HARQ ACK"); } } } // Configure for stop indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); //@siclog "Step 11" siclog@ // 254B PDCP SDU + 3B PDCP Header + 3B RLC Header =260B MAC SDU v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(17, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(17, crs_NR_PDCP_SDU_254B))); v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(18, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(18, crs_NR_PDCP_SDU_254B1))); v_EncodedRlcPdu3 := f_NR_RLC_AMD_FullPDU_Encvalue(19, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(19, crs_NR_PDCP_SDU_254B2))); // 260 MAC SDU + 3B MAC Header =263B MAC Sub PDU // 3 MAC SUB PDU = 3*263 = 789B = 6312 bits // 6312 + 88 bit Padding =6400 (an allowed TB size as per 38.523-3 Annex B) //88bit padding = 1B padding header + 10B padding v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI16(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1), cs_NR_MAC_SDU_SubPDU_LI16(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu2), cs_NR_MAC_SDU_SubPDU_LI16(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu3) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL, cs_NR_MAC_Padding_SubPDU(tsc_NR_MAC_Padding_10B)) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); //@siclog "Step 12" siclog@ f_NR_RxRLCSTATUS_PDU_ACK_SN_Only(20, p_NR_DRB_Id); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); //@siclog "Step 13" siclog@ // 122B PDCP SDU + 3B PDCP Header + 3B RLC Header =128B MAC SDU v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(20, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(20, crs_NR_PDCP_SDU_122B))); v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(21, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(21, crs_NR_PDCP_SDU_122B1))); v_EncodedRlcPdu3 := f_NR_RLC_AMD_FullPDU_Encvalue(22, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(22, crs_NR_PDCP_SDU_122B2))); // 128 MAC SDU + 2B MAC Header =130B MAC Sub PDU // 3 MAC SUB PDU = 3*130 = 390B = 3120 bits // 3120 + 120 bit Padding =3240 (an allowed TB size as per 38.523-3 Annex B) //88bit padding = 1B padding header + 10B padding v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu2), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu3) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL, cs_NR_MAC_Padding_SubPDU(tsc_NR_MAC_Padding_14B)) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); //@siclog "Step 14" siclog@ f_NR_RxRLCSTATUS_PDU_ACK_SN_Only(23, p_NR_DRB_Id); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); //@siclog "Step 15" siclog@ // 121B PDCP SDU + 3B PDCP Header + 3B RLC Header =127B MAC SDU v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(23, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(23, crs_NR_PDCP_SDU_121B))); // 127 MAC SDU + 2B MAC Header =129B MAC Sub PDU =1032 bits (an allowed TB size as per 38.523-3 Annex B) v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); //@siclog "Step 16" siclog@ f_NR_RxRLCSTATUS_PDU_ACK_SN_Only(24, p_NR_DRB_Id); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); //@siclog "Step 17" siclog@ // 117B PDCP SDU + 3B PDCP Header + 3B RLC Header =123B MAC SDU // 123B MAC SDU + 2B MAC Header =125B MAC Sub PDU v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(24, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(24, crs_NR_PDCP_SDU_117B))); // 254B PDCP SDU + 3B PDCP Header + 3B RLC Header =260B MAC SDU // 260B MAC SDU + 3B MAC Header =263B MAC Sub PDU v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(25, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(25, crs_NR_PDCP_SDU_254B1))); // 263B + 125B =388B =3104 bits (an allowed TB size as per 38.523-3 Annex B) v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1), cs_NR_MAC_SDU_SubPDU_LI16(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu2) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); //@siclog "Step 18" siclog@ f_NR_RxRLCSTATUS_PDU_ACK_SN_Only(26, p_NR_DRB_Id); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18"); } function f_TC_7_1_1_2_1_NR_TestBody2(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { var NR_MAC_SDU_Type v_EncodedRlcPdu1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; timer t_Watchdog := 0.6; // Configure for indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // @siclog "Steps 19a3 - 19a4" siclog@ // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(26, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); t_Watchdog.start; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); // @siclog "Step 19a5" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)); t_Watchdog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 23"); // Configure for stop indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); }

After Change

function f_TC_7_1_1_2_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { var RNTI_Value_Type v_C_RNTI; var integer j; var integer v_Nack_Count; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_MAC_SDU_Type v_EncodedRlcPdu2; var NR_MAC_SDU_Type v_EncodedRlcPdu3; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL; //Anritsu TTCN WA for 7.1.1.2.1.ENDC_FR2 timer t_Watchdog := 0.6; //configure SS for no Auto Time alignement //@sic R5s200009 sic@ f_NR_ULGrantConfiguration_Common(nr_Cell1, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Stop); // Configure for indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // @siclog "Steps 1 - 2" siclog@ // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); var B6_Type p_LCID; select (p_NR_DRB_Id) { case (1) { p_LCID := '000100'B; } case (2) { p_LCID := '000101'B; } case else { FatalError (__FILE__, __LINE__, "wrong DRB ID"); } } t_Watchdog.start; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(p_LCID, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); // @siclog "Step 3" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); t_Watchdog.stop; // store cell info parameters for C-RNTI v_C_RNTI := f_NR_CellInfo_GetRNTI(nr_Cell1); // @siclog "Steps 4-5" siclog@ // Configure SS with a different RNTI than allocated to UE f_NR_SS_C_RNTI_Config(nr_Cell1, tsc_RNTI_Value_Diff); // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B))); t_Watchdog.start; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(p_LCID, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); // @siclog "Step 6" siclog@ alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, (ack, nack))) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6: UE is sending HARQ ACK/NACK when sent addressed to different CRNTI"); } [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } } // Configure SS with a same RNTI allocated to UE f_NR_SS_C_RNTI_Config(nr_Cell1, v_C_RNTI); for (j :=0; j < 16; j:= j+1) { v_Nack_Count := 0; // Configure SS to corrupt CRC in first transmission of DL MAC PDU //@sic R5s190360 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 400); //Anritsu TTCN WA for 7.1.1.2.1.ENDC_FR2 v_TimingDL.Subframe.Number := 2; //Anritsu TTCN WA for 7.1.1.2.1.NR5GC if (v_TimingDL.SFN.Number mod 2 == 1){ //Anritsu TTCN WA for 7.1.1.2.1.NR5GC v_TimingDL.SFN.Number := (v_TimingDL.SFN.Number + 1) mod 1024; } f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Error1AndNormal, cs_TimingInfo_NR(v_TimingDL))); //Anritsu TTCN WA for 7.1.1.2.1.ENDC_FR2 //@siclog "Step 7" siclog@ v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(j+1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(j+1, crs_NR_PDCP_SDU_36B))); // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) t_Watchdog.start; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(p_LCID, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment(j))); //@sic R5s190360 sic@ //Anritsu TTCN WA for 7.1.1.2.1.ENDC_FR2 //@siclog "Step 8" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, nack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); //@siclog "Step 9" siclog@ // MAC PDU retransmision automatically performed by SS //@siclog "Step 10" siclog@ alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); t_Watchdog.stop; } [v_Nack_Count<3] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, nack)) { v_Nack_Count := v_Nack_Count +1; repeat; } [] t_Watchdog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10: UE did not send HARQ ACK"); } } } // Configure for stop indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); //@siclog "Step 11" siclog@ // 254B PDCP SDU + 3B PDCP Header + 3B RLC Header =260B MAC SDU v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(17, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(17, crs_NR_PDCP_SDU_254B))); v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(18, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(18, crs_NR_PDCP_SDU_254B1))); v_EncodedRlcPdu3 := f_NR_RLC_AMD_FullPDU_Encvalue(19, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(19, crs_NR_PDCP_SDU_254B2))); // 260 MAC SDU + 3B MAC Header =263B MAC Sub PDU // 3 MAC SUB PDU = 3*263 = 789B = 6312 bits // 6312 + 88 bit Padding =6400 (an allowed TB size as per 38.523-3 Annex B) //88bit padding = 1B padding header + 10B padding v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI16(p_LCID, v_EncodedRlcPdu1), cs_NR_MAC_SDU_SubPDU_LI16(p_LCID, v_EncodedRlcPdu2), cs_NR_MAC_SDU_SubPDU_LI16(p_LCID, v_EncodedRlcPdu3) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL, cs_NR_MAC_Padding_SubPDU(tsc_NR_MAC_Padding_10B)) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); //@siclog "Step 12" siclog@ f_NR_RxRLCSTATUS_PDU_ACK_SN_Only(20, p_NR_DRB_Id); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); //@siclog "Step 13" siclog@ // 122B PDCP SDU + 3B PDCP Header + 3B RLC Header =128B MAC SDU v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(20, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(20, crs_NR_PDCP_SDU_122B))); v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(21, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(21, crs_NR_PDCP_SDU_122B1))); v_EncodedRlcPdu3 := f_NR_RLC_AMD_FullPDU_Encvalue(22, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(22, crs_NR_PDCP_SDU_122B2))); // 128 MAC SDU + 2B MAC Header =130B MAC Sub PDU // 3 MAC SUB PDU = 3*130 = 390B = 3120 bits // 3120 + 120 bit Padding =3240 (an allowed TB size as per 38.523-3 Annex B) //88bit padding = 1B padding header + 10B padding v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(p_LCID, v_EncodedRlcPdu1), cs_NR_MAC_SDU_SubPDU_LI8(p_LCID, v_EncodedRlcPdu2), cs_NR_MAC_SDU_SubPDU_LI8(p_LCID, v_EncodedRlcPdu3) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL, cs_NR_MAC_Padding_SubPDU(tsc_NR_MAC_Padding_14B)) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); //@siclog "Step 14" siclog@ f_NR_RxRLCSTATUS_PDU_ACK_SN_Only(23, p_NR_DRB_Id); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); //@siclog "Step 15" siclog@ // 121B PDCP SDU + 3B PDCP Header + 3B RLC Header =127B MAC SDU v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(23, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(23, crs_NR_PDCP_SDU_121B))); // 127 MAC SDU + 2B MAC Header =129B MAC Sub PDU =1032 bits (an allowed TB size as per 38.523-3 Annex B) v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(p_LCID, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); //@siclog "Step 16" siclog@ f_NR_RxRLCSTATUS_PDU_ACK_SN_Only(24, p_NR_DRB_Id); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); //@siclog "Step 17" siclog@ // 117B PDCP SDU + 3B PDCP Header + 3B RLC Header =123B MAC SDU // 123B MAC SDU + 2B MAC Header =125B MAC Sub PDU v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(24, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(24, crs_NR_PDCP_SDU_117B))); // 254B PDCP SDU + 3B PDCP Header + 3B RLC Header =260B MAC SDU // 260B MAC SDU + 3B MAC Header =263B MAC Sub PDU v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(25, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(25, crs_NR_PDCP_SDU_254B1))); // 263B + 125B =388B =3104 bits (an allowed TB size as per 38.523-3 Annex B) v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(p_LCID, v_EncodedRlcPdu1), cs_NR_MAC_SDU_SubPDU_LI16(p_LCID, v_EncodedRlcPdu2) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); //@siclog "Step 18" siclog@ f_NR_RxRLCSTATUS_PDU_ACK_SN_Only(26, p_NR_DRB_Id); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18"); } function f_TC_7_1_1_2_1_NR_TestBody2(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { var NR_MAC_SDU_Type v_EncodedRlcPdu1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; timer t_Watchdog := 0.6; // Configure for indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // @siclog "Steps 19a3 - 19a4" siclog@ // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(26, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); var B6_Type p_LCID; select (p_NR_DRB_Id) { case (1) { p_LCID := '000100'B; } case (2) { p_LCID := '000101'B; } case else { FatalError (__FILE__, __LINE__, "wrong DRB ID"); } } t_Watchdog.start; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(p_LCID, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); // @siclog "Step 19a5" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)); t_Watchdog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 23"); // Configure for stop indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); }