3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211657

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4653 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_EUTRA_OptionalMultiPDN_IMSRegistration_PTCSetup |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-11-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Function f_EUTRA_OptionalMultiPDN_IMSRegistration_PTCSetup is used in several LTE-IRAT test cases to perform an optional IMS registration in case it is supportend by UE after a Tracking Area Update procedure. f_EUTRA_OptionalMultiPDN_IMSRegistration_PTCSetup will call f_IP_ChangeDrbMappingEUTRA when there are no values set at routing table for LTE, in fact, PDNs are started later, as can be observed at f_EUTRA_OptionalMultiPDN_IMSRegistration_PTCSetup implementation. As current values of vc_IP_PTC_PDN_MappingTable are set to omit there is no actual DRB mapping needed to change, calling f_IP_ChangeDrbMappingEUTRA is unnnecessary and can provoke an error when UE supports IMS. This issue can be solved removing f_IP_ChangeDrbMappingEUTRA call. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed f_IP_ChangeDrbMappingEUTRA function call from f_EUTRA_OptionalMultiPDN_IMSRegistration_PTCSetup implementation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case(s) may end with a runttime error. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.4, 6.2.3.4a, 6.2.3.13, 6.2.3.31, 6.2.3.21, 6.2.4.x where x is (1,3,4,5,6,7), 9.2.2.1.10, 9.2.3.3.1, 9.2.3.3.5a and 9.2.3.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	1

1	Corrections required	1

1.1	Correction to function f_EUTRA_OptionalMultiPDN_IMSRegistration_PTCSetup	1

# Corrections required

## Correction to function f_EUTRA_OptionalMultiPDN_IMSRegistration_PTCSetup

| Function name | f_EUTRA_OptionalMultiPDN_IMSRegistration_PTCSetup |
| --- | --- |
| Reason for change | Current values of vc_IP_PTC_PDN_MappingTable are set to omit there is no actual DRB mapping needed to change, calling f_IP_ChangeDrbMappingEUTRA is unnnecessary and can provoke an error when UE supports IMS. This issue can be solved removing f_IP_ChangeDrbMappingEUTRA call.. |
| Summary of change | Removed f_IP_ChangeDrbMappingEUTRA function call from f_EUTRA_OptionalMultiPDN_IMSRegistration_PTCSetup implementation. |
| TTCN module | Common\EUTRA\EUTRA_NASCommonFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_EUTRA_OptionalMultiPDN_IMSRegistration_PTCSetup(EUTRA_CellId_Type p_CellId) runs on EUTRA_PTC { var EPS_BearerIdentity v_EpsBearerId := f_EUTRA_InitialRegistrationEpsBearerId(firstPDN); var EPS_BearerIdentity v_EpsBearerId2; var PDN_Index_Type v_FirstPdn := f_EUTRA_InitialRegistrationPdnIndex(firstPDN); if (pc_IMS) { f_EUTRA_Update_RANType (p_CellId); // @sic R5s170173 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_IpcanInd); // @sic R5s170176 sic@ f_IP_ChangeDrbMappingEUTRA(IP, p_CellId, tsc_DRB1, v_FirstPdn); // @sic R5s190434 sic@ f_IP_Handling_StartPDN(IP, v_FirstPdn, cs_DrbInfo_EUTRA(p_CellId, f_EUTRA_NB_EpsBearerAssociatedDRB(v_EpsBearerId))); // @sic R5s190434 sic@ if (tsc_EUTRA_Registration_MultiplePDN) { // @sic R5s180190 sic@ v_EpsBearerId2 := f_EUTRA_InitialRegistrationEpsBearerId(secondPDN); f_IP_Handling_StartPDN(IP, f_EUTRA_InitialRegistrationPdnIndex(secondPDN), cs_DrbInfo_EUTRA(p_CellId, f_EUTRA_NB_EpsBearerAssociatedDRB(v_EpsBearerId2))); // @sic R5s170964, R5s180190, R5s190434 sic@ } } } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_EUTRA_OptionalMultiPDN_IMSRegistration_PTCSetup(EUTRA_CellId_Type p_CellId) runs on EUTRA_PTC { var EPS_BearerIdentity v_EpsBearerId := f_EUTRA_InitialRegistrationEpsBearerId(firstPDN); var EPS_BearerIdentity v_EpsBearerId2; var PDN_Index_Type v_FirstPdn := f_EUTRA_InitialRegistrationPdnIndex(firstPDN); if (pc_IMS) { f_EUTRA_Update_RANType (p_CellId); // @sic R5s170173 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_IpcanInd); // @sic R5s170176 sic@ <<REMOVED CODE>> f_IP_ChangeDrbMappingEUTRA(IP, p_CellId, tsc_DRB1, v_FirstPdn); // @sic R5s190434 sic@ << REMOVED CODE>> f_IP_Handling_StartPDN(IP, v_FirstPdn, cs_DrbInfo_EUTRA(p_CellId, f_EUTRA_NB_EpsBearerAssociatedDRB(v_EpsBearerId))); // @sic R5s190434 sic@ if (tsc_EUTRA_Registration_MultiplePDN) { // @sic R5s180190 sic@ v_EpsBearerId2 := f_EUTRA_InitialRegistrationEpsBearerId(secondPDN); f_IP_Handling_StartPDN(IP, f_EUTRA_InitialRegistrationPdnIndex(secondPDN), cs_DrbInfo_EUTRA(p_CellId, f_EUTRA_NB_EpsBearerAssociatedDRB(v_EpsBearerId2))); // @sic R5s170964, R5s180190, R5s190434 sic@ } } } <<SKIPPED CODE>>