3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211713

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2226 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR MAC test case 7.1.1.3.2b |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Current implementation is trying to reassign the parameter p_TimeDomainResourceAssignment instead of the variable v_TimeDomainResourceAssignment. In this case the variable v_TimeDomainResourceAssignment is left unitialised causing a run-time error later. - according step 2/7 in Table 7.1.1.3.2b.3.2-1, the UL grant is scheduled 150ms after the DL data trasnmsision. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Replace p_TimeDomainResourceAssignment with v_TimeDomainResourceAssignment - Replace 300ms with 150ms |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not execute correctly |  |  |  |  |  |  |  |  |
|  |  | 8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.2b.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_3_2b_NR_TestBody_Loop |
| --- | --- |
| Reason for change | - Current implementation is trying to reassign the parameter p_TimeDomainResourceAssignment instead of the variable v_TimeDomainResourceAssignment. In this case the variable v_TimeDomainResourceAssignment is left unitialised causing a run-time error later. - according step 2/7 in Table 7.1.1.3.2b.3.2-1, the UL grant is scheduled 150ms after the DL data trasnmsision. |
| Summary of change | - Replace p_TimeDomainResourceAssignment with v_TimeDomainResourceAssignment - replace 300ms with 150ms |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_3_2b_NR_TestBody_Loop(DRB_Identity p_NR_DRB_Id1, DRB_Identity p_NR_DRB_Id2, DRB_Identity p_NR_DRB_Id3, integer p_N1, //Number of SDUs for each run integer p_N2, integer p_N3, integer p_T1,// Time period and total duration integer p_T2, integer p_D1, //Amount of data to compare with (octets) integer p_D2, integer p_D3, template (omit) NR_DciCommon_TimeDomainResourceAssignment_Type p_TimeDomainResourceAssignment := omit) runs on NR_BASE_PTC { var template (value) NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var SubFrameTiming_Type v_Timing; var template (value) NR_PDCP_SDU_Type v_NR_PDCP_SDU := crs_NR_PDCP_SDU_317B ; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DRB_DataPerSlotList1; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DRB_DataPerSlotList2; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DRB_DataPerSlotList3; var template (omit) NR_DciCommon_TimeDomainResourceAssignment_Type v_TimeDomainResourceAssignment; timer t_WatchDog := 1.0; // Local wait Timer if (isvalue (p_TimeDomainResourceAssignment)) //Loop 2 { // v_NR_PDCP_SDU := crs_NR_PDCP_SDU_157B ; v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(9224, v_NR_UplinkBWP); v_TimeDomainResourceAssignment := p_TimeDomainResourceAssignment; }else { v_NR_ResourceAllocation := cs_NR_ResourceAllocation(23,25) ; //PUSCH FR1: MCS Index table = 5.1.3.1-1, Mapping type = TypeA, dmrs-AdditionalPosition = 0, number of CDM groups = 1, modulation order <= 6, PUSCH-duration = 4 //PUSCH FR2: MCS Index table = 5.1.3.1-1, Mapping type = TypeA, dmrs-AdditionalPosition = 1, number of CDM groups = 1, modulation order <= 6, PUSCH-duration = 4 //--TBS-----L_RBs-----I_MCS- // 4608 23 25 p_TimeDomainResourceAssignment := cs_NR_DciCommon_TimeDomainResourceAssignment('0'B); } f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); // runs 1 to 4 f_NR_HarqErrorHandling_Set(raiseInconc); // Start 'Harq Errors not tolerant' //@siclog "Step 1" siclog@ // SS sends: // N1 320-octet RLC SDUs on DRB1 // N2 320-octet RLC SDUs on DRB2 // N3 320-octet RLC SDUs on DRB3 v_DRB_DataPerSlotList1:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(v_NR_PDCP_SDU, p_N1); v_DRB_DataPerSlotList2:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(v_NR_PDCP_SDU, p_N2); v_DRB_DataPerSlotList3:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(v_NR_PDCP_SDU, p_N3); v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); //300 ms in future //Exceptionally the time is extended to 300 as in loop 4, 179 PDCP SDU's each of 317 bytes get scheduled; f_TC_7_1_1_3_2_Step1(v_DRB_DataPerSlotList1, v_DRB_DataPerSlotList2, v_DRB_DataPerSlotList3, p_NR_DRB_Id1, p_NR_DRB_Id2, p_NR_DRB_Id3, v_Timing); //@siclog "Step 2" siclog@ // UL Grant of D[j] every T1ms for a duration of T2ms if (p_T2 != 0) { v_Timing := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_Timing, 300); f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell1, cs_TimingInfo_NR(v_Timing), v_NR_ResourceAllocation, p_T1, (p_T2 / p_T1), v_TimeDomainResourceAssignment); }; //@siclog "Step 3" siclog@ ………. }

After Change

function f_TC_7_1_1_3_2b_NR_TestBody_Loop(DRB_Identity p_NR_DRB_Id1, DRB_Identity p_NR_DRB_Id2, DRB_Identity p_NR_DRB_Id3, integer p_N1, //Number of SDUs for each run integer p_N2, integer p_N3, integer p_T1,// Time period and total duration integer p_T2, integer p_D1, //Amount of data to compare with (octets) integer p_D2, integer p_D3, template (omit) NR_DciCommon_TimeDomainResourceAssignment_Type p_TimeDomainResourceAssignment := omit) runs on NR_BASE_PTC { var template (value) NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var SubFrameTiming_Type v_Timing; var template (value) NR_PDCP_SDU_Type v_NR_PDCP_SDU := crs_NR_PDCP_SDU_317B ; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DRB_DataPerSlotList1; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DRB_DataPerSlotList2; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DRB_DataPerSlotList3; var template (omit) NR_DciCommon_TimeDomainResourceAssignment_Type v_TimeDomainResourceAssignment; timer t_WatchDog := 1.0; // Local wait Timer if (isvalue (p_TimeDomainResourceAssignment)) //Loop 2 { // v_NR_PDCP_SDU := crs_NR_PDCP_SDU_157B ; v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(9224, v_NR_UplinkBWP); v_TimeDomainResourceAssignment := p_TimeDomainResourceAssignment; }else { v_NR_ResourceAllocation := cs_NR_ResourceAllocation(23,25) ; //PUSCH FR1: MCS Index table = 5.1.3.1-1, Mapping type = TypeA, dmrs-AdditionalPosition = 0, number of CDM groups = 1, modulation order <= 6, PUSCH-duration = 4 //PUSCH FR2: MCS Index table = 5.1.3.1-1, Mapping type = TypeA, dmrs-AdditionalPosition = 1, number of CDM groups = 1, modulation order <= 6, PUSCH-duration = 4 //--TBS-----L_RBs-----I_MCS- // 4608 23 25 v_TimeDomainResourceAssignment := cs_NR_DciCommon_TimeDomainResourceAssignment('0'B); } f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); // runs 1 to 4 f_NR_HarqErrorHandling_Set(raiseInconc); // Start 'Harq Errors not tolerant' //@siclog "Step 1" siclog@ // SS sends: // N1 320-octet RLC SDUs on DRB1 // N2 320-octet RLC SDUs on DRB2 // N3 320-octet RLC SDUs on DRB3 v_DRB_DataPerSlotList1:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(v_NR_PDCP_SDU, p_N1); v_DRB_DataPerSlotList2:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(v_NR_PDCP_SDU, p_N2); v_DRB_DataPerSlotList3:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(v_NR_PDCP_SDU, p_N3); v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); //300 ms in future //Exceptionally the time is extended to 300 as in loop 4, 179 PDCP SDU's each of 317 bytes get scheduled; f_TC_7_1_1_3_2_Step1(v_DRB_DataPerSlotList1, v_DRB_DataPerSlotList2, v_DRB_DataPerSlotList3, p_NR_DRB_Id1, p_NR_DRB_Id2, p_NR_DRB_Id3, v_Timing); //@siclog "Step 2" siclog@ // UL Grant of D[j] every T1ms for a duration of T2ms if (p_T2 != 0) { v_Timing := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_Timing, 150); f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell1, cs_TimingInfo_NR(v_Timing), v_NR_ResourceAllocation, p_T1, (p_T2 / p_T1), v_TimeDomainResourceAssignment); }; //@siclog "Step 3" siclog@ ……. }