3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220028

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2259 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 7.1.3.5.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In function fl_TC_7_1_3_5_3_ENDC_NR_TestBody, Reestablish DRB2 is performed with activation time . And in step 7, as part of fl_TC_7_1_3_5_3_ENDC_NR_TestBody , UL grant information is sent with immediate activation time . As the grant is sent with immediate activation , there is a possbility that UE can send the PDCP status report in the middle of DRB2 reestablish steps . Hence it is proposed to send the Ul grant with the timing information in align to the DRB2 configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Ul grant information is sent with same activation time as DRB2 configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.3.ENDC, 7.1.3.5.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function fl_TC_7_1_3_5_3_ENDC_NR_TestBody	2

1.2	Correction to function f_TC_7_1_3_5_3_NR_TestBody_Step7to9	5

1.3	Correction to function fl_TC_7_1_3_5_3_NR_TestBody	8

# Corrections required

## Correction to function fl_TC_7_1_3_5_3_ENDC_NR_TestBody

| Function name | fl_TC_7_1_3_5_3_ENDC_NR_TestBody |
| --- | --- |
| Reason for change | In function fl_TC_7_1_3_5_3_ENDC_NR_TestBody, Reestablish DRB2 is performed with activation time . And in step 7, as part of fl_TC_7_1_3_5_3_ENDC_NR_TestBody , UL grant information is sent with immediate activation time . As the grant is sent with immediate activation , there is a possbility that UE can send the PDCP status report in the middle of DRB2 reestablish steps . Hence it is proposed to send the Ul grant with the timing information in align to the DRB2 configuration. |
| Summary of change | V_Timing is passed as parameter to the function f_TC_7_1_3_5_3_NR_TestBody_Step7to9 |
| TTCN module | ENDC/7_1_3/PDCP_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_7_1_3_5_3_ENDC_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on ENDC_NR_PTC { var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var template (value) RadioBearerConfig v_RadioBearerConfig; var template (omit) PDCP_Config v_PDCP_Config; var template (value) CellGroupConfig v_SCGConfig := f_NR_GetSCGConfigDef(nr_Cell1); // @sic R5s191042, R5-198180 sic@ var SubFrameTiming_Type v_Timing; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc; f_TC_7_1_3_5_3_NR_TestBody_Step1to4A (p_NR_PDCP_State, nr_Cell1); //@siclog "Step 5-6" siclog@ // The SS transmits a RRCConnectionReconfiguration message containing NR RRCReconfiguration. // The UE transmits a RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete. v_PDCP_Config := cs_NR_PDCP_Config_Split_moreThan1RLC; v_RadioBearerConfig := cs_NR_RbRecoverPDCP(secondary, v_PDCP_Config); //Prepare CellGroupConfig as per Table 7.1.3.5.3.3.3-2A: CellGroupConfig-PDCP v_SCGConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; // @sic R5s191042, R5-198180 sic@ v_SCGConfig.spCellConfig := omit; //Reestablish DRB2 RLC in SS v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1); f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {cs_NR_SS_DRB_Release(tsc_NR_DRB2)}, cs_TimingInfo_NR(v_Timing)); v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing, 5); // add 5ms v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, tsc_NR_DRB2, -, cs_NR_PDCP_RbConfig_TransparentMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList, cs_TimingInfo_NR(v_Timing)); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, v_SCGConfig, -, -, -, noRACH_Check); //@sic draft prose CR 7.1.3.5.3: no HO -> no RACH sic@ f_TC_7_1_3_5_3_NR_TestBody_Step7to9 (p_NR_PDCP_State, nr_Cell1, nr_Cell1); //@sic R5s210082 sic@ //Coordination message to trigger the EUTRA Postamble f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); }

After Change:

function fl_TC_7_1_3_5_3_ENDC_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on ENDC_NR_PTC { var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var template (value) RadioBearerConfig v_RadioBearerConfig; var template (omit) PDCP_Config v_PDCP_Config; var template (value) CellGroupConfig v_SCGConfig := f_NR_GetSCGConfigDef(nr_Cell1); // @sic R5s191042, R5-198180 sic@ var SubFrameTiming_Type v_Timing; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc; f_TC_7_1_3_5_3_NR_TestBody_Step1to4A (p_NR_PDCP_State, nr_Cell1); //@siclog "Step 5-6" siclog@ // The SS transmits a RRCConnectionReconfiguration message containing NR RRCReconfiguration. // The UE transmits a RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete. v_PDCP_Config := cs_NR_PDCP_Config_Split_moreThan1RLC; v_RadioBearerConfig := cs_NR_RbRecoverPDCP(secondary, v_PDCP_Config); //Prepare CellGroupConfig as per Table 7.1.3.5.3.3.3-2A: CellGroupConfig-PDCP v_SCGConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; // @sic R5s191042, R5-198180 sic@ v_SCGConfig.spCellConfig := omit; //Reestablish DRB2 RLC in SS v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1); f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {cs_NR_SS_DRB_Release(tsc_NR_DRB2)}, cs_TimingInfo_NR(v_Timing)); v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing, 5); // add 5ms v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, tsc_NR_DRB2, -, cs_NR_PDCP_RbConfig_TransparentMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList, cs_TimingInfo_NR(v_Timing)); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, v_SCGConfig, -, -, -, noRACH_Check); //@sic draft prose CR 7.1.3.5.3: no HO -> no RACH sic@ f_TC_7_1_3_5_3_NR_TestBody_Step7to9 (p_NR_PDCP_State, nr_Cell1, nr_Cell1,v_Timing); //@sic R5s210082 sic@ //Coordination message to trigger the EUTRA Postamble f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); }

## Correction to function f_TC_7_1_3_5_3_NR_TestBody_Step7to9

| Function name | f_TC_7_1_3_5_3_NR_TestBody_Step7to9 |
| --- | --- |
| Reason for change | In function fl_TC_7_1_3_5_3_ENDC_NR_TestBody, Reestablish DRB2 is performed with activation time . And in step 7, as part of fl_TC_7_1_3_5_3_ENDC_NR_TestBody , UL grant information is sent with immediate activation time . As the grant is sent with immediate activation , there is a possbility that UE can send the PDCP status report in the middle of DRB2 reestablish steps . Hence it is proposed to send the Ul grant with the timing information in align to the DRB2 configuration. |
| Summary of change | One Ul grant for Pdcp Status report is sent with activation time in align with the Drb2 reconfiguration |
| TTCN module | /NR_TC_Common/7_1_3/PDCP_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_3_5_3_NR_TestBody_Step7to9(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, NR_CellId_Type p_PSCellId, NR_CellId_Type p_PCellId) runs on NR_BASE_PTC { var DRB_Identity v_DrbId := p_NR_PDCP_State.DRB_Id; var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var integer k; var NR_CellId_Type v_PSCellId := p_PSCellId; var NR_CellId_Type v_NRCellId := p_PCellId; // CellId to be used in step 9 var template (value) NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(v_PSCellId, tsc_NR_BWP_Id); var SubFrameTiming_Type v_TimingUL; if (f_GetTestcaseAttrib_ENDC(testcasename())) { v_NRCellId := p_PSCellId;}; //ENDC tc p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc; //@siclog "Step 7" siclog@ // The SS assigns 1 UL grant of sufficent size to allow the UE to send only PDCP status report. // 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header + 3 octets BSR or padding = 13 octets v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(104, v_NR_UplinkBWP); // @sic R5s191042, R5-198180 sic@ f_NR_OneULGrantTransmission(v_PSCellId, cs_TimingInfo_Now, v_NR_ResourceAllocation); //@siclog "Step 8" siclog@ // The UE sends PDCP Control PDUs on PSCell via RLC-AM RB with the following content to the SS: // D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 3. DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(v_PSCellId, v_DrbId, cr_RlcBearerRouting_NR(v_PSCellId), -, {cr_NR_PDCP_STATUS_REPORT(3)})); //@siclog "Step 8A" siclog@ // After 100 ms the SS allocates 3 UL grants every 20ms of sufficient size to enable the UE to return each received PDCP PDU in one looped back PDCP PDU on PSCell. // 44 resp. 64 octets IPData + 3 octets PDCP header + 3 octets RLC header + 2 octets MAC header + 2 octets BSR or padding = 54 resp. 74 octets if (f_IpAddressIsIPv4(f_LoopbackModeB_IP_Address_UE())) { // @sic R5s200153 sic@ v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(432, v_NR_UplinkBWP); // @sic R5s191042, R5-198180 sic@ } else { v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(608, v_NR_UplinkBWP); } v_TimingUL := f_NR_GetNextSendOccasion_UL(v_PSCellId); f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(v_PSCellId, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, 3); //@sic R5-197255, R5s200029 sic@ //EXCEPTION: Step 9 shall be repeated for k=0 to 2 (increment=1). //@siclog "Step 9" siclog@ // Check: Does the UE send the PDCP Data PDU #k via the AM RLC entity configured for PSCell for Connectivity ENDC and PCell for Connectivity (NR-DC). // D/C field = 1 (PDCP Data PDU) and PDCP SN = k. Data is previously received data from PDU #k ? p_NR_PDCP_State.RX_NEXT:= 0; for (k:= 0; k <= 2; k:= k+1) { f_NR_PDCP_Recv(p_NR_PDCP_State,v_NRCellId,v_IPData,cr_RlcBearerRouting_NR(v_NRCellId)); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); }

After Change:

function f_TC_7_1_3_5_3_NR_TestBody_Step7to9(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, NR_CellId_Type p_PSCellId, NR_CellId_Type p_PCellId, SubFrameTiming_Type v_Timing) runs on NR_BASE_PTC { var DRB_Identity v_DrbId := p_NR_PDCP_State.DRB_Id; var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var integer k; var NR_CellId_Type v_PSCellId := p_PSCellId; var NR_CellId_Type v_NRCellId := p_PCellId; // CellId to be used in step 9 var template (value) NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(v_PSCellId, tsc_NR_BWP_Id); var SubFrameTiming_Type v_TimingUL; if (f_GetTestcaseAttrib_ENDC(testcasename())) { v_NRCellId := p_PSCellId;}; //ENDC tc p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc; //@siclog "Step 7" siclog@ // The SS assigns 1 UL grant of sufficent size to allow the UE to send only PDCP status report. // 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header + 3 octets BSR or padding = 13 octets v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(104, v_NR_UplinkBWP); // @sic R5s191042, R5-198180 sic@ f_NR_OneULGrantTransmission(v_PSCellId, cs_TimingInfo_NR(v_Timing), v_NR_ResourceAllocation); //@siclog "Step 8" siclog@ // The UE sends PDCP Control PDUs on PSCell via RLC-AM RB with the following content to the SS: // D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 3. DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(v_PSCellId, v_DrbId, cr_RlcBearerRouting_NR(v_PSCellId), -, {cr_NR_PDCP_STATUS_REPORT(3)})); //@siclog "Step 8A" siclog@ // After 100 ms the SS allocates 3 UL grants every 20ms of sufficient size to enable the UE to return each received PDCP PDU in one looped back PDCP PDU on PSCell. // 44 resp. 64 octets IPData + 3 octets PDCP header + 3 octets RLC header + 2 octets MAC header + 2 octets BSR or padding = 54 resp. 74 octets if (f_IpAddressIsIPv4(f_LoopbackModeB_IP_Address_UE())) { // @sic R5s200153 sic@ v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(432, v_NR_UplinkBWP); // @sic R5s191042, R5-198180 sic@ } else { v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(608, v_NR_UplinkBWP); } v_TimingUL := f_NR_GetNextSendOccasion_UL(v_PSCellId); f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(v_PSCellId, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, 3); //@sic R5-197255, R5s200029 sic@ //EXCEPTION: Step 9 shall be repeated for k=0 to 2 (increment=1). //@siclog "Step 9" siclog@ // Check: Does the UE send the PDCP Data PDU #k via the AM RLC entity configured for PSCell for Connectivity ENDC and PCell for Connectivity (NR-DC). // D/C field = 1 (PDCP Data PDU) and PDCP SN = k. Data is previously received data from PDU #k ? p_NR_PDCP_State.RX_NEXT:= 0; for (k:= 0; k <= 2; k:= k+1) { f_NR_PDCP_Recv(p_NR_PDCP_State,v_NRCellId,v_IPData,cr_RlcBearerRouting_NR(v_NRCellId)); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); }

## Correction to function fl_TC_7_1_3_5_3_NR_TestBody

| Function name | fl_TC_7_1_3_5_3_NR_TestBody |
| --- | --- |
| Reason for change | In function fl_TC_7_1_3_5_3_ENDC_NR_TestBody, Reestablish DRB2 is performed with activation time . And in step 7, as part of fl_TC_7_1_3_5_3_ENDC_NR_TestBody , UL grant information is sent with immediate activation time . As the grant is sent with immediate activation , there is a possbility that UE can send the PDCP status report in the middle of DRB2 reestablish steps . Hence it is proposed to send the Ul grant with the timing information in align to the DRB2 configuration. |
| Summary of change | Below changes are done as SA and ENDC versions are sharing the same function f_TC_7_1_3_5_3_NR_TestBody_Step7to9 |
| TTCN module | NR5GC/7_1_3/PDCP_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_7_1_3_5_3_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State, NR_CellId_Type p_PcellId, NR_CellId_Type p_PScellId, integer p_DRB_Id) runs on NR_BASE_PTC { var template (value) DRB_ToAddModList v_DRB_ToAddModList; var template (value) RadioBearerConfig v_RadioBearerConfig2; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) PDCP_Config v_PDCP_Config_Split; var template (value) CellGroupConfig v_CellGroupConfigMCG; var template (value) CellGroupConfig v_CellGroupConfigSCG; //Test Body step 1 to 4A f_TC_7_1_3_5_3_NR_TestBody_Step1to4A(p_NR_PDCP_State, p_PScellId); //@siclog "Step 5-6" siclog@ // The SS transmits a RRCConnectionReconfiguration message containing NR RRCReconfiguration. // The UE transmits a RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete. v_CellGroupConfigSCG := f_NR_GetSCGConfigNRDC_SCG(p_PScellId, p_DRB_Id, -, true_); v_CellGroupConfigMCG := f_NR_GetCellgroupConfig_DRB (p_PcellId, {p_DRB_Id}, false); v_PDCP_Config_Split := cs_NR_PDCP_Config_Split(-, -, tsc_NR_CellGroupId_MCG); v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, p_DRB_Id, -, true_, v_PDCP_Config_Split)}; v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(v_CellGroupConfigSCG, v_CellGroupConfigMCG, v_RadioBearerConfig2); //SS transmits an RRCReconfiguration message containing NR RadioBearerConfig f_NR_SendRRCReconfiguration (p_PScellId, -,-, v_V1530Ext); //Test Body step 7 to 9 f_TC_7_1_3_5_3_NR_TestBody_Step7to9(p_NR_PDCP_State, p_PScellId, nr_Cell1); }

After Change:

function fl_TC_7_1_3_5_3_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State, NR_CellId_Type p_PcellId, NR_CellId_Type p_PScellId, integer p_DRB_Id) runs on NR_BASE_PTC { var template (value) DRB_ToAddModList v_DRB_ToAddModList; var template (value) RadioBearerConfig v_RadioBearerConfig2; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) PDCP_Config v_PDCP_Config_Split; var template (value) CellGroupConfig v_CellGroupConfigMCG; var template (value) CellGroupConfig v_CellGroupConfigSCG; var SubFrameTiming_Type v_Timing; //Test Body step 1 to 4A f_TC_7_1_3_5_3_NR_TestBody_Step1to4A(p_NR_PDCP_State, p_PScellId); //@siclog "Step 5-6" siclog@ // The SS transmits a RRCConnectionReconfiguration message containing NR RRCReconfiguration. // The UE transmits a RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete. v_CellGroupConfigSCG := f_NR_GetSCGConfigNRDC_SCG(p_PScellId, p_DRB_Id, -, true_); v_CellGroupConfigMCG := f_NR_GetCellgroupConfig_DRB (p_PcellId, {p_DRB_Id}, false); v_PDCP_Config_Split := cs_NR_PDCP_Config_Split(-, -, tsc_NR_CellGroupId_MCG); v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, p_DRB_Id, -, true_, v_PDCP_Config_Split)}; v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(v_CellGroupConfigSCG, v_CellGroupConfigMCG, v_RadioBearerConfig2); //SS transmits an RRCReconfiguration message containing NR RadioBearerConfig f_NR_SendRRCReconfiguration (p_PScellId, -,-, v_V1530Ext); //Test Body step 7 to 9 v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1); f_TC_7_1_3_5_3_NR_TestBody_Step7to9(p_NR_PDCP_State, p_PScellId, nr_Cell1,v_Timing); }