3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220072

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2286 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC Logged MDT test case 8.1.6.1.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - SS is expecting not to include IE logMeasAvailable in the RRCSetupComplete at step 7. Hence UE will not populate nonCriticalEextension for UE_MeasurementsAvailable_r16.. - UE use SRB2 when it will be having logMeasReport_r16 as per 38.331 clause 5.7.10.3 at step 21. “1> if the logMeasReport is included in the UEInformationResponse: 2> submit the UEInformationResponse message to lower layers for transmission via SRB2; 2> discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers; - else: - submit the UEInformationResponse message to lower layers for transmission via SRB1.” |  |  |  |  |  |  |  |  |
| Summary of change: |  | - nonCriicalExtension omitted from RRCSetupComplete. - UEInformationResponse reception modified for SRB2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will fail conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

3	Branches executed	3

4	Execution Log Files	3

4.1	Mediatek MT6983	3

5	References	4

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.6.1.2.4 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	8.1.6.1.2.4

ATS Version:	iwd-TTCN3-B2020-09_D21wk49

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR,

UE used:	Mediatek MT6983

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_8_1_6_1_2_4_TestBody() |
| --- | --- |
| Reason for change | - SS is expecting not to include IE logMeasAvailable in the RRCSetupComplete at step 7. Hence UE will not populate noncriticalextension for UE_MeasurementsAvailable_r16. - UE use SRB2 when it will be having logMeasReport_r16 as per 38.331 clause 5.7.10.3 at step 21. “1> if the logMeasReport is included in the UEInformationResponse: 2> submit the UEInformationResponse message to lower layers for transmission via SRB2; 2> discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers; 1> else: 2> submit the UEInformationResponse message to lower layers for transmission via SRB1.” |
| Summary of change | - nonCriicalExtension omitted from RRCSetupComplete. - UEInformationResponse reception modified for SRB2. |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_6_1_2_4_TestBody() runs on NR5GC_PTC { // @sic R5-216300 sic@ var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var PLMN_Identity v_PlmnID_NRCell1; var PLMN_Identity v_PlmnID_NRCell11; var TrackingAreaCode v_TAC_NRCell11; var B48_Type v_AbsoluteTimeInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_GutiParameters_Type v_GutiParams ; var template(present) NG_MobileIdentity v_S_TMSI; var template(present) LogMeasInfo_r16 v_LogMeasInfo_r16; var CellIdentity v_CellIdentity_NRCell11; var PhysCellId v_PhysCellId_NRCell1; var ARFCN_ValueNR v_CarrierFreq_NRCell1; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (present) MeasQuantityResults v_MeasQuantityResults; var integer v_FieldCnt; var integer i; v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, -78, -82), cs_NR_CellPower(nr_Cell11, -85, -91)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, -85, -91), cs_NR_CellPower(nr_Cell11, -78, -82)}; f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Steps 1" siclog@ //SS transmits a LoggedMeasurementConfiguration message with reportType is set to 'periodical' and LoggingInterval is set to 'ms10240', to configure the UE to perform logging of measurement results while in RRC_IDLE v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN(nr_Cell1); v_AbsoluteTimeInfo := f_MDT_GetAbsolute_TimeStamp(); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo, cs_LoggedMeasurementConfiguration_reportType_Periodical(ms10240)))); //@siclog "Steps 2" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 3" siclog@ //Wait 5s and the LoggingInterval will not be expired f_Delay(5.0); //@siclog "Steps 4" siclog@ //The SS changes Cell 1 and Cell 11 levels according to the row "T1" in table 8.1.6.1.2.4.3.2-1/2 f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Steps 5" siclog@ //The UE transmits an RRCSetupRequest message on Cell 11 to initiate a tracking area update procedure //@siclog "Steps 6" siclog@ //SS transmit an RRCSetup message //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell11,tsc_NR_RRC_TI_Def,?,-,-,-,-, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16_AllOmit)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Steps 8-10" siclog@ //Steps 4 to 6 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on Cell 11 f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell11, v_GMM_MobilityInfo); //@siclog "Steps 11" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 12-14" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508 Table 4.5.4.2-3 are executed to successfully complete the service request procedure f_NR_Paging_Def(nr_Cell11, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell11); f_NR_RRCSetup_Def(nr_Cell11); //@siclog "Steps 15" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell11, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(true_, omit, omit, omit, omit)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell11); v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get(), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0010'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } //@siclog "Steps 16-19" siclog@ //Steps 5 to 8 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell11, v_ReceivedMsg); //@siclog "Steps 20" siclog@ //The SS transmits a UEInformationRequest message on Cell 11 SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell11, -, cs_NR_UEInformationRequest_LOG)); //@siclog "Steps 21" siclog@ //Check: Does the UE transmit a UEInformationResponse message SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell11, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, omit, omit, cr_LogMeasReport_r16(v_AbsoluteTimeInfo, cr_TraceReference_r16(v_PlmnID_NRCell1), '001A'O, '05'O, ?, omit,omit,omit), omit, omit, omit, omit))) -> value v_ReceivedAsp ; v_FieldCnt := lengthof(v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16); if (v_FieldCnt < 2) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21: Not enough LogMeasurementInfo entries"); } v_PlmnID_NRCell11 := f_NR_CellInfo_GetPLMN(nr_Cell11); v_CellIdentity_NRCell11 := f_NR_CellInfo_GetCellIdentity(nr_Cell11); v_TAC_NRCell11 := f_NR_CellInfo_GetTAC (nr_Cell11); v_CarrierFreq_NRCell1 := f_NR_CellInfo_GetFrequencySSB(nr_Cell1); v_PhysCellId_NRCell1 := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_MeasQuantityResults := f_NR_Get_MeasQuantityResults(); for (i:=0; i<v_FieldCnt; i:=i+1) { v_LogMeasInfo_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[i]; if (not match(valueof(v_LogMeasInfo_r16), cr_LogMeasInfo_r16(-, cr_CGI_Info_Logging_r16(v_PlmnID_NRCell11, v_CellIdentity_NRCell11, v_TAC_NRCell11), cr_MeasResultServCell_r16(v_MeasQuantityResults), {cr_MeasResultLogging2NR_r16(v_CarrierFreq_NRCell1, {cr_MeasResultLoggingNR_r16(v_PhysCellId_NRCell1, v_MeasQuantityResults)}) }, omit))){ if (i == v_FieldCnt-1) {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21: None of the LogMeasurementInfo matched the template")} } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "step21"); break; } } } //End of f_TC_8_1_6_1_2_4_TestBody

After Change

function f_TC_8_1_6_1_2_4_TestBody() runs on NR5GC_PTC { // @sic R5-216300 sic@ var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var PLMN_Identity v_PlmnID_NRCell1; var PLMN_Identity v_PlmnID_NRCell11; var TrackingAreaCode v_TAC_NRCell11; var B48_Type v_AbsoluteTimeInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_GutiParameters_Type v_GutiParams ; var template(present) NG_MobileIdentity v_S_TMSI; var template(present) LogMeasInfo_r16 v_LogMeasInfo_r16; var CellIdentity v_CellIdentity_NRCell11; var PhysCellId v_PhysCellId_NRCell1; var ARFCN_ValueNR v_CarrierFreq_NRCell1; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (present) MeasQuantityResults v_MeasQuantityResults; var integer v_FieldCnt; var integer i; v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, -78, -82), cs_NR_CellPower(nr_Cell11, -85, -91)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, -85, -91), cs_NR_CellPower(nr_Cell11, -78, -82)}; f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Steps 1" siclog@ //SS transmits a LoggedMeasurementConfiguration message with reportType is set to 'periodical' and LoggingInterval is set to 'ms10240', to configure the UE to perform logging of measurement results while in RRC_IDLE v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN(nr_Cell1); v_AbsoluteTimeInfo := f_MDT_GetAbsolute_TimeStamp(); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo, cs_LoggedMeasurementConfiguration_reportType_Periodical(ms10240)))); //@siclog "Steps 2" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 3" siclog@ //Wait 5s and the LoggingInterval will not be expired f_Delay(5.0); //@siclog "Steps 4" siclog@ //The SS changes Cell 1 and Cell 11 levels according to the row "T1" in table 8.1.6.1.2.4.3.2-1/2 f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Steps 5" siclog@ //The UE transmits an RRCSetupRequest message on Cell 11 to initiate a tracking area update procedure //@siclog "Steps 6" siclog@ //SS transmit an RRCSetup message //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell11,tsc_NR_RRC_TI_Def,?,-,-,-,-, -); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Steps 8-10" siclog@ //Steps 4 to 6 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on Cell 11 f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell11, v_GMM_MobilityInfo); //@siclog "Steps 11" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 12-14" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508 Table 4.5.4.2-3 are executed to successfully complete the service request procedure f_NR_Paging_Def(nr_Cell11, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell11); f_NR_RRCSetup_Def(nr_Cell11); //@siclog "Steps 15" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell11, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(true_, omit, omit, omit, omit)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell11); v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get(), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0010'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } //@siclog "Steps 16-19" siclog@ //Steps 5 to 8 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell11, v_ReceivedMsg); //@siclog "Steps 20" siclog@ //The SS transmits a UEInformationRequest message on Cell 11 SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell11, -, cs_NR_UEInformationRequest_LOG)); //@siclog "Steps 21" siclog@ //Check: Does the UE transmit a UEInformationResponse message SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell11,tsc_NR_RbId_SRB2, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, omit, omit, cr_LogMeasReport_r16(v_AbsoluteTimeInfo, cr_TraceReference_r16(v_PlmnID_NRCell1), '001A'O, '05'O, ?, omit,omit,omit), omit, omit, omit, omit))) -> value v_ReceivedAsp ; v_FieldCnt := lengthof(v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16); if (v_FieldCnt < 2) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21: Not enough LogMeasurementInfo entries"); } v_PlmnID_NRCell11 := f_NR_CellInfo_GetPLMN(nr_Cell11); v_CellIdentity_NRCell11 := f_NR_CellInfo_GetCellIdentity(nr_Cell11); v_TAC_NRCell11 := f_NR_CellInfo_GetTAC (nr_Cell11); v_CarrierFreq_NRCell1 := f_NR_CellInfo_GetFrequencySSB(nr_Cell1); v_PhysCellId_NRCell1 := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_MeasQuantityResults := f_NR_Get_MeasQuantityResults(); for (i:=0; i<v_FieldCnt; i:=i+1) { v_LogMeasInfo_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[i]; if (not match(valueof(v_LogMeasInfo_r16), cr_LogMeasInfo_r16(-, cr_CGI_Info_Logging_r16(v_PlmnID_NRCell11, v_CellIdentity_NRCell11, v_TAC_NRCell11), cr_MeasResultServCell_r16(v_MeasQuantityResults), {cr_MeasResultLogging2NR_r16(v_CarrierFreq_NRCell1, {cr_MeasResultLoggingNR_r16(v_PhysCellId_NRCell1, v_MeasQuantityResults)}) }, omit))){ if (i == v_FieldCnt-1) {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21: None of the LogMeasurementInfo matched the template")} } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "step21"); break; } } } //End of f_TC_8_1_6_1_2_4_TestBody

# Branches executed

On Anritsu, the test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2.

# Execution Log Files

## Mediatek MT6983

The Mediatek MT6983 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\ TC_8_1_6_1_2_4_LOG.xml

- PICS/PIXIT parameter file:

Anritsu\ TC_8_1_6_1_2_4_PIXIT.txt

# References

| [1] | R5s220075 - Supporting information for addition of NR5GC Logged MDT test case 8.1.6.1.2.4 |
| --- | --- |