3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220176

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2328 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.4.1.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1)In the current implementation of function fl_TC_8_1_4_1_5_TestBody , at step 2 , after sending the reconfiguration to trigger HO , timer_T304 is started but there is no corresponding place holder to handle the time out. 2) In the function f_NR5GC_RRCReestablishment_Steps3To7, when source cell is same as target cell , reset and release of bearers are done, which is leading to race condition |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1)T304 related function call is removed as the time out is leading to unexpected test case failure. Besides this is a UE side timer, and of a very short duration (1s), so checking its expiry or cancelling it will not really aff much value. 2) While configuring the bearers Follow On flag is set so that race condition can be avoided ; Introducing Follow On flag in rb configuration is made in align with step 13 , reset and release bearer |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to the function fl_TC_8_1_4_1_5_TestBody	2

1.2	Correction to the function f_NR5GC_RRCReestablishment_Steps3To7	4

# Corrections required

## Correction to the function fl_TC_8_1_4_1_5_TestBody

| Function name | fl_TC_8_1_4_1_5_TestBody |
| --- | --- |
| Reason for change | In the current implementation of function fl_TC_8_1_4_1_5_TestBody , at step 2 , after sending the reconfiguration to trigger HO , timer_T304 is started but there is no corresponding place holder to handle the time out. |
| Summary of change | T304 related function call is removed as the time out is leading to unexpected test case failure |
| TTCN module | develop/Common/NR5GC/8_1_4/ RRC_IntraNR_Handover_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_4_1_5_TestBody () runs on NR5GC_PTC { var template (value) DL_DCCH_Message v_RRCReconfigurationHO; var float v_T304Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 1.0); //T304 is ms1000 according to 38.508-1 Table 4.6.3-19. var SubFrameTiming_Type v_SubframeTiming; //Power level as per Table 8.1.4.1.5.3.2-1 var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)}; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; timer t_T304Min; // Reconfigure nr_Cell2 to no respond to any RA Preamble // Table 8.1.4.1.5.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond. f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 1" siclog@ // The SS changes the power level setting according to the row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ // The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. // Note: failure case UE doesn't go to nr_Cell2 => the standard function need not be used v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO)); t_T304Min.start(v_T304Min); //@siclog "Step 3" siclog@ //The SS changes the power level setting according to the row "T2". f_NR_SetCellPowerList(v_CellPowerList_AtT2); << SKIPPED CODE >> }

After Change:

function fl_TC_8_1_4_1_5_TestBody () runs on NR5GC_PTC { var template (value) DL_DCCH_Message v_RRCReconfigurationHO; var float v_T304Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 1.0); //T304 is ms1000 according to 38.508-1 Table 4.6.3-19. var SubFrameTiming_Type v_SubframeTiming; //Power level as per Table 8.1.4.1.5.3.2-1 var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)}; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; //timer t_T304Min; // Reconfigure nr_Cell2 to no respond to any RA Preamble // Table 8.1.4.1.5.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond. f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 1" siclog@ // The SS changes the power level setting according to the row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ // The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. // Note: failure case UE doesn't go to nr_Cell2 => the standard function need not be used v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO)); //COMMENTED OUT t_T304Min.start(v_T304Min); << SKIPPED CODE >> }

## Correction to the function f_NR5GC_RRCReestablishment_Steps3To7

| Function name | f_NR5GC_RRCReestablishment_Steps3To7 |
| --- | --- |
| Reason for change | In the function f_NR5GC_RRCReestablishment_Steps3To7, when source cell is same as target cell , reset and release of bearers are done next to next, which is leading to race condition |
| Summary of change | While configuring the bearers Follow On flag is set so that race condition can be avoided ; Introducing Follow On flag in rb configuration is made in align with step 13 , reset and release bearer |
| TTCN module | ttcn-3/ttcn/develop/Common/NR5GC/ NR5GC_RRCSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_RRCReestablishment_Steps3To7(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, NextHopChainingCount p_NextHopChainingCount := 0, template(omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit, template RRCReestablishmentComplete_v1610_IEs p_RRCReestablishmentComplete_v1610_IEs := omit ) runs on NR_BASE_PTC { // @sic R5-206369 R5-213516: added p_MacBearerRoutingCA sic@ var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(p_NR_SourceCellId); // 3. Source Cell: If the Source Cell is the same as the Target Cell: // Reset SRBs and release DRBs // Stop periodic TA if (p_NR_SourceCellId == p_NR_TargetCellId) { f_NR_SS_SRBs_DRBs_Release(p_NR_SourceCellId, cs_TimingInfo_Now, v_DrbIdList, tsc_NoCnfReq); //@sic R5-211417 R5s211169 sic@ f_NR_SS_SRBs_DRBs_Config(p_NR_SourceCellId, cs_TimingInfo_Now, omit, tsc_NoCnfReq); // @sic R5-217830 sic@ } // 4. void // @sic R5-217830 sic@ // 5. Target Cell: Activate Security f_NR_SS_AS_ActivateSecurity_Current(p_NR_TargetCellId, p_NextHopChainingCount, -, true); // 6. Target : Send RRCReestablishment SRB.send (cas_NR_SRB1_RrcPdu_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, cs_38508_RRCReestablishment_Def(tsc_NR_RRC_TI_Def, p_NextHopChainingCount), p_MacBearerRoutingCA)); // 7. Target Cell: Receive RRCReestablishmentComplete SRB.receive ( car_NR_SRB1_RrcPdu_IND ( p_NR_TargetCellId, cr_38508_RRCReestablishmentComplete(-, p_RRCReestablishmentComplete_v1610_IEs), -, p_MacBearerRoutingCA) ); }

After Change:

function f_NR5GC_RRCReestablishment_Steps3To7(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, NextHopChainingCount p_NextHopChainingCount := 0, template(omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit, template RRCReestablishmentComplete_v1610_IEs p_RRCReestablishmentComplete_v1610_IEs := omit ) runs on NR_BASE_PTC { // @sic R5-206369 R5-213516: added p_MacBearerRoutingCA sic@ var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(p_NR_SourceCellId); // 3. Source Cell: If the Source Cell is the same as the Target Cell: // Reset SRBs and release DRBs // Stop periodic TA if (p_NR_SourceCellId == p_NR_TargetCellId) { f_NR_SS_SRBs_DRBs_Release(p_NR_SourceCellId, cs_TimingInfo_Now, v_DrbIdList, tsc_NoCnfReq); //@sic R5-211417 R5s211169 sic@ f_NR_SS_SRBs_DRBs_Config(p_NR_SourceCellId, cs_TimingInfo_Now, omit, tsc_FollowOnFlag); // @sic R5-217830 sic@ } // 4. void // @sic R5-217830 sic@ // 5. Target Cell: Activate Security f_NR_SS_AS_ActivateSecurity_Current(p_NR_TargetCellId, p_NextHopChainingCount, -, true); // 6. Target : Send RRCReestablishment SRB.send (cas_NR_SRB1_RrcPdu_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, cs_38508_RRCReestablishment_Def(tsc_NR_RRC_TI_Def, p_NextHopChainingCount), p_MacBearerRoutingCA)); // 7. Target Cell: Receive RRCReestablishmentComplete SRB.receive ( car_NR_SRB1_RrcPdu_IND ( p_NR_TargetCellId, cr_38508_RRCReestablishmentComplete(-, p_RRCReestablishmentComplete_v1610_IEs), -, p_MacBearerRoutingCA) ); }