3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220207

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2347 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The changes mentioned in this document are needed in addition to R5s211708. In the current TTCN implementation of function fl_TC_11_1_6_NR5GC_TestBody, after Step 6 the Pdu session info list needs to be sent to Eutra. Also the NR Freq list should be sent to Eutra. This is missing. In the current TTCN implementation of function fl_TC_11_1_6_NR5GC_EUTRA_TestBody, timer t_Wait should be started before the alt statement. This needs to be fixed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In function fl_TC_11_1_6_NR5GC_TestBody, called functions f_NR5GC_SendPduSessionInfoListToEUTRA and f_NR5GC_IRAT_UECapabilityTransfer after calling function f_NR5GC_Send_GutiParametersToEUTRA. Started the timer t_wait after call to function fl_TAUReject_AttachWithoutN26 Please see detailed changes below, |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to function fl_TC_11_1_6_NR5GC_TestBody	3

1.2	Correction to function fl_TC_11_1_6_NR5GC_EUTRA_TestBody	6

# Corrections required

## Correction to function fl_TC_11_1_6_NR5GC_TestBody

| Function name | fl_TC_11_1_6_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation of function fl_TC_11_1_6_NR5GC_TestBody, after Step 6 the Pdu session info list needs to be sent to Eutra. Also the NR Freq list should be sent to Eutra. This is missing. This needs to be fixed. |
| Summary of change | In function fl_TC_11_1_6_NR5GC_TestBody, called functions f_NR5GC_SendPduSessionInfoListToEUTRA and f_NR5GC_IRAT_UECapabilityTransfer after calling function f_NR5GC_Send_GutiParametersToEUTRA. |
| TTCN module | EPS_Fallback_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_11_1_6_NR5GC_TestBody() runs on NR5GC_PTC { var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; var template (value) FreqBandList v_NR_FreqList; var integer v_NRBandInd := f_NR_CellInfo_GetBandIndicator(nr_Cell1); //@siclog "Step 2" siclog@ // The SS transmits a Paging message on NR Cell 1 f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Steps 3-4" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?); //@siclog "Step 5" siclog@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, ?, -, */*?*/)) {//WA#11.1.6_Correction_1.3 f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR_RRC_ActivateSecurity (nr_Cell1, v_ReceivedMsg.SecurityProtection.NasCount); //WA#R5s211708 // @siclog "Step 5A" siclog@ f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5-210133 sic@ f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Change Power Level")); // @sic R5-216190 sic@ //@siclog "Step 6" siclog@ //The SS transmits an RRCRelease message (IE redirectedCarrierInfo including EUTRA cell). v_RedirectedCarrierInfo := cs_NR_RedirectedCarrierInfoToEUTRA(v_CoOrd_SysInfo.Eutra[0].Arfcn); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCRelease(tsc_NR_RRC_TI_Def, -, v_RedirectedCarrierInfo))); // @sic R5s210662 sic@ f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetNextSendOccasion_DL(nr_Cell1)); // @sic R5s210662 sic@ // Send authentication paramters to EUTRA f_NR5GC_Send_GutiParametersToEUTRA(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); }

After Change:

function fl_TC_11_1_6_NR5GC_TestBody() runs on NR5GC_PTC { var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; var template (value) FreqBandList v_NR_FreqList; var integer v_NRBandInd := f_NR_CellInfo_GetBandIndicator(nr_Cell1); //@siclog "Step 2" siclog@ // The SS transmits a Paging message on NR Cell 1 f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Steps 3-4" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?); //@siclog "Step 5" siclog@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, ?, -, */*?*/)) {//WA#11.1.6_Correction_1.3 f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR_RRC_ActivateSecurity (nr_Cell1, v_ReceivedMsg.SecurityProtection.NasCount); //WA#R5s211708 // @siclog "Step 5A" siclog@ f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5-210133 sic@ f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Change Power Level")); // @sic R5-216190 sic@ //@siclog "Step 6" siclog@ //The SS transmits an RRCRelease message (IE redirectedCarrierInfo including EUTRA cell). v_RedirectedCarrierInfo := cs_NR_RedirectedCarrierInfoToEUTRA(v_CoOrd_SysInfo.Eutra[0].Arfcn); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCRelease(tsc_NR_RRC_TI_Def, -, v_RedirectedCarrierInfo))); // @sic R5s210662 sic@ f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetNextSendOccasion_DL(nr_Cell1)); // @sic R5s210662 sic@ // Send authentication paramters to EUTRA f_NR5GC_Send_GutiParametersToEUTRA(nr_Cell1); f_NR5GC_SendPduSessionInfoListToEUTRA(); //WA#11_1_6 f_NR5GC_IRAT_UECapabilityTransfer(nr_Cell1);//WA#11_1_6 f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); }

## Correction to function fl_TC_11_1_6_NR5GC_EUTRA_TestBody

| Function name | fl_TC_11_1_6_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation of function fl_TC_11_1_6_NR5GC_EUTRA_TestBody, timer t_Wait should be started before the alt statement. This needs to be fixed. |
| Summary of change | Started the timer t_wait after call to function fl_TAUReject_AttachWithoutN26 Please see detailed changes below, |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> f_EUTRA38_SetCellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); fl_TAUReject_AttachWithoutN26 (); alt { //WA#R5s211708 [] SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_Handover, ?)))) -> value v_ReceivedAsp { t_wait.stop; v_EpsBearerId := f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2(eutra_Cell1, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.pDN_CONNECTIVITY_REQUEST); a_EUTRA_IdleUpdated_ConnectToAdditionalPDN_Step3(eutra_Cell1); a_EUTRA38_MultiPDN_ConnectToAdditionalPDN_Step4(eutra_Cell1, v_EpsBearerId); } [] t_wait.timeout { } } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> f_EUTRA38_SetCellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); fl_TAUReject_AttachWithoutN26 (); t_wait.start; //WA#11.1.6 alt { //WA#R5s211708 [] SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_Handover, ?)))) -> value v_ReceivedAsp { t_wait.stop; v_EpsBearerId := f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2(eutra_Cell1, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.pDN_CONNECTIVITY_REQUEST); a_EUTRA_IdleUpdated_ConnectToAdditionalPDN_Step3(eutra_Cell1); a_EUTRA38_MultiPDN_ConnectToAdditionalPDN_Step4(eutra_Cell1, v_EpsBearerId); } [] t_wait.timeout { } } <<SKIPPED CODE>>