3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220229

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4662 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_EUTRA_WaitForIMSReRegister |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Current ttcn implementation does not handle IMS re-registration procedure correctly, as defined in table 4.5A.30.3-1, TS 36.508 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Made necessary changes, along with following specific changes: - Replaced template cms_IPCAN_IMS_IpcanInd with cms_IPCAN_IMS_ReregistrationStart, in order to trigger Reregistration procedure - Replaced function f_EUTRA_RbEst_Def with f_EUTRA_ServiceRequestAndActivate_SRB2_DRB, since Paging is not part of the procedure defined in table 4.5A.30.3-1, TS 36.508 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Testcases 6.1.1.1, 6.1.1.1b, 6.1.1.2, 6.1.1.2a, 6.1.1.3b, 6.1.1.6, 6.1.1.6a, 9.2.1.1.7, 9.2.1.1.7a, 9.2.3.1.4, 9.2.3.1.25, 9.2.3.2.13, 11.2.6, 11.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_EUTRA_WaitForIMSReRegister |
| --- | --- |
| Reason for change | Current ttcn implementation does not handle IMS re-registration procedure correctly, as defined in table 4.5A.30.3-1, TS 36.508 |
| Summary of change | Made necessary changes, along with following specific changes: - Replaced template cms_IPCAN_IMS_IpcanInd with cms_IPCAN_IMS_ReregistrationStart, in order to trigger Reregistration procedure - Replaced function f_EUTRA_RbEst_Def with f_EUTRA_ServiceRequestAndActivate_SRB2_DRB, since Paging is not part of the procedure defined in table 4.5A.30.3-1, TS 36.508 |
| TTCN module | EUTRA_NASSteps.ttcn |
| MCC160 Comment |  |

Before change:

function f_EUTRA_WaitForIMSReRegister(EUTRA_CellId_Type p_CellId) runs on EUTRA_PTC

{

var default v_DefaultVar := null;

f_IPCAN_DelayForUserPlaneSignalling(IMS[tsc_Index_IMS1], waitForIMS);

if (pc_IMS) {

v_DefaultVar := activate(a_EUTRA_IMS_QueryHandler(IMS[tsc_Index_IMS1]));

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_IpcanInd);

f_EUTRA_RbEst_Def(p_CellId);

f_EUTRA_RRC_ConnectionRelease(p_CellId);

deactivate(v_DefaultVar);

}

}

After change:

function f_EUTRA_WaitForIMSReRegister(EUTRA_CellId_Type p_CellId) runs on EUTRA_PTC

{

var default v_DefaultVar := null;

var IPCAN_RAN_Type v_RanType := f_EUTRA_IMS_GetRanType();

timer t_WaitForIMS := 10.0;

//code moved

if (pc_IMS) {

t_WaitForIMS.start;

alt {

[] t_WaitForIMS.timeout { } // Don't do anything

[] SRB.check(receive(car_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest(cr_EstablishmentCause_Any)))) {

t_WaitForIMS.stop;

v_DefaultVar := activate(a_EUTRA_IMS_QueryHandler(IMS[tsc_Index_IMS1]));

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_ReregistrationStart(v_RanType));

f_EUTRA_ServiceRequestAndActivate_SRB2_DRB(p_CellId);

f_IPCAN_DelayForUserPlaneSignalling(IMS[tsc_Index_IMS1], waitForIMS); // moved code

f_EUTRA_RRC_ConnectionRelease(p_CellId);

deactivate(v_DefaultVar);

}

}

}

}