3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220243

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2246 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC IRAT test case 8.1.1.3.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For test case 8.1.1.3.4, steps 3C-3D and 12-13 both state “Steps 5-6 of TS 38.508-1 [4] Table 4.9.7.2.2-1 of the generic procedure are performed.”. In TTCN implementation this is implemented by calling the function f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq() from fl_TAU_N1ToS1_CheckDataPath(), In this context the parameter p_FallbackSignalling should be set to true to stop the executing past step 6 on to steps 8a1 / 8b1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set p_FallbackSignalling true in the call to f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.3.4.NR5GC and others |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TAU_N1ToS1_CheckDataPath() |
| --- | --- |
| Reason for change | For test case 8.1.1.3.4, steps 3C-3D and 12-13 both state “Steps 5-6 of TS 38.508-1 [4] Table 4.9.7.2.2-1 of the generic procedure are performed.”. In TTCN implementation this is implemented by calling the function f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq() from fl_TAU_N1ToS1_CheckDataPath(), In this context the parameter p_FallbackSignalling should be set to true to stop the executing past step 6 on to steps 8a1 / 8b1. |
| Summary of change | Set p_FallbackSignalling true in the call to f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq() |
| TTCN module | RRC_ConnRel_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TAU_N1ToS1_CheckDataPath(EUTRA_CellId_Type p_CellId, RRC_TransactionIdentifier p_RRC_TI, charstring p_StepNo1 := "", charstring p_StepNo2 := "", template EUTRA_CellId_Type p_CellIdPreviouslyCampedOn := eutra_Cell_NonSpecific, boolean p_FirstN1_S1Transition := true) runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get(); var EUTRA38_DRBInfoList_Type v_ExistingDRBs; var template (value) RadioBearerList_Type v_DrbConfigList; var DRB_ToAddModList v_DRB_ToAddModList; …………….. f_EUTRA_Security_Set(v_SecurityParams); //Saving security parameters f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(p_CellId, v_NasInd, -, noRrcConnectionRelease); // @sic R5s201465 sic@ get all DRBs again in case more have been established ……….. }

After Change:

function fl_TAU_N1ToS1_CheckDataPath(EUTRA_CellId_Type p_CellId, RRC_TransactionIdentifier p_RRC_TI, charstring p_StepNo1 := "", charstring p_StepNo2 := "", template EUTRA_CellId_Type p_CellIdPreviouslyCampedOn := eutra_Cell_NonSpecific, boolean p_FirstN1_S1Transition := true) runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get(); var EUTRA38_DRBInfoList_Type v_ExistingDRBs; var template (value) RadioBearerList_Type v_DrbConfigList; var DRB_ToAddModList v_DRB_ToAddModList; ……………… f_EUTRA_Security_Set(v_SecurityParams); //Saving security parameters f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(p_CellId, v_NasInd, -, noRrcConnectionRelease, -, -, -, -,true); // @sic R5s201465 sic@ get all DRBs again in case more have been established ……………. }