3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220249

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2251 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for Rel-16 SON-MDT test cases 8.1.6.1.3.1 and 8.1.6.1.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It is not necessary for these test cases to be included in the f_GetTestcaseAttrib_EarlyContentionResolution() function because the function f_NR5GC_RRCReestablishment_SON_MDT() enforces the use of early contention during RRC reestablishment procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the test cases from the function f_GetTestcaseAttrib_EarlyContentionResolution() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | unnecessary test cases listed in test case attribute function |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.3.1.NR5GC, 8.1.6.1.3.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_GetTestcaseAttrib_EarlyContentionResolution() |
| --- | --- |
| Reason for change | It is not necessary for these test cases to be included in the f_GetTestcaseAttrib_EarlyContentionResolution() function because the function f_NR5GC_RRCReestablishment_SON_MDT() enforces the use of early contention during RRC reestablishment procedure. |
| Summary of change | Remove the test cases from the function f_GetTestcaseAttrib_EarlyContentionResolution() |
| TTCN module | TestcaseProperties.ttcn |
| MCC160 Comment |  |

Before Change

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_2_NR5GC") { return true; } case ("TC_11_3_6_NR5GC") { return true; } case ("TC_6_4_1_1_NR5GC") { return true; } case ("TC_6_4_2_2_NR5GC") { return true; } case ("TC_7_1_1_1_3_NR5GC") { return true; } case ("TC_7_1_1_1_5_NR5GC") { return true; } case ("TC_8_1_1_1_1_NR5GC") { return true; } case ("TC_8_1_1_2_1_NR5GC") { return true; } case ("TC_8_1_1_2_3_NR5GC") { return true; } case ("TC_8_1_1_4_1_NR5GC") { return true; } case ("TC_8_1_1_4_2_NR5GC") { return true; } case ("TC_8_1_2_1_4_NR5GC") { return true; } case ("TC_8_1_3_1_23_NR5GC") { return true; } case ("TC_8_1_4_1_5_NR5GC") { return true; } case ("TC_8_1_4_1_6_NR5GC") { return true; } case ("TC_8_1_5_6_1_NR5GC") { return true; } case ("TC_8_1_5_8_1_NR5GC") { return true; } case ("TC_8_1_5_8_2_1_NR5GC") { return true; } case ("TC_8_1_5_8_2_2_NR5GC") { return true; } case ("TC_8_1_5_8_2_3_NR5GC") { return true; } case ("TC_8_1_6_1_2_7_NR5GC") { return true; } case ("TC_8_1_6_1_3_1_NR5GC") { return true; } case ("TC_8_1_6_1_3_2_NR5GC") { return true; } case ("TC_8_1_6_1_3_3_NR5GC") { return true; } case ("TC_8_1_6_1_3_5_NR5GC") { return true; } case ("TC_8_1_6_1_4_1_NR5GC") { return true; } case ("TC_8_1_6_1_4_2_NR5GC") { return true; } case ("TC_8_1_6_1_4_3_NR5GC") { return true; } case ("TC_8_1_6_1_4_4_NR5GC") { return true; } case ("TC_8_1_6_1_4_5_NR5GC") { return true; } } return false; }

After Change

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_2_NR5GC") { return true; } case ("TC_11_3_6_NR5GC") { return true; } case ("TC_6_4_1_1_NR5GC") { return true; } case ("TC_6_4_2_2_NR5GC") { return true; } case ("TC_7_1_1_1_3_NR5GC") { return true; } case ("TC_7_1_1_1_5_NR5GC") { return true; } case ("TC_8_1_1_1_1_NR5GC") { return true; } case ("TC_8_1_1_2_1_NR5GC") { return true; } case ("TC_8_1_1_2_3_NR5GC") { return true; } case ("TC_8_1_1_4_1_NR5GC") { return true; } case ("TC_8_1_1_4_2_NR5GC") { return true; } case ("TC_8_1_2_1_4_NR5GC") { return true; } case ("TC_8_1_3_1_23_NR5GC") { return true; } case ("TC_8_1_4_1_5_NR5GC") { return true; } case ("TC_8_1_4_1_6_NR5GC") { return true; } case ("TC_8_1_5_6_1_NR5GC") { return true; } case ("TC_8_1_5_8_1_NR5GC") { return true; } case ("TC_8_1_5_8_2_1_NR5GC") { return true; } case ("TC_8_1_5_8_2_2_NR5GC") { return true; } case ("TC_8_1_5_8_2_3_NR5GC") { return true; } case ("TC_8_1_6_1_2_7_NR5GC") { return true; } /* REMOVED case ("TC_8_1_6_1_3_1_NR5GC") { return true; } case ("TC_8_1_6_1_3_2_NR5GC") { return true; } case ("TC_8_1_6_1_3_3_NR5GC") { return true; } */ case ("TC_8_1_6_1_3_5_NR5GC") { return true; } case ("TC_8_1_6_1_4_1_NR5GC") { return true; } case ("TC_8_1_6_1_4_2_NR5GC") { return true; } case ("TC_8_1_6_1_4_3_NR5GC") { return true; } case ("TC_8_1_6_1_4_4_NR5GC") { return true; } case ("TC_8_1_6_1_4_5_NR5GC") { return true; } } return false; }