3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220262

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2266 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.4.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of the function fl_TC_11_4_4_NR5GC_TestBody, after the PDU Session is released at Step 12, the PDN also should be stopped. This is not implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced a new variable v_PDN_Index to get the Pdn Index associated with the PDU Session that is released at Step 12, and then called function f_IP_Handling_StopPDN for this Pdn Index. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	4

1	Corrections required	4

1.1	Correction to function f_NR_UE_DeRegisterOnSwitchOff	4

# Corrections required

## Correction to function fl_TC_11_4_4_NR5GC_TestBody

| Function name | fl_TC_11_4_4_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation of the function fl_TC_11_4_4_NR5GC_TestBody, after the PDU Session is released at Step 12, the PDN also should be stopped. This is not implemented. |
| Summary of change | Introduced a new variable v_PDN_Index to get the Pdn Index associated with the PDU Session that is released at Step 12, and then called function f_IP_Handling_StopPDN for this Pdn Index. |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_11_4_4_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SRB_COMMON_IND v_ReceivedAsp; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var O1_Type v_PDUSessionIdToRelease; var PDUSessionInfoList_Type v_PDUSessionInfo; var integer i; var integer v_NoOfSessions; var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; // @siclog "Step 1" siclog@ // The SS configures: // - NR Cell 11 as "Serving cell" // - NR Cell 1 as "Non-Suitable "Off" cell". f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_NR_SetCellPower (nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // @siclog "Steps 2 - 4" siclog@ // The UE transmits an RRCSetupRequest message. // SS transmit an RRCSetup message. // The UE transmits an RRCSetupComplete message to confirm the successful completion of the connection establishment and // a REGISTRATION REQUEST message indicating "mobility registration updating" is sent to update the registration of the actual tracking area. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell11, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 5" siclog@ // SS sends a REGISTRATION REJECT message containing 5GMM cause value = #22 (Congestion). SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_NoSecurityProtection, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_Congestion), cs_GPRS_Timer2_3_IEI('5F'O, '001'B, '00011'B))))); //@siclog "Step 6" siclog@ // The SS transmits an RRCConnectionRelease message. f_NR_RRCRelease(nr_Cell11); //@siclog "Step 7" siclog@ //Make the UE attempt an IMS emergency call dialling an emergency number e.g. 112 or 911). (NOTE 1) f_UT_RequestIMSEmergencyCall(UT, px_EmergencyCallNumber); //@siclog "Step 8" siclog@ //@sic R5-213508 sic@ //Check: Does the UE perform steps 1-3 from the Registration procedure for mobility registration update as specified in TS 38.508-1 [4] subclause 4.9.5 v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell11, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { //@sic R5-213508 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@sic R5-213508 sic@ //@siclog "Step 8AA - 8AB" siclog@ //The SS transmits a SecurityModeCommand message. //The UE transmits a SecurityModeComplete message. f_NR_RRC_ActivateSecurity (nr_Cell11, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell11, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease); // @siclog "Step 8A - 9D" siclog@ //Check: Does the UE transmit an UL NAS TRANSPORT message with 'Request type' set to 'initial emergency request', and, a PDU SESSION ESTABLISHMENT REQUEST for establishing an emergency PDU? // Steps 9-13 from Generic Test Procedure for IMS Emergency call establishment with IMS Emergency registration as specified in TS 38.508-1 [4], subclause 4.9.11, including the parallel behaviour specified in table 4.9.11.2.2-2 are performed. //f_NR5GC_EmergencyCallWithRegistration(nr_Cell11); f_NR5GC_EmergencyCallPDUSession(nr_Cell11, tsc_NR_RbId_SRB1); //@sic R5s210515 sic@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); // Emergency call now active f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8A"); // @siclog "Step 10 - 11" siclog@ // Make the UE release the emergency call. (NOTE 1) f_NR5GC_EmergencyCallRelease(nr_Cell11); // @sic R%s211524 sic@ v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); v_NoOfSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); // @siclog "Step 12" siclog@ for (i := 0; i<v_NoOfSessions; i:=i+1) { // @sic R5s210515 R5-216200 sic@ v_PDUSessionIdToRelease := v_PDUSessionInfo[i].SessionId; v_MsgToSend := f_Get_NG_PDUSessionReleaseCommand(v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00011010'B), //5GSM cause Insufficient resources cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00101'B)); //T3396 = 5 minutes // p_NGSM_CongestionReattemptInd, SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB2, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend)));//@sic R5s210808sic@ SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell11, tsc_NR_RbId_SRB2, //@sic R5s210808sic@ cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToRelease)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Complete failed"); } } // @siclog "Step 13" siclog@ // Void // @siclog "Step 14" siclog@ // The SS releases the RRC connection. f_NR_RRCRelease(nr_Cell11); //@siclog "Step 15" siclog@ //Make the UE attempt an IMS emergency call dialling an emergency number e.g. 112 or 911). (NOTE 1) // Prose CR RRC Release fl_EmergencyCallAndRelease(nr_Cell11, px_EmergencyCallNumber, "Step 16"); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16 "); }

After Change:

function fl_TC_11_4_4_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SRB_COMMON_IND v_ReceivedAsp; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var O1_Type v_PDUSessionIdToRelease; var PDUSessionInfoList_Type v_PDUSessionInfo; var integer i; var integer v_NoOfSessions; var template (omit) PDN_Index_Type v_PDN_Index := omit; //WA#11_4_4 var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; // @siclog "Step 1" siclog@ // The SS configures: // - NR Cell 11 as "Serving cell" // - NR Cell 1 as "Non-Suitable "Off" cell". f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_NR_SetCellPower (nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // @siclog "Steps 2 - 4" siclog@ // The UE transmits an RRCSetupRequest message. // SS transmit an RRCSetup message. // The UE transmits an RRCSetupComplete message to confirm the successful completion of the connection establishment and // a REGISTRATION REQUEST message indicating "mobility registration updating" is sent to update the registration of the actual tracking area. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell11, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 5" siclog@ // SS sends a REGISTRATION REJECT message containing 5GMM cause value = #22 (Congestion). SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_NoSecurityProtection, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_Congestion), cs_GPRS_Timer2_3_IEI('5F'O, '001'B, '00011'B))))); //@siclog "Step 6" siclog@ // The SS transmits an RRCConnectionRelease message. f_NR_RRCRelease(nr_Cell11); //@siclog "Step 7" siclog@ //Make the UE attempt an IMS emergency call dialling an emergency number e.g. 112 or 911). (NOTE 1) f_UT_RequestIMSEmergencyCall(UT, px_EmergencyCallNumber); //@siclog "Step 8" siclog@ //@sic R5-213508 sic@ //Check: Does the UE perform steps 1-3 from the Registration procedure for mobility registration update as specified in TS 38.508-1 [4] subclause 4.9.5 v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell11, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { //@sic R5-213508 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@sic R5-213508 sic@ //@siclog "Step 8AA - 8AB" siclog@ //The SS transmits a SecurityModeCommand message. //The UE transmits a SecurityModeComplete message. f_NR_RRC_ActivateSecurity (nr_Cell11, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell11, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease); // @siclog "Step 8A - 9D" siclog@ //Check: Does the UE transmit an UL NAS TRANSPORT message with 'Request type' set to 'initial emergency request', and, a PDU SESSION ESTABLISHMENT REQUEST for establishing an emergency PDU? // Steps 9-13 from Generic Test Procedure for IMS Emergency call establishment with IMS Emergency registration as specified in TS 38.508-1 [4], subclause 4.9.11, including the parallel behaviour specified in table 4.9.11.2.2-2 are performed. //f_NR5GC_EmergencyCallWithRegistration(nr_Cell11); f_NR5GC_EmergencyCallPDUSession(nr_Cell11, tsc_NR_RbId_SRB1); //@sic R5s210515 sic@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); // Emergency call now active f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8A"); // @siclog "Step 10 - 11" siclog@ // Make the UE release the emergency call. (NOTE 1) f_NR5GC_EmergencyCallRelease(nr_Cell11); // @sic R%s211524 sic@ v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); v_NoOfSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); // @siclog "Step 12" siclog@ for (i := 0; i<v_NoOfSessions; i:=i+1) { // @sic R5s210515 R5-216200 sic@ v_PDUSessionIdToRelease := v_PDUSessionInfo[i].SessionId; v_MsgToSend := f_Get_NG_PDUSessionReleaseCommand(v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00011010'B), //5GSM cause Insufficient resources cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00101'B)); //T3396 = 5 minutes // p_NGSM_CongestionReattemptInd, SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB2, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend)));//@sic R5s210808sic@ SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell11, tsc_NR_RbId_SRB2, //@sic R5s210808sic@ cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; v_PDN_Index := f_Get_PDN_IndexForPDUSessionId (v_PDUSessionInfo, v_PDUSessionIdToRelease); //WA#11_4_4 if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToRelease)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Complete failed"); } } //WA#11_4_4 : Start if (isvalue(v_PDN_Index)) { // @sic R5s211524 sic@ f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(valueof(v_PDN_Index))); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Cannot find PDN Index of PDU Session being released"); } //WA#11_4_4 : Stop // @siclog "Step 13" siclog@ // Void // @siclog "Step 14" siclog@ // The SS releases the RRC connection. f_NR_RRCRelease(nr_Cell11); //@siclog "Step 15" siclog@ //Make the UE attempt an IMS emergency call dialling an emergency number e.g. 112 or 911). (NOTE 1) // Prose CR RRC Release fl_EmergencyCallAndRelease(nr_Cell11, px_EmergencyCallNumber, "Step 16"); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16 "); }