3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220266

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0766 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for IMS 5GS video call test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently during IMS video call setup procedure, two QoS rules are added in the PDU SESSION MODIFICATION COMMAND but the packet filter lists associated with these are both created using the same template cs_PacketFilterList_MediaUDP and so both refer to the RTP/RTCP ports set up for the voice stream (tsc_IMS_MediaPort_M1). There is no active packet filter for the RTP / RTCP ports used by the video stream. Due to this the UE can set the video stream to “port=0” / “a=inactive” in the next SDP content. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Create new template and function to build packet filter for video stream, using the consant tsc_IMS_MediaPort_M2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail IMS video calling test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | e.g. 8.27.NR5GC, 8.29.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	3

2.3	Change 3	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | New function fl_BuildRemotePortRangeVideoCall() |
| --- | --- |
| Reason for change | Currently during IMS video call setup procedure, two QoS rules are added in the PDU SESSION MODIFICATION COMMAND but the packet filter lists associatrd with these are the same and both referring to the RTP/RTCP ports set up for the voice stream. There is noa ctive packet filter for the RTP / RTCP ports used by the video stream. Due to this the UE can set the video stream to “port=0” / “a=inactive” in the next SDP content. |
| Summary of change | Created new function fl_BuildRemotePortRangeVideoCall to build a remote port range packet filter for video stream, using the consant tsc_IMS_MediaPort_M2. |
| TTCN module | NG_NAS_Common.ttcn |
| MCC160 Comment |  |

New function

function fl_BuildRemotePortRangeVideoCall () return octetstring { var octetstring v_PacketFilter; v_PacketFilter := '51'O; // id = remote port range v_PacketFilter := v_PacketFilter & int2oct(tsc_IMS_MediaPort_M2, 2) & int2oct(tsc_IMS_MediaPort_M2+1, 2); return v_PacketFilter; }

## Change 2

| Function name | New template cs_PacketFilterList_MediaUDP_VideoCall |
| --- | --- |
| Reason for change | Currently during IMS video call setup procedure, two QoS rules are added in the PDU SESSION MODIFICATION COMMAND but the packet filter lists associatrd with these are the same and both referring to the RTP/RTCP ports set up for the voice stream. There is noa ctive packet filter for the RTP / RTCP ports used by the video stream. Due to this the UE can set the video stream to “port=0” / “a=inactive” in the next SDP content. |
| Summary of change | New template for packet filter list for video call is created, utilising new function fl_BuildRemotePortRangeVideoCall() (see change 1) |
| TTCN module | NG_NAS_Templates.ttcn |
| MCC160 Comment |  |

New template

template (value) NG_PacketFilterList cs_PacketFilterList_MediaUDP_VideoCall (B4_Type p_Id) := { /* @status APPROVED (IMS, NR5GC, NR5GC_IRAT, POS) */ new := { cs_NG_PacketFilter_Type2 ('11'B, // bi-directional p_Id, fl_BuildRemotePortRangeVideoCall() & fl_BuildProtocolIdNextHeaderUDP()) // Protocol id / Next Header UDP } };

## Change 3

| Function name | f_BuildDefaultQoSRules() |
| --- | --- |
| Reason for change | Currently during IMS video call setup procedure, two QoS rules are added in the PDU SESSION MODIFICATION COMMAND but the packet filter lists associatrd with these are the same and both referring to the RTP/RTCP ports set up for the voice stream. There is noa ctive packet filter for the RTP / RTCP ports used by the video stream. Due to this the UE can set the video stream to “port=0” / “a=inactive” in the next SDP content. |
| Summary of change | New template cs_PacketFilterList_MediaUDP_VideoCall (see change 2) is used in place of existing cs_PacketFilterList_MediaUDP for defining QoS rule for video. |
| TTCN module | NG_NAS_Templates.ttcn |
| MCC160 Comment |  |

Before Change

function f_BuildDefaultQoSRules (charstring p_RuleNum, template (omit) IEI8_Type p_IEI := omit) return template (value) QoS_Rules { var template (value) QoS_Rules v_QoS_Rules; var template (value) QoS_Rule v_VoiceQoS := cs_QoS_Rule('03'O, int2bit(7, 6), '01'O, -, cs_PacketFilterList_MediaUDP('0110'B), '0'B); // @sic R5-210627 sic@; if (p_RuleNum == "7_Voice") { v_QoS_Rules := cs_QoS_Rules ({v_VoiceQoS}, p_IEI) // @sic R5-213440 sic@ } else if (p_RuleNum == "7_VoiceVideo") { v_QoS_Rules := cs_QoS_Rules ({v_VoiceQoS, cs_QoS_Rule('04'O, int2bit(8, 6), '02'O, -, cs_PacketFilterList_MediaUDP('0111'B), '0'B)}, p_IEI) // @sic R5-210627, R5-216161 sic@ } else { v_QoS_Rules := cs_QoS_Rules ({f_BuildDefaultQoSRule(p_RuleNum)}, p_IEI); } return v_QoS_Rules; }

After Change

function f_BuildDefaultQoSRules (charstring p_RuleNum, template (omit) IEI8_Type p_IEI := omit) return template (value) QoS_Rules { var template (value) QoS_Rules v_QoS_Rules; var template (value) QoS_Rule v_VoiceQoS := cs_QoS_Rule('03'O, int2bit(7, 6), '01'O, -, cs_PacketFilterList_MediaUDP('0110'B), '0'B); // @sic R5-210627 sic@; if (p_RuleNum == "7_Voice") { v_QoS_Rules := cs_QoS_Rules ({v_VoiceQoS}, p_IEI) // @sic R5-213440 sic@ } else if (p_RuleNum == "7_VoiceVideo") { v_QoS_Rules := cs_QoS_Rules ({v_VoiceQoS, cs_QoS_Rule('04'O, int2bit(8, 6), '02'O, -, cs_PacketFilterList_MediaUDP_VideoCall('0111'B), '0'B)}, p_IEI) // @sic R5-210627, R5-216161 sic@ } else { v_QoS_Rules := cs_QoS_Rules ({f_BuildDefaultQoSRule(p_RuleNum)}, p_IEI); } return v_QoS_Rules; }