3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220273

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4663 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE 11.2.6 test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2022-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current test case describes an establishment of an IMS MO speech call at step 20A-20I. Current implementation of the test case expects two IMS MO speech calls. This can cause issues as test case is still waiting an INVITE (MO) during Postamble procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed for statement in function f_TC_11_2_6_IMS1 to only expect an IMS MO speech call during complete test case execution. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case may end with a runttime error. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	1

1	Corrections required	1

1.1	Correction to function f_TC_11_2_6_IMS1	1

# Corrections required

## Correction to function f_TC_11_2_6_IMS1

| Function name | f_TC_11_2_6_IMS1 |
| --- | --- |
| Reason for change | Current test case describes an establishment of an IMS MO speech call at step 20A-20I. Current implementation of the test case expects two IMS MO speech calls. This can cause issues as test case is still waiting an INVITE (MO) during Postamble procedures. |
| Summary of change | Removed for statement in function f_TC_11_2_6_IMS1 to only expect an IMS MO speech call during complete test fcase execution. |
| TTCN module | LTE\11\IMS_EmergencyCall_IMS.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_TC_11_2_6_IMS1() runs on IMS_PTC { // @sic R5s150177 sic@ @sic R5-150733 sic@ var template (present) SipUrl v_OtherCalleeUri := cr_SipUrl_Common((c_sipScheme, c_telScheme), ?); // @sic R5s150177 change 9, R5s150204: cr_SipUri_HostPort(?) sic@ @sic R5s150855 change 5 sic@ var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; /* @sic R5w140112: IMS_InviteRequestWithSdp_Type instead of IMS_DATA_REQ */ var MessageHeader v_MessageHeader_Response183; <<REMOVED CODE>> var integer v_MaxMOCalls := 2; //2 IMS MO calls to be initiated var integer i; <<REMOVED CODE>> f_IMS_PTC_Init(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(-, -, -, "600000"); // @sic R5-151769 change 1 sic@ f_IMS_IPCAN_SendCoOrdMsg(IPCAN); <<REMOVED CODE>> for (i:=0; i< v_MaxMOCalls; i:=i+1) { <<REMOVED CODE>> /* Table 11.2.6.x.x.x Parallel behaviour Steps 5-13 expected sequence defined in annex C.21 of TS 34.229-1 [35]. */ v_InviteRequestWithSdp := f_IMS_MOCallSetup_AnnexC21_Step2_A4(v_OtherCalleeUri); f_IMS_IPCAN_WaitForTrigger(IPCAN); v_MessageHeader_Response183 := f_IMS_MOCallSetup_AnnexC21_Step3_4(v_InviteRequestWithSdp); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); // @sic R5s140589 sic@ f_IMS_MOCallSetup_AnnexC21C25_Steps5_8(v_InviteRequestWithSdp, v_MessageHeader_Response183); f_IMS_MOCallSetup_AnnexC21C25_Step9_11(v_InviteRequestWithSdp); f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //IMS call to be released f_IMS_IPCAN_WaitForTrigger(IPCAN); f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite); //Inform EUTRA : IMS call released f_IMS_IPCAN_SendCoOrdMsg(IPCAN); <<REMOVED CODE>> } <<REMOVED CODE>> f_IMS_IpcanReleaseWithOptionalDeregistration(); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_TC_11_2_6_IMS1() runs on IMS_PTC { // @sic R5s150177 sic@ @sic R5-150733 sic@ var template (present) SipUrl v_OtherCalleeUri := cr_SipUrl_Common((c_sipScheme, c_telScheme), ?); // @sic R5s150177 change 9, R5s150204: cr_SipUri_HostPort(?) sic@ @sic R5s150855 change 5 sic@ var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; /* @sic R5w140112: IMS_InviteRequestWithSdp_Type instead of IMS_DATA_REQ */ var MessageHeader v_MessageHeader_Response183; f_IMS_PTC_Init(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(-, -, -, "600000"); // @sic R5-151769 change 1 sic@ f_IMS_IPCAN_SendCoOrdMsg(IPCAN); /* Table 11.2.6.x.x.x Parallel behaviour Steps 5-13 expected sequence defined in annex C.21 of TS 34.229-1 [35]. */ v_InviteRequestWithSdp := f_IMS_MOCallSetup_AnnexC21_Step2_A4(v_OtherCalleeUri); f_IMS_IPCAN_WaitForTrigger(IPCAN); v_MessageHeader_Response183 := f_IMS_MOCallSetup_AnnexC21_Step3_4(v_InviteRequestWithSdp); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); // @sic R5s140589 sic@ f_IMS_MOCallSetup_AnnexC21C25_Steps5_8(v_InviteRequestWithSdp, v_MessageHeader_Response183); f_IMS_MOCallSetup_AnnexC21C25_Step9_11(v_InviteRequestWithSdp); f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //IMS call to be released f_IMS_IPCAN_WaitForTrigger(IPCAN); f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite); //Inform EUTRA : IMS call released f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); } <<SKIPPED CODE>>