3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220344

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0071 | rev | - | Current version: | 14.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MCPTT ambient listening test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 36.579-2 Tables 6.2.14.3.3-2 and 6.2.16.3.3-2 specify MCPTT info to be according to TS 36.579-1 table 5.5.3.2.1-1 with condition PRIVATE-CALL and ambient-listening specific content, but currently in TTCN only the ambient-listening specific fields are set correctly but other fields remain uninitialised. The same issue exists for Tables 6.2.15.3.3-2 and 6.2.17.3.3-2 referring to TS 36.579-1 table 5.5.3.2.2-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New functions fl_MCPTT_Info_AmbientListening_RX and fl_MCPTT_Info_AmbientListening_TX to build up MCPTT-Info for ambient listening test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency of prose and TTCN; depending on the TTCN compiler runtime errors may occur. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.15.MCPTT, 6.2.16.MCPTT |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## fl_MCPTT_Info_AmbientListening_RX (new)

function fl_MCPTT_Info_AmbientListening_RX(charstring p_AmbientListeningType) return template (present) MCPTT_Info_Type { var template (present) MCPTT_Info_Type v_MCPTT_Info := cr_MCPTT_Info_Base; var charstring v_AmbientListeningTypeExt := f_MCX_McpttInfoExt_Element("ambient-listening-type", p_AmbientListeningType); v_MCPTT_Info.mcptt_Params.session_type := "ambient-listening"; v_MCPTT_Info.mcptt_Params.anyExt.elem_list := { *, v_AmbientListeningTypeExt, * }; return v_MCPTT_Info; }

## fl_MCPTT_Info_AmbientListening_TX (new)

function fl_MCPTT_Info_AmbientListening_TX(charstring p_AmbientListeningType) return template (value) MCPTT_Info_Type { var template (value) MCPTT_Info_Type v_MCPTT_Info := cs_MCPTT_Info_Base; var charstring v_AmbientListeningTypeExt := f_MCX_McpttInfoExt_Element("ambient-listening-type", p_AmbientListeningType); v_MCPTT_Info.mcptt_Params.session_type := "ambient-listening"; v_MCPTT_Info.mcptt_Params.anyExt.elem_list := { v_AmbientListeningTypeExt }; return v_MCPTT_Info; }

## fl_MCX_CallSetupMO_AmbientListening

function fl_MCX_CallSetupMO_AmbientListening(MCX_CommencementMode_Type p_CommencementMode) runs on MCX_IMS_PTC return IMS_InviteRequestWithSdp_Type { var MCX_InviteRequestWithSdpAndXml_Type v_InviteRequestWithSdpAndXml; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var INVITE_Request v_InviteRequest; var MCX_MediaControlCondition_Type v_FloorControlConditionSdpOffer; var MCX_MediaControlConditionSdpAnswer_Type v_FloorControlConditionSdpAnswer; var charstring v_AmbientListeningType; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (value) Contact v_Contact; var template (value) MessageHeader v_MessageHeader200OK; var template (value) SDP_Message v_SDP_Answer; var template (value) SDP_attribute v_DirectionAttributeRX; var template (value) SemicolonParam_List v_FeatureParamList; var template (present) MCX_SdpInfoRX_Type v_SdpInfoRX; var template (present) MCX_MIME_XmlBodies_Type v_XmlBodies; select (p_CommencementMode) { case (AMBIENT_LISTENING_REMOTE) { v_AmbientListeningType := "remote-init"; v_DirectionAttributeRX := cs_SDP_Attribute_recvonly; v_FloorControlConditionSdpOffer := floorControlWithoutImplictGrantRequest; v_FloorControlConditionSdpAnswer := implicitFloorRequestNoted; } case (AMBIENT_LISTENING_LOCAL) { v_AmbientListeningType := "local-init"; v_DirectionAttributeRX := cs_SDP_Attribute_sendonly; v_FloorControlConditionSdpOffer := floorControlWithImplictGrantRequest; v_FloorControlConditionSdpAnswer := implicitFloorRequestGranted; } } // SDP info v_SdpInfoRX := f_MCPTT_SdpInfo_RX(PRIVATE_CALL, SDP_OFFER, v_FloorControlConditionSdpOffer, v_DirectionAttributeRX); // XML Bodies v_XmlBodies := f_MCPTT_InviteMO_XmlBodies_Init(PRIVATE_CALL); v_XmlBodies.McpttInfo := fl_MCPTT_Info_AmbientListening_RX(v_AmbientListeningType); // Receive INVITE v_InviteRequestWithSdpAndXml := f_MCPTT_InviteMO_ReceiveINVITE(PRIVATE_CALL, -, p_CommencementMode, -, v_SdpInfoRX, v_XmlBodies); v_InviteRequestWithSdp := v_InviteRequestWithSdpAndXml.InviteRequestWithSdp; v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_InviteRequestWithSdp.RoutingInfo); v_InviteRequest := v_InviteRequestWithSdp.Invite; // Send 100 Trying f_MCX_InviteMO_Send100Trying(v_InviteRequestWithSdp); // Send 200 OK v_Contact := f_MCX_CallSetupMO_ContactSS(MCPTT); v_MessageHeader200OK := f_MCX_CommonResponse_200_MessageHeaderTX(v_InviteRequest.msgHeader, v_Contact, cs_ContentTypeSDP); v_FeatureParamList := { cs_GenericParam("g.3gpp.mcptt.ambient-listening-call-release") }; v_MessageHeader200OK.featureCaps := cs_FeatureCaps(v_FeatureParamList); v_SDP_Answer := f_MCPTT_SdpAnswer_TX(v_InviteRequestWithSdp.SdpOffer, v_FloorControlConditionSdpAnswer); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine200, v_MessageHeader200OK, cs_MessageBody_SDP(v_SDP_Answer)))); // Receive ACK f_MCX_InviteMO_ReceiveACK(-, v_InviteRequest); return v_InviteRequestWithSdp; }

## fl_MCX_CallSetupMT_AmbientListening

function fl_MCX_CallSetupMT_AmbientListening(MCX_CommencementMode_Type p_CommencementMode) runs on MCX_IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var charstring v_AmbientListeningType; var template (value) MCX_MIME_XmlBodies_Type v_XmlBodiesTX; var template (value) SDP_Message v_SdpOfferTX; var template (value) SDP_attribute v_DirectionAttributeTX; var template (value) SDP_attribute v_DirectionAttributeRX; var template (present) MCX_SdpInfoRX_Type v_SdpInfoRX; var template (present) MCPTT_Info_Type v_McpttInfo; var boolean v_AllowTrying := true; select (p_CommencementMode) { case (AMBIENT_LISTENING_REMOTE) { v_AmbientListeningType := "remote-init"; v_DirectionAttributeTX := cs_SDP_Attribute_recvonly; /* @sic R5s211377 - additional change: exchange of RX/TX sic@ */ v_DirectionAttributeRX := cs_SDP_Attribute_sendonly; /* @sic R5s211377 - additional change: exchange of RX/TX sic@ */ } case (AMBIENT_LISTENING_LOCAL) { v_AmbientListeningType := "local-init"; v_DirectionAttributeTX := cs_SDP_Attribute_sendonly; /* @sic R5s211377 - additional change: exchange of RX/TX sic@ */ v_DirectionAttributeRX := cs_SDP_Attribute_recvonly; /* @sic R5s211377 - additional change: exchange of RX/TX sic@ */ } } // SDP info v_SdpOfferTX := f_MCPTT_SdpOffer_TX(PRIVATE_CALL); v_SdpOfferTX.media_list[0].attributes := f_SDP_Attributes_Add_TX(v_SdpOfferTX.media_list[0].attributes, v_DirectionAttributeTX); // XML Bodies v_XmlBodiesTX := f_MCPTT_InviteMT_XmlBodies_Init(PRIVATE_CALL); v_XmlBodiesTX.McpttInfo := fl_MCPTT_Info_AmbientListening_TX(v_AmbientListeningType); // Send INVITE v_InviteRequestWithSdp := f_MCX_InviteMT_SendINVITE(MCPTT, PRIVATE_CALL, -, p_CommencementMode, -, -, v_SdpOfferTX, v_XmlBodiesTX); // receive 100 Trying, 200 OK, send ACK v_SdpInfoRX := f_MCPTT_SdpInfo_RX(PRIVATE_CALL, SDP_ANSWER, -, v_DirectionAttributeRX); v_McpttInfo := f_MCX_CallControl_MCPTTInfo_RX(PRIVATE_CALL, INVITE_RSP); f_MCPTT_InviteMT_Receive200OK_SendACK(PRIVATE_CALL, v_InviteRequestWithSdp, v_AllowTrying, -, v_SdpInfoRX, v_McpttInfo); }