3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220400

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2181 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC R16 SON-MDT test case 8.1.6.1.4.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC RRC test case 8.1.6.1.4.2 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC RRC test case 8.1.6.1.4.2 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.4.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 R5-216804 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1 Change 1	3

2.2 Change 2	4

3.	Branches executed	8

4.	Execution Log Files	8

4.1	Qualcomm SM8450+SDX65	8

5.	References	8

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.6.1.4.2 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

Narendra Kalahasti

narendra.kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	8.1.6.1.4.2.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk37

System Simulator used:	Keysight S8704A Protocol Conformance Toolset and Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm SM8450+SDX65

Verification Status:	PASS

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_8_1_6_1_4_2_TestBody |
| --- | --- |
| Reason for change | - Due to the proposed change of R5-216804, the test case only requires one cell. - SRB2 is not established during the test case body (see change 2) |
| Summary of change | - Remove references to NR cell 2 and update system information combination. - Postamble on SRB2 |
| TTCN module | RRC_MDT_IntraNR_CEF_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_6_1_4_2_NR5GC() runs on NR5GC_PTC

{ //Connection Establishment Failure / Logging and reporting / RRC Resume

f_NR5GC_Init(NR_2);

//Create and configure cell

f_NR_CellConfig_Def(nr_Cell1);

f_NR_CellConfig_Def(nr_Cell2);

f_NR5GC_Preamble(nr_Cell1, STATE_INACTIVE_2A);

f_NR_TestBody_Set(true);

fl_TC_8_1_6_1_4_2_TestBody();

f_NR_TestBody_Set(false);

f_NR_Postamble(nr_Cell2, STATE_CONNECTED_3A);

}

After Change:

function f_TC_8_1_6_1_4_2_NR5GC() runs on NR5GC_PTC

{ //Connection Establishment Failure / Logging and reporting / RRC Resume

f_NR5GC_Init(NR_1);

//Create and configure cell

f_NR_CellConfig_Def(nr_Cell1);

//REMOVED f_NR_CellConfig_Def(nr_Cell2);

f_NR5GC_Preamble(nr_Cell1, STATE_INACTIVE_2A);

f_NR_TestBody_Set(true);

fl_TC_8_1_6_1_4_2_TestBody();

f_NR_TestBody_Set(false);

f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A, tsc_NR_RbId_SRB1);

}

## 2.2 Change 2

| Function name | fl_TC_8_1_6_1_4_2_TestBody |
| --- | --- |
| Reason for change | - After T319 Expiry, the UE enters IDLE and forgets about previous RRC security context. SS should also be locally released, otherwise, Security Keys saved during Inactive state would be used. (maybe add to prose also?) - delay changed from 2.0 to 1.0 to reflect T319 expiry in the UE - Steps on NR cell 2 no longer needed - AS Security needs to be activated so UE can send UE Information Response (TS 38.331, clause 5.7.10.3) |
| Summary of change | - Used f_NR_RRCRelease_Local () function to set the SS to IDLE. This resets RRC security keys also to NULL. - Changed f_Delay(2.0) -> f_Delay(1.0) - Remove steps 4-12 and add new steps 3A-3E - Used f_NR_RRC_ActivateSecurity() to activate AS Security prior to completing the mobility registration Note : The associated prose CR on 38.523-1 R5-216804 is raised at RAN5#93 |
| TTCN module | RRC_MDT_IntraNR_CEF_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_6_1_4_2_TestBody() runs on NR5GC_PTC

{

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var CellIdentity v_CellIdentity_Cell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1);

var PLMN_Identity v_PLMN_Identity_Cell1 := f_NR_CellInfo_GetPLMN (nr_Cell1);

var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1;

v_CellPowerList_AtT1 := {

cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),

cs_NR_CellPower(nr_Cell2, -94, -91)

};

//@siclog "Step 1" siclog@

//The SS transmits a Paging message on NR Cell 1.

f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE);

//@siclog "Step 2" siclog@

//The UE transmits an RRCResumeRequest message.

f_NR_RRC_ResumeRequest_Def(nr_Cell1);

//@siclog "Step 3" siclog@

//The SS waits for 2sec to ensure that the UE detects T319 expiry.

f_Delay(2.0);

//@siclog "Step 4" siclog@

//The SS adjusts cell levels according to row T1 of table 8.1.6.1.4.2.3.2-1 for FR1 and table 8.1.6.1.4.2.3.2-2 for FR2.

f_NR_SetCellPowerList(v_CellPowerList_AtT1);

//@siclog "Step 5" siclog@

//The SS transmits a Paging message on NR Cell 2.

f_NR_Paging_Def(nr_Cell2, RRC_IDLE);

//@siclog "Step 6" siclog@

//The UE transmits an RRCSetupRequest message.

f_NR_RRC_SetupRequest_Def(nr_Cell2);

//@siclog "Step 7" siclog@

//The SS transmits an RRCSetup message.

f_NR_RRCSetup_Def(nr_Cell2);

//@siclog "Step 8" siclog@

//Check: Does the UE transmit an RRCSetupComplete message including connEstFailInfoAvailable IE set it to true? UE initiates the session management procedure by including the SERVICE REQUEST message.

v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell2,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, -, -, -,

cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(-, -, -, true_))));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8");

//@siclog "Steps 9-12" siclog@

//Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3

f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell2, v_ReceivedMsg);

//@siclog "Step 13" siclog@

//The SS sends a UEInformationRequest message to get connEstFailReportReq.

SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell2,

-,

cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, -, true_)));

//@siclog "Step 14" siclog@

//Check: Does the UE send a UEInformationResponse message with connEstFailReport?

SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell2,

cr_UEInformationResponse_r16_Common(tsc_NR_RRC_TI_Def,

-, -, -,

cr_ConnEstFailReport_r16(cr_MeasResultFailedCell_r16(cr_CGI_Info_Logging_r16(v_PLMN_Identity_Cell1, v_CellIdentity_Cell1)),

omit))));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14");

}

After Change:

function fl_TC_8_1_6_1_4_2_TestBody() runs on NR5GC_PTC

{

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var CellIdentity v_CellIdentity_Cell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1);

var PLMN_Identity v_PLMN_Identity_Cell1 := f_NR_CellInfo_GetPLMN (nr_Cell1);

var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

/* REMOVED v_CellPowerList_AtT1 := {

cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),

cs_NR_CellPower(nr_Cell2, -94, -91)

};*/

f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetCurrentTiming(nr_Cell1));

//@siclog "Step 1" siclog@

//The SS transmits a Paging message on NR Cell 1.

f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE);

//@siclog "Step 2" siclog@

//The UE transmits an RRCResumeRequest message.

f_NR_RRC_ResumeRequest_Def(nr_Cell1);

//@siclog "Step 3" siclog@

//The SS waits for 2sec to ensure that the UE detects T319 expiry.

f_Delay(f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(1.0)));

//steps 3A-3E as per R5-21826

//@siclog "Step 3A" siclog@

//The UE transmits an RRCSetupRequest message.

f_NR_RRC_SetupRequest_Def(nr_Cell1);

//@siclog "Step 3B" siclog@

//The SS transmits an RRCSetup message.

f_NR_RRCSetup_Def(nr_Cell1);

//@siclog "Step 3C" siclog@

//Check: Does the UE transmit an RRCSetupComplete message including connEstFailInfoAvailable IE set it to true? UE initiates the session management procedure by including the SERVICE REQUEST message.

v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, -, -, -,

cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(-, -, -, true_))));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3C");

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

//@siclog "Steps 3D" siclog@

//Steps 4 to 5 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.9.5.2.2-1

f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease);

f_NR_RRC_ActivateSecurity (nr_Cell1, v_ReceivedMsg.SecurityProtection.NasCount); // Use NasCount returned from Service Req

/* REMOVED – steps 4-12

//@siclog "Step 4" siclog@

//The SS adjusts cell levels according to row T1 of table 8.1.6.1.4.2.3.2-1 for FR1 and table 8.1.6.1.4.2.3.2-2 for FR2.

f_NR_SetCellPowerList(v_CellPowerList_AtT1);

f_Delay(0.7);

//@siclog "Step 5" siclog@

//The SS transmits a Paging message on NR Cell 2.

f_NR_Paging_Def(nr_Cell2, RRC_IDLE);

//@siclog "Step 6" siclog@

//The UE transmits an RRCSetupRequest message.

f_NR_RRC_SetupRequest_Def(nr_Cell2);

//@siclog "Step 7" siclog@

//The SS transmits an RRCSetup message.

f_NR_RRCSetup_Def(nr_Cell2);

//@siclog "Step 8" siclog@

//Check: Does the UE transmit an RRCSetupComplete message including connEstFailInfoAvailable IE set it to true? UE initiates the session management procedure by including the SERVICE REQUEST message.

v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell2,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, -, -, -,

cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(-, -, -, true_))));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8");

//@siclog "Steps 9-12" siclog@

//Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3

f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell2, v_ReceivedMsg);

*/ //

//@siclog "Step 13" siclog@

//The SS sends a UEInformationRequest message to get connEstFailReportReq.

SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1,

-,

cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, -, true_)));

//@siclog "Step 14" siclog@

//Check: Does the UE send a UEInformationResponse message with connEstFailReport?

SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1,

cr_UEInformationResponse_r16_Common(tsc_NR_RRC_TI_Def,

-, -, -,

cr_ConnEstFailReport_r16(cr_MeasResultFailedCell_r16(cr_CGI_Info_Logging_r16(v_PLMN_Identity_Cell1, v_CellIdentity_Cell1)),

omit))));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14");

}

# Branches executed

The test case was executed on NR band n78 uing NR ciphering nea1 and integrity algorithm nia1

# Execution Log Files

## Qualcomm SM8450+SDX65

The Qualcomm SM8450+SDX65 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_8_1_6_1_4_2_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_8_1_6_1_4_2_PIXIT.xml

The Qualcomm SM8450+SDX65 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu/TC_8_1_6_1_4_2_LOG.xml

- PICS/PIXIT parameter file:

Anritsu/TC_8_1_6_1_4_4_PICS.txt

# References

| [1] | R5s211599 : NR5GC FR1 : Supporting information for addition of NR5GC R16 SON-MDT test case 8.1.6.1.4.2 |
| --- | --- |