3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220415

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2417 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GMM test case 9.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current implementation of f_UT_ReadAllowedNSSAI() will return the allowed NSSAI according to the 1st +CGNSSAIRDP line returned by the UE. This may cause the test case 9.1.4.1 to fail at step 68 if the UE reports allowed NSSAI for multiple PLMN IDs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Pass PLMN ID to f_UT_ReadAllowedNSSAI so the correct set of allowed NSSAI can be returned. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | function fl_TC_9_1_4_1_TestBody() runs on NR5GC_PTC |
| --- | --- |
| Reason for change | Current implementation of f_UT_ReadAllowedNSSAI() will return the allowed NSSAI according to the 1st +CGNSSAIRDP line returned by the UE. This may cause the test case 9.1.4.1 to fail at step 68 if the UE reports allowed NSSAI for multiple PLMN IDs. |
| Summary of change | Pass PLMN ID to f_UT_ReadAllowedNSSAI so the correct set of allowed NSSAI can be returned. |
| TTCN module | ConfigurationUpdate_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_9_1_4_1_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellG := f_NR5GC_MapNASCell(ngc_CellG, NG_CellsOnDifferentPLMN); /*Skipping code */ //@siclog "Step 68" siclog@ v_AT_Response := f_UT_ReadAllowedNSSAI(UT); if (match (v_AT_Response, f_Convert_ListOfNSSAI_ForATCommand(cs_NSSAI_Allowed_Def))) { // @sic R5s201141, R5-26256 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 68"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 68"); } //@siclog "Step 69 - 70" siclog@ }

After Change:

function fl_TC_9_1_4_1_TestBody() runs on NR5GC_PTC { …………………….. //@siclog "Step 68" siclog@ v_AT_Response := f_UT_ReadAllowedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN_CellG)); if (match (v_AT_Response, f_Convert_ListOfNSSAI_ForATCommand(cs_NSSAI_Allowed_Def))) { // @sic R5s201141, R5-26256 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 68"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 68"); } //@siclog "Step 69 - 70" siclog@ }

## Change 2

| Function name | f_UT_ReadAllowedNSSAI() |
| --- | --- |
| Reason for change | Current implementation of f_UT_ReadAllowedNSSAI() will return the allowed NSSAI according to the 1st +CGNSSAIRDP line returned by the UE. This may cause the test case 9.1.4.1 to fail at step 68 if the UE reports allowed NSSAI for multiple PLMN IDs. |
| Summary of change | Take PLMN id string as additional input parameter to return correct list of allowed NSSAI |
| TTCN module | UpperTesterFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_UT_ReadAllowedNSSAI(UT_PTC_MTC_PORT p_Port, template (omit) charstring p_PLMN_ID := omit, boolean p_Access3GPP := true) return charstring { var charstring v_ResultString := f_UT_ReadNSSAI_DynamicParams(p_Port, Default_Rejected_Configured_Allowed, p_PLMN_ID); var CharStringList_Type v_ListOfLines := f_StringSplitLines(v_ResultString, "<CR><LF>"); // a list charstrings separated by CR LF @sic R5s210431 sic@ var charstring v_Allowed := ""; var CharStringList_Type v_CharStringList; var integer i; v_ListOfLines := f_CharStringList_Remove(v_ListOfLines, ""); // remove empty lines from the list @sic R5s210431 sic@ v_CharStringList := f_StringSplit(v_ListOfLines[1], {","}); // @sic R5s210431 sic@ if (p_Access3GPP) { i := 4; } else { i := 6; } if (isvalue(v_CharStringList[i])) { // @sic R5s210431 sic@ v_Allowed := regexp(v_CharStringList[i], """(*)""", 0); } return v_Allowed; }

After Change:

function f_UT_ReadAllowedNSSAI(UT_PTC_MTC_PORT p_Port, template (omit) charstring p_PLMN_ID := omit, boolean p_Access3GPP := true) return charstring { var charstring v_ResultString := f_UT_ReadNSSAI_DynamicParams(p_Port, Default_Rejected_Configured_Allowed, p_PLMN_ID); var CharStringList_Type v_ListOfLines := f_StringSplitLines(v_ResultString, "<CR><LF>"); // a list charstrings separated by CR LF @sic R5s210431 sic@ var charstring v_Allowed := ""; var charstring v_Allowed1 := ""; var CharStringList_Type v_CharStringList; var integer i; var integer j:=1; v_ListOfLines := f_CharStringList_Remove(v_ListOfLines, ""); // remove empty lines from the list @sic R5s210431 sic@ while (isvalue(v_ListOfLines[j])) { v_CharStringList := f_StringSplit(v_ListOfLines[j], {","}); v_Allowed1 := regexp(v_CharStringList[0],"*C5GNSSAIRDP: ""(*)""*", 0); if(match (p_PLMN_ID,v_Allowed1) ) { break; } j:= j+1; } if (p_Access3GPP) { i := 4; } else { i := 6; } if (isvalue(v_CharStringList[i])) { // @sic R5s210431 sic@ v_Allowed := regexp(v_CharStringList[i], """(*)""", 0); } return v_Allowed; }