3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220416

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2418 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC test case 8.1.1.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Prose for step 1 states: The SS transmits one IP packet to the UE on the DRB associated with the default PDU session on NR Cell 1. (Note 2) … Note 2: Default PDU session type is set in the PICS pc_APN_Default_Configuration. Current TTCN implementation is using the DRB associated with 1st PDU session, not taking into account pc_APN_Default_Configuration |  |  |  |  |  |  |  |  |
| Summary of change: |  | Find DRB associated with default PDU session according to PICS setting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case is not conisstent with specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_8_1_1_2_3_TestBody() |
| --- | --- |
| Reason for change | Prose for step 1 states: The SS transmits one IP packet to the UE on the DRB associated with the default PDU session on NR Cell 1. (Note 2) … Note 2: Default PDU session type is set in the PICS pc_APN_Default_Configuration. Current TTCN implementation is using the DRB associated with 1st PDU session, not taking into account pc_APN_Default_Configuration |
| Summary of change | Find DRB associated with default PDU session according to PICS setting |
| TTCN module | RRC_ConnEst_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_1_2_3_TestBody() runs on NR5GC_PTC { var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var RejectWaitTime v_RejectWaitTime := 10; var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var integer v_DRBId; var template (value) QosFlow_Identification_Type v_QoS_Id; var float v_Wait := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(v_RejectWaitTime)); //@sic R5s191085, R5s210051 sic@ //@siclog "Step 1" siclog@ //The SS transmits one IP packet to the UE on the DRB associated with the default PDU session on Cell 1. //Note 2: Default PDU session type is set in the PICS pc_APN_Default_Configuration v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_QoS_Id := f_NR_GetQosFlowIdentification_ForDRBId(v_DRBId); DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QoS_Id, {v_IPData})); //@siclog "Step 2" siclog@ /* Skipping code */ }

After Change:

function fl_TC_8_1_1_2_3_TestBody() runs on NR5GC_PTC { var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var RejectWaitTime v_RejectWaitTime := 10; var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var integer v_DRBId; var template (value) QosFlow_Identification_Type v_QoS_Id; var float v_Wait := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(v_RejectWaitTime)); //@sic R5s191085, R5s210051 sic@ //@siclog "Step 1" siclog@ //The SS transmits one IP packet to the UE on the DRB associated with the default PDU session on Cell 1. //Note 2: Default PDU session type is set in the PICS pc_APN_Default_Configuration v_PDUType := f_GetDefaultAPN_DNNType_FromPICS(); select(v_PDUType) { case(None) { v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); } case (Internet_DNN) { v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); } case (IMS_DNN) { v_DRBId := f_NR_GetDefaultDRB_ForIMSPDUSession(); } } v_QoS_Id := f_NR_GetQosFlowIdentification_ForDRBId(v_DRBId); DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QoS_Id, {v_IPData})); //@siclog "Step 2" siclog@ /*Skipping Code */ }