3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220432

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0154 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_POS_NR_CheckCapabilities_NR_Ecid |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The IE nr-ECID-MeasSupported-r16 has a variable size that goes from 1 to 8 according to 3GPP TS 37.355 R16. Current TTCN implementation iterates through the values of the supported NR ECID measurement with a fixed length of 4. This causes failure in case the UE reports less than 4 methods. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Iterate through nr-ECID-MeasSupported-r16 based on its length instead of using a fixed value |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.1.NR5GC, 9.3.4.1.NR5GC, 9.3.4.2.NR5GC, 9.3.4.3.NR5GC, 9.3.4.4.NR5GC, 9.3.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1.1 Correction to f_POS_NR_CheckCapabilities_NR_Ecid	2

## 1.1 Correction to f_POS_NR_CheckCapabilities_NR_Ecid

| Function name | Correction to f_POS_NR_CheckCapabilities_NR_Ecid |
| --- | --- |
| Reason for change | Current TTCN implementation iterates through the values of the supported NR ECID measurement with a fixed length of 4. This causes failure in case the UE reports less than 4 methods. |
| Summary of change | Iterate through nr-ECID-MeasSupported-r16 based on its length instead of using a fixed value |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

function f_POS_NR_CheckCapabilities_NR_Ecid (NR_ECID_ProvideCapabilities_r16 p_NrEcidProvideCapabilities) runs on NR_BASE_PTC

{ //@sic POS Rel-16 R5-213638 sic@

var integer i;

var bitstring v_Bitstring := f_ConvertBoolToBit(pc_NR_ECID_SSRsrp) &

f_ConvertBoolToBit(pc_NR_ECID_SSRsrq) &

f_ConvertBoolToBit(pc_NR_ECID_CSIRsrp) &

f_ConvertBoolToBit(pc_NR_ECID_CSIRsrq);

for (i := 0; i < 3; i := i + 1) {

if (not (v_Bitstring[i] == p_NrEcidProvideCapabilities.nr_ECID_MeasSupported_r16[i])) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for NR ECID measurements");

}

}

if (pc_NR_ECID_periodicalReporting != ispresent(p_NrEcidProvideCapabilities.periodicalReporting_r16)) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for NR ECID periodicalReporting");

}

if (pc_NR_ECID_triggeredReporting != ispresent(p_NrEcidProvideCapabilities.triggeredReporting_r16)) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for NR ECID triggeredReporting");

}

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_POS_NR_CheckCapabilities_NR_Ecid (NR_ECID_ProvideCapabilities_r16 p_NrEcidProvideCapabilities) runs on NR_BASE_PTC

{ //@sic POS Rel-16 R5-213638 sic@

var integer i;

var bitstring v_Bitstring := f_ConvertBoolToBit(pc_NR_ECID_SSRsrp) &

f_ConvertBoolToBit(pc_NR_ECID_SSRsrq) &

f_ConvertBoolToBit(pc_NR_ECID_CSIRsrp) &

f_ConvertBoolToBit(pc_NR_ECID_CSIRsrq);

for (i := 0; i < lengthof(p_NrEcidProvideCapabilities.nr_ECID_MeasSupported_r16)-1; i := i + 1) {

if (not (v_Bitstring[i] == p_NrEcidProvideCapabilities.nr_ECID_MeasSupported_r16[i])) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for NR ECID measurements");

}

}

if (pc_NR_ECID_periodicalReporting != ispresent(p_NrEcidProvideCapabilities.periodicalReporting_r16)) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for NR ECID periodicalReporting");

}

if (pc_NR_ECID_triggeredReporting != ispresent(p_NrEcidProvideCapabilities.triggeredReporting_r16)) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for NR ECID triggeredReporting");

}

}

<<SKIPPED CODE>>