3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220483

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2459 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC test case 7.1.1.3.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As the DRB Identity 2 on the NR cell is configured as an AM DRB with statusReportRequired set to TRUE, putting MAC in TM mode at the beginning of the TC, there is no handling of the PDCP Status Report sent by the UE on the NR Cell in the TTCN as MAC is in TM mode. Hence, MAC cannot be put in test mode until the signalling to communicate the change in the PUSCH_AggegationFactor has been sent to the UE at Step 0A on the EUTRA cell |  |  |  |  |  |  |  |  |
| Summary of change: |  | MAC is first put in Normal mode to complete the signalling procedure at Step 0A and then put in test mode after receiving the trigger from EUTRA PTC indicating start of Test Body |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE shall fail the testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.9.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of ContentsA con

Table of ContentsA con	2

1	Corrections required	3

1.1	f_TC_7_1_1_3_9_ENDC_NR	3

# Corrections required

## f_TC_7_1_1_3_9_ENDC_NR

| Function name | f_TC_7_1_1_3_9_ENDC_NR |
| --- | --- |
| Reason for change | As the DRB Identity 2 on the NR cell is configured as an AM DRB with statusReportRequired set to TRUE, putting MAC in Test mode at the beginning of the TC, there is no handling of the PDCP Status Report in the TTCN sent by the UE on the NR Cell as MAC is in TM mode. Hence, MAC cannot be put in test mode until the signalling to communicate the change in the PUSCH_AggegationFactor has been sent to the UE at Step 0A on the EUTRA cell |
| Summary of change | MAC is first put in Normal mode to complete the signalling procedure at Step 0A and then put in test mode after receiving the trigger from EUTRA PTC indicating start of Test Body |
| TTCN module | MAC_ENDC_NR |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_3_9_ENDC_NR() runs on ENDC_NR_PTC { // Correct Handling of UL HARQ process / PUSCH Aggregation var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2)}; //Init Cell parameters f_ENDC_NR_Init(); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0, v_SS_Drb_ConfigList); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); fl_TC_7_1_1_3_9_ENDC_NR_TestBody(); f_NR_TestBody_Set(false); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

After Change:

function f_TC_7_1_1_3_9_ENDC_NR() runs on ENDC_NR_PTC { // Correct Handling of UL HARQ process / PUSCH Aggregation var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; // REMOVED := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2)}; //WA# //Init Cell parameters f_ENDC_NR_Init(); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode //WA# v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0, v_SS_Drb_ConfigList); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); //WA# v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_NR_TestBody_Set(true); fl_TC_7_1_1_3_9_ENDC_NR_TestBody(); f_NR_TestBody_Set(false); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }