3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220485

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0803 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Checking the absence of geolocation information in INVITE request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2022-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is no possibility to verify explicitly that geolocation information is not provided by the UE in INVITE request when such information is not available at the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Geolocation information was initialized as ‘omit’ in INVITE request. Depending upon geolocation information being available to the UE the expected parameters were adjusted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A non-conformant UE may pass the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.1.2, J.19.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Prose CR is associated. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1.	Correction to f_IMS_InviteRequest_MessageHeaderRX	2

2.	Correction to f_IMS_A_2_1_Invite_CommonChecks	6

3.	Correction to f_IMS_INVITE_ReceiveRequest_A_2_1_Optional	9

4.	Correction to f_IMS_SRVCC_ReInvite_AnnexC24_Step1	10

5.	Correction to f_IMS_SRVCC_ReInvite_AnnexC31_Step1	11

6.	Correction to f_ECall_InitialInvite_AnnexC47	12

7.	Correction to f_IMS_INVITE_ReceiveRequest	14

8.	Correction to f_IMS_MOCallSetup_ReceiveINVITE_Optional	14

## Correction to f_IMS_InviteRequest_MessageHeaderRX

| Function name | f_IMS_InviteRequest_MessageHeaderRX |
| --- | --- |
| Reason for change | Geolocation and GeolocationRouting not defaulted to omit in the template for Invite Request if condition A8 is false. |
| Summary of change | Geolocation and GeolocationRouting defaulted to omit in the template for Invite Request if condition A8 is false. |
| TTCN module | IMS_Procedures_CallControl4G5G |

function f_IMS_InviteRequest_MessageHeaderRX(template (present) SipUrl p_ExpectedCalleeUri, OptionTag_List p_OptionTagsForSupported := tsc_OptionTagList_100rel, INVITE_A_2_1_Context_Type p_Context := A_2_1_A4, boolean p_A3 := pc_MultimediaTelephonyService, /* @sic R5s130510 additional change 1: tsc_IMS_MTSI replaced by pc_MultimediaTelephonyService sic@ */ boolean p_A8 := false, boolean p_A11 := false) runs on IMS_PTC return template (present) MessageHeader { var boolean v_IsGiba := f_IMS_PTC_SecurityScheme_IsGiba(); // => A1/A2 var template (present) MessageHeader v_MessageHeader_Invite := cr_MessageHeader_Dummy; var template (present) RouteBody_List v_RouteBodyList; var IMS_ProtectedPorts_Type v_Protected; var template integer v_Port_us_Contact := *; var template integer v_Port_ps_Route := *; var template DisplayName v_DisplayName := *; var template (present) SipUrl v_SipUrlFrom := ?; /* per default any SIP URI is accepted in the From header in the first place; further checks may be applied in f_IMS_A_2_1_Invite_CommonChecks (depending on the conditions acc. to A.2.1 */ var template (present) charstring v_Pcscf; var charstring v_MediaType := c_sdpApplication; var template (present) CallId v_CallId; var template (present) integer v_CSeqValue := ?; var template (present) From v_From; var template (present) To v_To; var template AcceptBody_List v_AcceptBodyList := {}; var OptionTag_List v_OptionTag_List := p_OptionTagsForSupported; var boolean v_GRUUsInSIP := pc_IMS_GRUUsInSIP; var boolean v_A4 := (p_Context == A_2_1_A4); // INVITE creating a dialog @sic R5-166280 sic@ var boolean v_A5 := (p_Context == A_2_1_A5); // re-INVITE within a dialog @sic R5-166280 sic@ var boolean v_A6 := (p_Context == A_2_1_A6); // emergency session in case of no registration @sic R5-166280 sic@ var boolean v_A19 := (p_Context == A_2_1_A19); // emergency session in case of no registration (GIBA) @sic R5-174545 sic@ var boolean v_A26 := pc_IMS_Session_Timer; // UE supports Session Timer @sic R5-192321 sic@ if (pc_IMS_CS_PS_SRVCCMidCall or pc_IMS_CS_PS_SRVCCAlert) { // @sic R5-145746 sic@ v_OptionTag_List := v_OptionTag_List & tsc_OptionTagList_norefersub; } if (v_A4 and v_A26) { /* @sic R5-192321 sic@ NOTE: A4, A6, A7 and A19 are mutual exclusive; use of A20, A21 (eCall) is unclear */ v_OptionTag_List := v_OptionTag_List & tsc_OptionTagList_timer; /* @sic R5-173024: support of session timer acc. RFC 4028 sic@ @sic R5-180230: end of grace period sic@ */ } if (v_A6 or v_A19) { // INVITE for creating an emergency session in case of no registration // Route, Contact: ports to be used v_Port_us_Contact := ?; v_Port_ps_Route := tsc_IMS_PortNumber_5060; if (not v_A19) { /* @sic R5-191118 sic@ */ // From: Display v_DisplayName := cr_DisplayName("Anonymous"); /* @sic R5s140005, R5-140901, R5w140111 sic@ */ } v_GRUUsInSIP := false; /* @sic R5s160366: no GRUU in case of no registration sic@ */ } else { // Route, Contact: ports to be used if (not v_IsGiba or (p_Context == A_2_1_A7)) { v_Protected := f_IMS_PTC_Security_GetProtectedPorts(); v_Port_us_Contact := v_Protected.Port_us; v_Port_ps_Route := v_Protected.Port_ps; } else { /* @sic R5s160366: GIBA sic@ */ v_Port_us_Contact := ?; v_Port_ps_Route := tsc_IMS_PortNumber_5060 ifpresent; } } if (v_A5) { /* re-invite @sic R5s140017 MCC160 implementation sic@ */ v_RouteBodyList := f_IMS_Request_RouteSet_RouteBodyList_RX(); v_CallId := f_IMS_PTC_ImsInfo_DialogGetCallId(); v_From := f_IMS_RequestInDialog_FromHeaderRX(); v_To := f_IMS_RequestInDialog_ToHeaderRX(); v_CSeqValue := f_IMS_RequestInDialog_CSeqValueRX(); /* @sic R5s140235, R5s140385 change 3 MCC160 implementation sic@ */ } else { /* normal case: no re-invite */ // route body acc. to conditions in A.2.1 v_Pcscf := ?; /* @sic R5s140934 change 6: P-CSCF address is checked later on (f_IMS_A_2_1_Invite_CommonChecks -> f_IMS_MessageHeader_Request_CheckRoute) sic@ */ v_RouteBodyList[0] := cr_RouteBody(*, cr_SipUri_HostPort_lr(v_Pcscf, v_Port_ps_Route)); /* @sic R5s140873: v_Pcscf; R5s140934 change 6 sic@ @sic R5-155174: "loose routing" parameter mandatory for any kind of call (i.e. emergency call too, in contrast to R5s150817) sic@ */ if (not match(p_Context, (A_2_1_A6, A_2_1_A7, A_2_1_A19))) { // => no emergency call v_RouteBodyList[1] := cr_RouteBody(*, cr_SipUri_HostPort_lr(tsc_IMS_Scscf)); } v_CallId := cr_CallId(?); v_From := cr_From(v_DisplayName, v_SipUrlFrom); v_To := cr_ToDef(p_ExpectedCalleeUri); } if (p_A8) { v_MessageHeader_Invite.geolocation := cr_GeolocationAny; v_MessageHeader_Invite.geolocationRouting := cr_GeolocationRoutingSupported; // @sic R5-134798 sic@ v_MediaType := tsc_MimeType_MultipartMixed; } else { v_MessageHeader_Invite.geolocation := omit; v_MessageHeader_Invite.geolocationRouting := omit; } v_MessageHeader_Invite.via := f_IMS_SipRequest_ViaRX(); /* additional checks in f_IMS_A_2_1_Invite_CommonChecks: check of FQDN or IP address @sic R5-133358: rport is optional for tcp sic@ */ v_MessageHeader_Invite.route := cr_Route(v_RouteBodyList); v_MessageHeader_Invite.fromField := v_From; /* additional checks in f_IMS_A_2_1_Invite_CommonChecks: check of FQDN or IP address, check against P-Preferred-Identity header */ v_MessageHeader_Invite.toField := v_To; /* additional checks in f_IMS_A_2_1_Invite_CommonChecks: check of tag */ v_MessageHeader_Invite.callId := v_CallId; /* additional checks in f_IMS_A_2_1_Invite_CommonChecks: value acc. to A4 or A5 */ v_MessageHeader_Invite.cSeq := cr_CseqDef(v_CSeqValue, "INVITE"); /* additional checks in f_IMS_A_2_1_Invite_CommonChecks: A5 */ v_MessageHeader_Invite.supported := cr_Supported(v_OptionTag_List); v_MessageHeader_Invite.contact := cr_Contact(f_Contact_SipUri_HostPortRX(?, v_Port_us_Contact, v_GRUUsInSIP)); /* additional checks in f_IMS_A_2_1_Invite_CommonChecks: check of FQDN or IP address, check of feature-param for A3, A6, A10, A12 @sic R5s150721, R5-153760: cr_SipUri_HostPort replaced by f_Contact_SipUri_HostPortRX sic@ @sic R5s160366: v_GRUUsInSIP sic@ */ v_MessageHeader_Invite.contentType := cr_ContentType(v_MediaType); v_MessageHeader_Invite.maxForwards := cr_MaxForwardsDef; // additional headers for specific conditions: if (v_IsGiba or v_A6 or v_A19) { v_MessageHeader_Invite.securityVerify := omit; if (v_A6 or v_A19) { v_MessageHeader_Invite.require := omit; v_MessageHeader_Invite.proxyRequire := omit; } } else { // -> A1 or A7 v_MessageHeader_Invite.securityVerify := f_IMS_PTC_BuildSecurityVerifyHeader(); if (not v_A5) { /* @sic R5s140017 change 9.3 sic@ */ v_MessageHeader_Invite.require := cr_RequireSecAgree; v_MessageHeader_Invite.proxyRequire := cr_ProxyRequireSecAgree; } } if (not v_IsGiba) { v_MessageHeader_Invite.pAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()); } if (v_A4) { /* @sic R5s140017 change 9.3 sic@ */ v_AcceptBodyList := {cr_AcceptBody(c_sdpApplication), cr_AcceptBody(c_ims3gppApplication)}; } if (pc_IMS_CS_PS_SRVCCMidCall) { // @sic R5-145746 sic@ v_AcceptBodyList[lengthof(v_AcceptBodyList)] := cr_AcceptBody("application/vnd.3gpp.mid-call+xml"); // add another element in the array } if (pc_IMS_CS_PS_SRVCCAlert) { // @sic R5-145746 sic@ v_AcceptBodyList[lengthof(v_AcceptBodyList)] := cr_AcceptBody("application/vnd.3gpp.state-and-event-info+xml"); // add another element in the array v_MessageHeader_Invite.recvInfo := cr_RecvInfo_Def(tsc_IMS_InfoPackageType_3gpp); // @sic R5-145746, R5s150704, sic@ } if (lengthof(v_AcceptBodyList) > 0) { if (v_A5) { // @sic R5s150971 header optional if A5 sic@ v_MessageHeader_Invite.accept := cr_Accept(superset(all from v_AcceptBodyList)) ifpresent; } else { v_MessageHeader_Invite.accept := cr_Accept(superset(all from v_AcceptBodyList)); } } if (p_Context == A_2_1_A7) { v_MessageHeader_Invite.pPreferredID := cr_PPreferredID(?); // Checked in f_IMS_A_2_1_Invite_CommonChecks } if (p_A3 and v_A4) { /* @sic R5s140263 change 5, modified by R5-142949 (-> A4 only) sic@ */ v_MessageHeader_Invite.pPreferredService := cr_PPreferredService(tsc_IMS_FeatureParamValue_3gpp_icsi_ref); /* @sic R5s140565 change 2 sic@ */ } if ((p_A3 and v_A4) or p_A11) { /* @sic R5-142949 sic@ */ v_MessageHeader_Invite.acceptContact := cr_AcceptContact; // Checked in f_IMS_A_2_1_Invite_CommonChecks (depending on conditions) } if (pc_EarlyMedia and not v_A5) { /* @sic R5-162934: condition A16 for A.2.1 sic@ @sic R5-166280: "not A5" sic@ */ v_MessageHeader_Invite.pEarlyMedia := cr_PEarlyMedia({"supported"}); } return v_MessageHeader_Invite; }

## Correction to f_IMS_A_2_1_Invite_CommonChecks

| Function name | f_IMS_A_2_1_Invite_CommonChecks |
| --- | --- |
| Reason for change | Geolocation and GeolocationRouting not needed to be checked any more in this function because of the changes in template for Invite Request |
| Summary of change | Geolocation and GeolocationRouting not checked any more in this function because of the changes in template for Invite Request |
| TTCN module | IMS_Procedures_CallControl4G5G |

function f_IMS_A_2_1_Invite_CommonChecks(IMS_DATA_REQ p_IMS_INVITE_REQ, INVITE_A_2_1_Context_Type p_Context := A_2_1_A4, boolean p_A3 := false, boolean p_A11 := false, boolean p_SrvccAlertingSupported := pc_IMS_SRVCCAlert, boolean p_OriginatingIdentificationRestriction := false, boolean p_A18 := pc_BSRVCC) runs on IMS_PTC { var MessageHeader v_MessageHeader := p_IMS_INVITE_REQ.Request.Invite.msgHeader; var SemicolonParam_List v_ContactParams := {}; var SemicolonParam_List v_AcceptContactParams := {}; var InternetProtocol_Type v_Protocol := p_IMS_INVITE_REQ.RoutingInfo.Protocol; var SipUrl v_SipUrlFrom := f_Addr_Union_GetSipUrl(v_MessageHeader.fromField.addressField); var SipUrl v_SipUrlPreferred; var template SipUrl v_ExpectedEmergencyUrl := f_IMS_PTC_ImsInfo_GetSipUriRX(emergencyId, *); var PPreferredIDValue_List v_PreferredIDValueList; var CharStringList_Type v_PreferredPublicUserIdList; var boolean v_EmergencyCallWithoutRegistration := (p_Context == A_2_1_A6); var boolean v_AllowFQDN := not v_EmergencyCallWithoutRegistration; // for A6 there is no FQDN for the UE var boolean v_CheckRport := v_EmergencyCallWithoutRegistration; var boolean v_A10 := pc_IMS_Video_FeatureTag; var boolean v_A12 := p_SrvccAlertingSupported and (p_Context != A_2_1_A5); /* @sic R5s140370 change 7 sic@ */ var boolean v_A29 := pc_NG114_v1_0_mmtel; var boolean v_A30 := pc_NG114_v1_0_audio; var boolean v_A31 := pc_NG114_v1_0_video; var integer i; var template (present) GenValue v_SipInstanceValue; var template integer v_Port_us_Via := *; if ((v_EmergencyCallWithoutRegistration and (v_Protocol==udp))) { /* @sic R5-172412 sic@ */ v_Port_us_Via := tsc_IMS_PortNumber_5060; } if (ispresent(v_MessageHeader.contact.contactBody.contactAddresses[0].contactParams)) { v_ContactParams := v_MessageHeader.contact.contactBody.contactAddresses[0].contactParams; } if (ispresent(v_MessageHeader.acceptContact.acValues[0].acRcParams)) { v_AcceptContactParams := v_MessageHeader.acceptContact.acValues[0].acRcParams; } // **** Common checks **** f_IMS_MessageHeader_Request_CheckVia(v_MessageHeader, v_Protocol, v_AllowFQDN, v_CheckRport, v_Port_us_Via); /* check whether via contains "SIP URI with IP address or FQDN" (see @sic R5s120727 sic@) @sic R5-172412: v_Port_us_Via sic@ */ f_IMS_MessageHeader_CheckContactAddr(v_MessageHeader, v_EmergencyCallWithoutRegistration); /* check whether contact contains "SIP URI with IP address or FQDN" (see @sic R5s120727 sic@) @sic R5s160366: v_EmergencyCallWithoutRegistration sic@ */ if (p_Context != A_2_1_A5) { f_IMS_MessageHeader_CheckTo(v_MessageHeader, noToTag); /* check that there is no To tag */ f_IMS_MessageHeader_Request_CheckRoute(v_MessageHeader); /* @sic R5s140934 change 7 sic@ */ } // **** Check URIs in From header, P-Preferred-Identity header and further headers **** select (p_Context) { case (A_2_1_A4) { // Check Call-Id if (match(v_MessageHeader.callId.callid, f_IMS_PTC_ImsInfo_GetCallIdRegistration())) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A4: invalid 'CallId' in INVITE (has to be different than in previous REGISTER)"); } // Additional checks regarding SIP URI in the From header @sic R5-130750 change 3 sic@ if (not p_OriginatingIdentificationRestriction) { /* @sic R5-160975, R5s160687: p_OriginatingIdentificationRestriction sic@ */ if (not f_IMS_PublicUserId_Match(f_IMS_PTC_ImsInfo_GetAllIMPUs(), v_SipUrlFrom)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "invalid 'From'"); } if (ispresent(v_MessageHeader.pPreferredID.pPreferredIDValueList)) { v_PreferredIDValueList := v_MessageHeader.pPreferredID.pPreferredIDValueList; for (i:=0; i < lengthof(v_PreferredIDValueList); i := i+1) { v_PreferredPublicUserIdList[i] := f_SIP_SipUrl_GetPublicUserIdentity(f_Addr_Union_GetSipUrl(v_PreferredIDValueList[i])); } if (not f_IMS_PublicUserId_Match(v_PreferredPublicUserIdList, v_SipUrlFrom)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A4: invalid 'From': does not match with P-Preferred-Identity"); } } } } case (A_2_1_A5) { /* @sic R5s140017 MCC160 implementation: no further checks for A5 are needed in this function anymore sic@ */ } case (A_2_1_A6) { v_SipInstanceValue := cr_GenValueQuoted(f_SIP_InstanceId()); /* @sic R5s130756 change 11; R5w140111: GenValue; R5s141212 sic@ */ if (not match(v_ContactParams, cr_SemicolonParam_List_OneSpecificParam("+sip.instance", v_SipInstanceValue))) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "IMEI code of the UE, coded as specified in draft-montemurro-gsma-imei-urn, not matching"); } } case (A_2_1_A7) { if (not match(v_SipUrlFrom, v_ExpectedEmergencyUrl)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A7: invalid 'From'"); } v_SipUrlPreferred := f_Addr_Union_GetSipUrl(v_MessageHeader.pPreferredID.pPreferredIDValueList[0]); if (not match(v_SipUrlPreferred, v_ExpectedEmergencyUrl)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A7: invalid 'P-Preferred-Identity'"); } } case (A_2_1_A19) { // @sic R5-174545, Change 2 R5s180463 sic@ // Via, Route and Contact header are checked in function f_IMS_InviteRequest_MessageHeaderRX if (not match(v_SipUrlFrom, v_ExpectedEmergencyUrl)) { // @sic R5-191118 sic@ f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A19: invalid 'From'"); } } case else { FatalError(__FILE__, __LINE__, "unknown context"); } } // **** Check further Contact feature-param acc. to further conditions of A.2.1: **** // if (p_A3 or v_A29) { // A.1.2 condition A3: MTSI @sic R5-211418: v_A29 sic@ if (not match(v_ContactParams, cr_FeatureParamList_3gpp_icsi_ref)) { // @sic R5s130738: string needs to be quoted; R5-142247 sic@ f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A3: Missing feature-param in Contact header"); } } if (p_A3 and (p_Context == A_2_1_A4)) { // @sic R5-142949 sic@ if (not match(v_AcceptContactParams, cr_FeatureParamList_3gpp_icsi_ref)) { // @sic R5s130738: string needs to be quoted; R5-142247 sic@ f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A3: Missing feature-param in AcceptContact header"); } } if (v_A10 or v_A31) { /* A.1.2 condition A10/A31: UE supports video feature tag @sic R5-211418: v_A31 sic@ A.1.2 condition A11: INVITE for creating a video call @sic R5-153766: "v_A10 and p_A11 and (p_Context != A_2_1_A6) and (p_Context != A_2_1_A7)" sic@ @sic R5-160553: "v_A10 and p_A11" sic@ @sic R5-176371: "v_A10" for ContactParams, "v_A10 and p_A11" for AcceptContactParams sic@ */ if (not match(v_ContactParams, cr_VideoInContactParams)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A10, A11: Missing feature-param in Contact header"); } if (p_A11 and not match(v_AcceptContactParams, cr_VideoInContactParams)) { /* @sic R5-176371 sic@ */ f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A10, A11: Missing feature-param in Accept-Contact header"); } } if ((v_A30 or f_IMS_PTC_AudioFeatureTagIsRequired()) and not match(v_ContactParams, cr_AudioInContactParams)) { // check 'feature-param' @sic R5-165943, R5-170564 sic@ @sic R5-173021 sic@ @sic R5-177127 sic@ @sic R5-211418: v_A30 sic@ f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "missing audio feature-param in Contact header"); } if (v_A12) { // A.1.2 condition A12: INVITE for creating a voice or video call and UE supports g.3gpp.srvcc-alerting media feature tag if (not match(v_ContactParams, cr_SemicolonParam_List_OneSpecificParamNoValue(tsc_IMS_FeatureCap_SrvccAlerting))) { /* @sic R5s140370 change 7: tsc_IMS_FeatureCap_SrvccAlerting sic@ */ f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A12: Missing feature-param"); } } if (p_A18 and (p_Context != A_2_1_A5)) { // A.1.2 condition A18: UE indicates g.3gpp.ps2cs-srvcc-orig-pre-alerting media feature tag in INVITE request @sic R5-171486 sic@ @sic R5-182793 sic@ if (not match(v_ContactParams, cr_SemicolonParam_List_OneSpecificParamNoValue(tsc_IMS_FeatureCap_SrvccPreAlerting))) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A18: Missing feature-param"); } } }

## Correction to f_IMS_INVITE_ReceiveRequest_A_2_1_Optional

| Function name | f_IMS_INVITE_ReceiveRequest_A_2_1_Optional |
| --- | --- |
| Reason for change | Geolocation and GeolocationRouting not needed to be checked any more in this function because of the changes in template for Invite Request. Parameter A8 becomes superfluous. |
| Summary of change | Geolocation and GeolocationRouting not checked any more in this function because of the changes in template for Invite Request. Parameter A8 becomes superfluous. |
| TTCN module | IMS_Procedures_CallControl4G5G |

function f_IMS_INVITE_ReceiveRequest_A_2_1_Optional(INVITE_A_2_1_Context_Type p_Context, template (present) SipUrl p_ExpectedCalleeUri, template (present) MessageHeader p_MessageHeader_Invite, template (omit) float p_WaitDuration := omit, boolean p_A3 := pc_MultimediaTelephonyService, /* @sic R5s130510 additional change 1: tsc_IMS_MTSI replaced by pc_MultimediaTelephonyService sic@ */ boolean p_A11 := false, boolean p_SrvccAlertingSupported := pc_IMS_SRVCCAlert, boolean p_OriginatingIdentificationRestriction := false) runs on IMS_PTC return template (omit) IMS_DATA_REQ { var template (omit) IMS_DATA_REQ v_IMS_DATA_REQ; var boolean v_UseMIME := (ispresent(p_MessageHeader_Invite.contentType) and match(tsc_MimeType_MultipartMixed, p_MessageHeader_Invite.contentType.mTypeSubtype)); var template (present) MessageBody v_MessageBody := f_SIP_MessageBodySDP_RX(v_UseMIME); /* @sic R5w140112 sic@ @sic R5s210009: v_UseMIME instead of p_A8 sic@ */ var boolean v_IsReInvite := (p_Context == A_2_1_A5); var boolean v_IsEmergencyCallWithoutRegistration := (p_Context == A_2_1_A6); v_IMS_DATA_REQ := f_IMS_INVITE_ReceiveRequest_Optional(p_ExpectedCalleeUri, p_MessageHeader_Invite, v_MessageBody, p_WaitDuration, v_IsReInvite, v_IsEmergencyCallWithoutRegistration); // @sic R5s190882 sic@ if (isvalue(v_IMS_DATA_REQ)) { f_IMS_A_2_1_Invite_CommonChecks(valueof(v_IMS_DATA_REQ), p_Context, p_A3, p_A11, p_SrvccAlertingSupported, p_OriginatingIdentificationRestriction); /* @sic R5-160975: p_OriginatingIdentificationRestriction sic@ */ } return v_IMS_DATA_REQ; }

## Correction to f_IMS_SRVCC_ReInvite_AnnexC24_Step1

| Function name | f_IMS_SRVCC_ReInvite_AnnexC24_Step1 |
| --- | --- |
| Reason for change | Parameter A8 no longer needed to be passed to f_IMS_INVITE_ReceiveRequest_A_2_1_Optional as it has become superfluous. |
| Summary of change | Parameter A8 no longer passed to f_IMS_INVITE_ReceiveRequest_A_2_1_Optional as it has become superfluous. |
| TTCN module | IMS_CommonSRVCC |

function f_IMS_SRVCC_ReInvite_AnnexC24_Step1(charstring p_OtherCalleeUri, template (omit) float p_WaitDuration := omit, boolean p_GeolocationInfoAvailable := false) runs on IMS_PTC return template (omit) IMS_InviteRequestWithSdp_Type { var template (present) SDP_Message v_SDP_MessageRX := f_IMS_BuildSDP_AnnexC24_Step1(); var template (omit) IMS_DATA_REQ v_IMS_DATA_REQ; var IMS_DATA_REQ v_IMS_DATA_REQ_Val; var SDP_Message v_SDP_Message; var INVITE_A_2_1_Context_Type v_Context := A_2_1_A5; var template (present) SipUrl v_CalleeSipUrl := f_SIP_BuildSipUri_RX(p_OtherCalleeUri, *); /* @sic R5-151957 sic@ */ var template (present) MessageHeader v_MessageHeader_Invite := f_IMS_InviteRequest_MessageHeaderRX(v_CalleeSipUrl, -, v_Context, -, p_GeolocationInfoAvailable); // @sic R5s190882 sic@ v_IMS_DATA_REQ := f_IMS_INVITE_ReceiveRequest_A_2_1_Optional(v_Context, v_CalleeSipUrl, v_MessageHeader_Invite, p_WaitDuration); if (isvalue(v_IMS_DATA_REQ)) { v_IMS_DATA_REQ_Val := valueof(v_IMS_DATA_REQ); v_SDP_Message := f_IMS_MessageBody_CheckSdpMessageAndGeolocation(v_IMS_DATA_REQ_Val.Request.Invite.messageBody, v_SDP_MessageRX, p_GeolocationInfoAvailable); return f_IMS_InviteRequestWithSdpMO(v_IMS_DATA_REQ_Val, v_SDP_Message); } return omit; }

## Correction to f_IMS_SRVCC_ReInvite_AnnexC31_Step1

| Function name | f_IMS_SRVCC_ReInvite_AnnexC31_Step1 |
| --- | --- |
| Reason for change | Parameter A8 no longer needed to be passed to f_IMS_INVITE_ReceiveRequest_A_2_1_Optional as it has become superfluous. |
| Summary of change | Parameter A8 no longer passed to f_IMS_INVITE_ReceiveRequest_A_2_1_Optional as it has become superfluous. |
| TTCN module | IMS_CommonSRVCC |

function f_IMS_SRVCC_ReInvite_AnnexC31_Step1(charstring p_OtherCalleeUri, boolean p_GeolocationInfoAvailable := false, template (omit) float p_WaitDuration := omit) // @sic R5-174016 sic@ runs on IMS_PTC return template (omit) IMS_InviteRequestWithSdp_Type { var template (omit) float v_WaitDuration := p_WaitDuration; // @sic R5-174016 sic@ var template (present) SDP_Message v_SDP_MessageRX := f_IMS_BuildSDP_AnnexC31_Step1(); var template (omit) IMS_DATA_REQ v_IMS_DATA_REQ; var IMS_DATA_REQ v_IMS_DATA_REQ_Val; var SDP_Message v_SDP_Message; var INVITE_A_2_1_Context_Type v_Context := A_2_1_A5; var template (present) SipUrl v_CalleeSipUrl := f_SIP_BuildSipUri_RX(p_OtherCalleeUri, *); /* @sic R5s150204 sic@ */ var template (present) MessageHeader v_MessageHeader_Invite; v_MessageHeader_Invite := f_IMS_InviteRequest_MessageHeaderRX(v_CalleeSipUrl, -, v_Context, -, p_GeolocationInfoAvailable); // @sic R5s190882 sic@ v_MessageHeader_Invite.reason := f_IMS_SIP_Reason487_RX(); // @sic R5s140370 change 3 - MCC160 implementation sic@ v_IMS_DATA_REQ := f_IMS_INVITE_ReceiveRequest_A_2_1_Optional(v_Context, v_CalleeSipUrl, v_MessageHeader_Invite, v_WaitDuration); if (isvalue(v_IMS_DATA_REQ)) { // @sic R5-174016 sic@ v_IMS_DATA_REQ_Val := valueof(v_IMS_DATA_REQ); v_SDP_Message := f_IMS_MessageBody_CheckSdpMessageAndGeolocation(v_IMS_DATA_REQ_Val.Request.Invite.messageBody, v_SDP_MessageRX, p_GeolocationInfoAvailable); return f_IMS_InviteRequestWithSdpMO(v_IMS_DATA_REQ_Val, v_SDP_Message); } return omit; }

## Correction to f_ECall_InitialInvite_AnnexC47

| Function name | f_ECall_InitialInvite_AnnexC47 |
| --- | --- |
| Reason for change | Parameter A8 no longer needed to be passed to f_IMS_INVITE_ReceiveRequest_A_2_1_Optional as it has become superfluous. |
| Summary of change | Parameter A8 no longer passed to f_IMS_INVITE_ReceiveRequest_A_2_1_Optional as it has become superfluous. |
| TTCN module | IMS_eCall_Functions |

function f_ECall_InitialInvite_AnnexC47(ECall_Type p_ECallType) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { var IMS_DATA_REQ v_IMS_DATA_REQ; var MIME_Message v_MIME_Message; var MIME_Part v_MIME_Part; var SDP_Message v_SdpOffer; var template (present) SDP_Message v_SDP_Message_Step1; var template (present) SipUrl v_ECallPSAP_Rx := f_SipUrl_eCall_RX(p_ECallType); var template (present) MessageHeader v_MessageHeader; var template (present) AcceptBody_List v_AcceptBodyList := { cr_AcceptBody(tsc_MimeType_EmergencyCallDataControlXML), * }; // condition A20, A21 var template (present) SemicolonParam_List v_DispositionParams := { cr_GenericParam("handling", "optional"), * }; // condition A20, A21 var boolean v_A8 := pc_IMS_Geolocation; var INVITE_A_2_1_Context_Type v_Context := A_2_1_A7; var charstring v_CidUrl; var ECall_Type v_ECallType := p_ECallType; // Step 1. Receive INVITE for an eCall v_MessageHeader := f_IMS_InviteRequest_MessageHeaderRX(v_ECallPSAP_Rx, -, v_Context, -, v_A8); if ((v_ECallType == MANUAL) or (v_ECallType == AUTOMATIC)) { // @sic R5-220971 sic@ v_MessageHeader.callInfo := cr_CallInfo(-, "EmergencyCallData.eCall.MSD"); // condition A20, A21 @sic R5-210324: "EmergencyCallData" instead of "emergencyCallData" sic@ v_MessageHeader.recvInfo := cr_RecvInfo_Def("EmergencyCallData.eCall.MSD"); // condition A20, A21 v_MessageHeader.accept := cr_Accept(v_AcceptBodyList); // condition A20, A21 } v_MessageHeader.contentType := cr_ContentTypeMIME; // condition A20, A21, A8, A25 @sic R5-217960 sic@ v_IMS_DATA_REQ := valueof(f_IMS_INVITE_ReceiveRequest_A_2_1_Optional(v_Context, v_ECallPSAP_Rx, v_MessageHeader)); v_MIME_Message := v_IMS_DATA_REQ.Request.Invite.messageBody.mimeMessageBody; // Check PIDF if (v_A8) { v_MIME_Part := f_IMS_MIME_MessageBody_GetBody(v_MIME_Message, tsc_MimeType_Pidf); f_IMS_DecodeAndCheckGeolocation(v_MIME_Part.body.xmlBody); } // Check SDP v_SDP_Message_Step1 := f_IMS_BuildSDP_EmergencyCallOffer_RX(); // Build RX SDP template using codecs AMR and AMR_WB v_MIME_Part := f_IMS_MIME_MessageBody_GetBody(v_MIME_Message, c_sdpApplication); v_SdpOffer := f_IMS_SIP_DecodeMatchAndCheckSDP(v_MIME_Part.body.sdpMessageBody, v_SDP_Message_Step1); // Check eCall MSD v_CidUrl := v_IMS_DATA_REQ.Request.Invite.msgHeader.callInfo.callInfoBody[0].url; f_IMS_ECall_MIME_DecodeAndCheckEmergencyCallData(v_MIME_Message, tsc_MimeType_EmergencyCallDataMSD, v_CidUrl, v_DispositionParams); return f_IMS_InviteRequestWithSdpMO(v_IMS_DATA_REQ, v_SdpOffer); }

## Correction to f_IMS_INVITE_ReceiveRequest

| Function name | f_IMS_INVITE_ReceiveRequest |
| --- | --- |
| Reason for change | Parameter A8 no longer needed to be passed to f_IMS_INVITE_ReceiveRequest_A_2_1_Optional as it has become superfluous. |
| Summary of change | Parameter A8 no longer passed to f_IMS_INVITE_ReceiveRequest_A_2_1_Optional as it has become superfluous. |
| TTCN module | IMS_Procedures_CallControl4G5G |

function f_IMS_INVITE_ReceiveRequest(INVITE_A_2_1_Context_Type p_Context := A_2_1_A4, OptionTag_List p_OptionTagsForSupported := tsc_OptionTagList_100rel, template SipUrl p_OtherCalleeUri := omit, boolean p_A3 := pc_MultimediaTelephonyService, /* @sic R5s130510 additional change 1: tsc_IMS_MTSI replaced by pc_MultimediaTelephonyService sic@ */ boolean p_A8 := false, boolean p_A11 := false, boolean p_SrvccAlertingSupported := pc_IMS_SRVCCAlert) runs on IMS_PTC return IMS_DATA_REQ { var template (omit) float v_WaitDuration := omit; // no wait duration var template (omit) IMS_DATA_REQ v_IMS_DATA_REQ; var template (present) SipUrl v_ExpectedCalleeUri := f_IMS_INVITE_CalleeUriRX(p_OtherCalleeUri, p_Context); var template (present) MessageHeader v_MessageHeader_Invite := f_IMS_InviteRequest_MessageHeaderRX(v_ExpectedCalleeUri, p_OptionTagsForSupported, p_Context, p_A3, p_A8, p_A11); // @sic R5s190882 sic@ v_IMS_DATA_REQ := f_IMS_INVITE_ReceiveRequest_A_2_1_Optional(p_Context, v_ExpectedCalleeUri, v_MessageHeader_Invite, v_WaitDuration, p_A3, p_A11, p_SrvccAlertingSupported); return valueof(v_IMS_DATA_REQ); // as we don't have any WaitDuration f_IMS_INVITE_ReceiveRequest_A_2_1_Optional returns a value (omit could only occur when a timer expires) }

## Correction to f_IMS_MOCallSetup_ReceiveINVITE_Optional

| Function name | f_IMS_MOCallSetup_ReceiveINVITE_Optional |
| --- | --- |
| Reason for change | Parameter A8 no longer needed to be passed to f_IMS_INVITE_ReceiveRequest_A_2_1_Optional as it has become superfluous. |
| Summary of change | Parameter A8 no longer passed to f_IMS_INVITE_ReceiveRequest_A_2_1_Optional as it has become superfluous. |
| TTCN module | IMS_Procedures_CallControl4G5G |

function f_IMS_MOCallSetup_ReceiveINVITE_Optional(template (omit) float p_WaitDuration := omit, template SipUrl p_OtherCalleeUri := omit, template SDP_Message p_SDP_Message := omit, IMS_SDP_AudioCodecList_Type p_AudioCodecList, boolean p_PreconditionSupported := true, boolean p_OIR := false, boolean p_GeolocationInfoAvailable := false) runs on IMS_PTC return template (omit) IMS_InviteRequestWithSdp_Type { var OptionTag_List v_OptionTagsForSupported := f_IMS_Invite_OptionTagsSupported(p_PreconditionSupported); /* @sic R5s180197 sic@ */ var template (omit) IMS_DATA_REQ v_IMS_DATA_REQ; var template (omit) IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp := omit; var template (present) SDP_Message v_SDP_MessageTemplate; var INVITE_A_2_1_Context_Type v_Context := A_2_1_A4; var template (present) SipUrl v_ExpectedCalleeUri := f_IMS_INVITE_CalleeUriRX(p_OtherCalleeUri, v_Context); var template (present) MessageHeader v_MessageHeader_Invite := f_IMS_InviteRequest_MessageHeaderRX(v_ExpectedCalleeUri, v_OptionTagsForSupported, v_Context, -, p_GeolocationInfoAvailable); // @sic R5s190882 sic@ var IMS_DATA_REQ v_IMS_DATA_REQ_Value; var SDP_Message v_SDP_Message; var INVITE_Request v_InviteRequest; var boolean v_CheckForbiddenCodecParameters; /* @sic R5-180231 sic@ */ if (ispresent(p_SDP_Message)) { /* specific SDP message content @sic R5s180197 sic@ */ v_SDP_MessageTemplate := p_SDP_Message; v_CheckForbiddenCodecParameters := false; /* no automatic check for forbidden parameters in case of specific (i.e. non C.21(abcd)) SDP messages */ } else { /* SDP message content according to C.21(abcd) */ v_SDP_MessageTemplate := f_IMS_BuildSDP_InitialOfferAudio_RX(p_PreconditionSupported, p_AudioCodecList); /* @sic R5s150704 - additional changes sic@ @sic R5-172931 sic@ */ v_CheckForbiddenCodecParameters := true; /* automatic check for forbidden parameters for Cat-M1 (C.21d), EVS (C.44), WLAN (C.21a) and normal cases (C.21); @sic R5-180231: grace period, R5-192309: grace period for normal cases expired sic@ */ } // Step 2. Receive INVITE Request v_IMS_DATA_REQ := f_IMS_INVITE_ReceiveRequest_A_2_1_Optional(v_Context, v_ExpectedCalleeUri, v_MessageHeader_Invite, p_WaitDuration, -, -, -, p_OIR); if (isvalue(v_IMS_DATA_REQ)) { v_IMS_DATA_REQ_Value := valueof(v_IMS_DATA_REQ); v_InviteRequest := v_IMS_DATA_REQ_Value.Request.Invite; v_SDP_Message := f_IMS_MessageBody_CheckSdpMessageAndGeolocation(v_InviteRequest.messageBody, v_SDP_MessageTemplate, p_GeolocationInfoAvailable); // check media description: f_IMS_SDP_CheckMediaDescr_AudioCodecs(v_SDP_Message.media_list[0], p_AudioCodecList, v_CheckForbiddenCodecParameters); /* @sic R5-151968: f_IMS_SDP_CheckMediaDescr_EVS_16000_1, f_IMS_SDP_CheckMediaDescr_AMR_16000_1 replaced by f_IMS_SDP_CheckMediaDescr_AudioCodec sic@ @sic R5-172931: f_IMS_SDP_CheckMediaDescr_AudioCodecs instead of f_IMS_SDP_CheckMediaDescr_AudioCodec sic@ @sic R5-180231: v_CheckForbiddenCodecParameters sic@ */ v_InviteRequestWithSdp := f_IMS_InviteRequestWithSdpMO(v_IMS_DATA_REQ_Value, v_SDP_Message); } return v_InviteRequestWithSdp; }

| Function name | f_IMS_InviteRequest_MessageHeaderRX |
| --- | --- |
| Reason for change | Geolocation and GeolocationRouting not defaulted to omit in the template for Invite Request if condition A8 is false. |
| Summary of change | Geolocation and GeolocationRouting defaulted to omit in the template for Invite Request if condition A8 is false. |
| TTCN module | IMS_Procedures_CallControl4G5G |

function f_IMS_InviteRequest_MessageHeaderRX(template (present) SipUrl p_ExpectedCalleeUri, OptionTag_List p_OptionTagsForSupported := tsc_OptionTagList_100rel, INVITE_A_2_1_Context_Type p_Context := A_2_1_A4, boolean p_A3 := pc_MultimediaTelephonyService, /* @sic R5s130510 additional change 1: tsc_IMS_MTSI replaced by pc_MultimediaTelephonyService sic@ */ boolean p_A8 := false, boolean p_A11 := false) runs on IMS_PTC return template (present) MessageHeader { var boolean v_IsGiba := f_IMS_PTC_SecurityScheme_IsGiba(); // => A1/A2 var template (present) MessageHeader v_MessageHeader_Invite := cr_MessageHeader_Dummy; var template (present) RouteBody_List v_RouteBodyList; var IMS_ProtectedPorts_Type v_Protected; var template integer v_Port_us_Contact := *; var template integer v_Port_ps_Route := *; var template DisplayName v_DisplayName := *; var template (present) SipUrl v_SipUrlFrom := ?; /* per default any SIP URI is accepted in the From header in the first place; further checks may be applied in f_IMS_A_2_1_Invite_CommonChecks (depending on the conditions acc. to A.2.1 */ var template (present) charstring v_Pcscf; var charstring v_MediaType := c_sdpApplication; var template (present) CallId v_CallId; var template (present) integer v_CSeqValue := ?; var template (present) From v_From; var template (present) To v_To; var template AcceptBody_List v_AcceptBodyList := {}; var OptionTag_List v_OptionTag_List := p_OptionTagsForSupported; var boolean v_GRUUsInSIP := pc_IMS_GRUUsInSIP; var boolean v_A4 := (p_Context == A_2_1_A4); // INVITE creating a dialog @sic R5-166280 sic@ var boolean v_A5 := (p_Context == A_2_1_A5); // re-INVITE within a dialog @sic R5-166280 sic@ var boolean v_A6 := (p_Context == A_2_1_A6); // emergency session in case of no registration @sic R5-166280 sic@ var boolean v_A19 := (p_Context == A_2_1_A19); // emergency session in case of no registration (GIBA) @sic R5-174545 sic@ var boolean v_A26 := pc_IMS_Session_Timer; // UE supports Session Timer @sic R5-192321 sic@ if (pc_IMS_CS_PS_SRVCCMidCall or pc_IMS_CS_PS_SRVCCAlert) { // @sic R5-145746 sic@ v_OptionTag_List := v_OptionTag_List & tsc_OptionTagList_norefersub; } if (v_A4 and v_A26) { /* @sic R5-192321 sic@ NOTE: A4, A6, A7 and A19 are mutual exclusive; use of A20, A21 (eCall) is unclear */ v_OptionTag_List := v_OptionTag_List & tsc_OptionTagList_timer; /* @sic R5-173024: support of session timer acc. RFC 4028 sic@ @sic R5-180230: end of grace period sic@ */ } if (v_A6 or v_A19) { // INVITE for creating an emergency session in case of no registration // Route, Contact: ports to be used v_Port_us_Contact := ?; v_Port_ps_Route := tsc_IMS_PortNumber_5060; if (not v_A19) { /* @sic R5-191118 sic@ */ // From: Display v_DisplayName := cr_DisplayName("Anonymous"); /* @sic R5s140005, R5-140901, R5w140111 sic@ */ } v_GRUUsInSIP := false; /* @sic R5s160366: no GRUU in case of no registration sic@ */ } else { // Route, Contact: ports to be used if (not v_IsGiba or (p_Context == A_2_1_A7)) { v_Protected := f_IMS_PTC_Security_GetProtectedPorts(); v_Port_us_Contact := v_Protected.Port_us; v_Port_ps_Route := v_Protected.Port_ps; } else { /* @sic R5s160366: GIBA sic@ */ v_Port_us_Contact := ?; v_Port_ps_Route := tsc_IMS_PortNumber_5060 ifpresent; } } if (v_A5) { /* re-invite @sic R5s140017 MCC160 implementation sic@ */ v_RouteBodyList := f_IMS_Request_RouteSet_RouteBodyList_RX(); v_CallId := f_IMS_PTC_ImsInfo_DialogGetCallId(); v_From := f_IMS_RequestInDialog_FromHeaderRX(); v_To := f_IMS_RequestInDialog_ToHeaderRX(); v_CSeqValue := f_IMS_RequestInDialog_CSeqValueRX(); /* @sic R5s140235, R5s140385 change 3 MCC160 implementation sic@ */ } else { /* normal case: no re-invite */ // route body acc. to conditions in A.2.1 v_Pcscf := ?; /* @sic R5s140934 change 6: P-CSCF address is checked later on (f_IMS_A_2_1_Invite_CommonChecks -> f_IMS_MessageHeader_Request_CheckRoute) sic@ */ v_RouteBodyList[0] := cr_RouteBody(*, cr_SipUri_HostPort_lr(v_Pcscf, v_Port_ps_Route)); /* @sic R5s140873: v_Pcscf; R5s140934 change 6 sic@ @sic R5-155174: "loose routing" parameter mandatory for any kind of call (i.e. emergency call too, in contrast to R5s150817) sic@ */ if (not match(p_Context, (A_2_1_A6, A_2_1_A7, A_2_1_A19))) { // => no emergency call v_RouteBodyList[1] := cr_RouteBody(*, cr_SipUri_HostPort_lr(tsc_IMS_Scscf)); } v_CallId := cr_CallId(?); v_From := cr_From(v_DisplayName, v_SipUrlFrom); v_To := cr_ToDef(p_ExpectedCalleeUri); } if (p_A8) { v_MessageHeader_Invite.geolocation := cr_GeolocationAny; v_MessageHeader_Invite.geolocationRouting := cr_GeolocationRoutingSupported; // @sic R5-134798 sic@ v_MediaType := tsc_MimeType_MultipartMixed; } else { v_MessageHeader_Invite.geolocation := omit; v_MessageHeader_Invite.geolocationRouting := omit; } v_MessageHeader_Invite.via := f_IMS_SipRequest_ViaRX(); /* additional checks in f_IMS_A_2_1_Invite_CommonChecks: check of FQDN or IP address @sic R5-133358: rport is optional for tcp sic@ */ v_MessageHeader_Invite.route := cr_Route(v_RouteBodyList); v_MessageHeader_Invite.fromField := v_From; /* additional checks in f_IMS_A_2_1_Invite_CommonChecks: check of FQDN or IP address, check against P-Preferred-Identity header */ v_MessageHeader_Invite.toField := v_To; /* additional checks in f_IMS_A_2_1_Invite_CommonChecks: check of tag */ v_MessageHeader_Invite.callId := v_CallId; /* additional checks in f_IMS_A_2_1_Invite_CommonChecks: value acc. to A4 or A5 */ v_MessageHeader_Invite.cSeq := cr_CseqDef(v_CSeqValue, "INVITE"); /* additional checks in f_IMS_A_2_1_Invite_CommonChecks: A5 */ v_MessageHeader_Invite.supported := cr_Supported(v_OptionTag_List); v_MessageHeader_Invite.contact := cr_Contact(f_Contact_SipUri_HostPortRX(?, v_Port_us_Contact, v_GRUUsInSIP)); /* additional checks in f_IMS_A_2_1_Invite_CommonChecks: check of FQDN or IP address, check of feature-param for A3, A6, A10, A12 @sic R5s150721, R5-153760: cr_SipUri_HostPort replaced by f_Contact_SipUri_HostPortRX sic@ @sic R5s160366: v_GRUUsInSIP sic@ */ v_MessageHeader_Invite.contentType := cr_ContentType(v_MediaType); v_MessageHeader_Invite.maxForwards := cr_MaxForwardsDef; // additional headers for specific conditions: if (v_IsGiba or v_A6 or v_A19) { v_MessageHeader_Invite.securityVerify := omit; if (v_A6 or v_A19) { v_MessageHeader_Invite.require := omit; v_MessageHeader_Invite.proxyRequire := omit; } } else { // -> A1 or A7 v_MessageHeader_Invite.securityVerify := f_IMS_PTC_BuildSecurityVerifyHeader(); if (not v_A5) { /* @sic R5s140017 change 9.3 sic@ */ v_MessageHeader_Invite.require := cr_RequireSecAgree; v_MessageHeader_Invite.proxyRequire := cr_ProxyRequireSecAgree; } } if (not v_IsGiba) { v_MessageHeader_Invite.pAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()); } if (v_A4) { /* @sic R5s140017 change 9.3 sic@ */ v_AcceptBodyList := {cr_AcceptBody(c_sdpApplication), cr_AcceptBody(c_ims3gppApplication)}; } if (pc_IMS_CS_PS_SRVCCMidCall) { // @sic R5-145746 sic@ v_AcceptBodyList[lengthof(v_AcceptBodyList)] := cr_AcceptBody("application/vnd.3gpp.mid-call+xml"); // add another element in the array } if (pc_IMS_CS_PS_SRVCCAlert) { // @sic R5-145746 sic@ v_AcceptBodyList[lengthof(v_AcceptBodyList)] := cr_AcceptBody("application/vnd.3gpp.state-and-event-info+xml"); // add another element in the array v_MessageHeader_Invite.recvInfo := cr_RecvInfo_Def(tsc_IMS_InfoPackageType_3gpp); // @sic R5-145746, R5s150704, sic@ } if (lengthof(v_AcceptBodyList) > 0) { if (v_A5) { // @sic R5s150971 header optional if A5 sic@ v_MessageHeader_Invite.accept := cr_Accept(superset(all from v_AcceptBodyList)) ifpresent; } else { v_MessageHeader_Invite.accept := cr_Accept(superset(all from v_AcceptBodyList)); } } if (p_Context == A_2_1_A7) { v_MessageHeader_Invite.pPreferredID := cr_PPreferredID(?); // Checked in f_IMS_A_2_1_Invite_CommonChecks } if (p_A3 and v_A4) { /* @sic R5s140263 change 5, modified by R5-142949 (-> A4 only) sic@ */ v_MessageHeader_Invite.pPreferredService := cr_PPreferredService(tsc_IMS_FeatureParamValue_3gpp_icsi_ref); /* @sic R5s140565 change 2 sic@ */ } if ((p_A3 and v_A4) or p_A11) { /* @sic R5-142949 sic@ */ v_MessageHeader_Invite.acceptContact := cr_AcceptContact; // Checked in f_IMS_A_2_1_Invite_CommonChecks (depending on conditions) } if (pc_EarlyMedia and not v_A5) { /* @sic R5-162934: condition A16 for A.2.1 sic@ @sic R5-166280: "not A5" sic@ */ v_MessageHeader_Invite.pEarlyMedia := cr_PEarlyMedia({"supported"}); } return v_MessageHeader_Invite; }