3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220492

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2466 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation, for the preamble the function f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); is called. This should be replaced with f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1, SUPPORTED); in order to indicate that interworking without N26 is supported. In the current TTCN implementation, after the NR Cell 1 is created, it is set at serving cell power immediately. Sometimes, this causes the testcase to fail because the UE may prefer to camp on NR Cell if its detected before Eutra Cell. According to preamble, the UE is first registered on Eutra. In order to address this instability, it is proposed to set the NR Cell to serving cell power after the coordination message from Eutra for “Preamble” is received. In the current TTCN implementation, at Step 5 the TTCN checks if the IE UL Data Status is present in SERVICE REQ. This is not needed, as the UE may not have any UL data at this point, and the Prose also does not mention to check this. Hence it is proposed to relax the check for IE UL Data Status at Step 5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of callng f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1);, called f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1, SUPPORTED); Called the function f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); after the call to f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Preamble"); Instead of setting ? for the IE UL Data Status, set it to *. Please detailed changes below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required to test case 11.1.6	2

1.1	f_TC_11_1_6_NR5GC_EUTRA	2

1.2	f_TC_11_1_6_NR5GC	3

1.3	fl_TC_11_1_6_NR5GC_TestBody	5

# Corrections required to test case 11.1.6

## f_TC_11_1_6_NR5GC_EUTRA

| Function name | f_TC_11_1_6_NR5GC_EUTRA |
| --- | --- |
| Reason for change | In the current TTCN implementation, for the preamble the function f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); is called. This should be replaced with f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1, SUPPORTED); in order to indicate that interworking without N26 is supported. |
| Summary of change | Instead of callng f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1);, called f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1, SUPPORTED); |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_11_1_6_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { //MT MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode without N26 interface / Success f_EUTRA38_IRAT_Init(c31, HANDLE_UM_DATA); // @sic R5s210414 sic@ //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_11_1_6_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble (eutra_Cell1, E2_CONNECTED); }

After Change:

function f_TC_11_1_6_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { //MT MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode without N26 interface / Success f_EUTRA38_IRAT_Init(c31, HANDLE_UM_DATA); // @sic R5s210414 sic@ //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1, SUPPORTED); //WA#11_1_6 f_EUTRA_TestBody_Set(true); fl_TC_11_1_6_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble (eutra_Cell1, E2_CONNECTED); }

## f_TC_11_1_6_NR5GC

| Function name | f_TC_11_1_6_NR5GC |
| --- | --- |
| Reason for change | In the current TTCN implementation, after the NR Cell 1 is created, it is set at serving cell power immediately. Sometimes, this causes the testcase to fail because the UE may prefer to camp on NR Cell if its detected before Eutra Cell. According to preamble, the UE is first registered on Eutra. In order to address this instability, it is proposed to set the NR Cell to serving cell power after the coordination message from Eutra for “Preamble” is received. |
| Summary of change | Called the function f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); after the call to f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Preamble"); |
| TTCN module | EPS_Fallback_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> f_NR5GC_IRAT_Init(NR_6); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // First pass capability info to EUTRA f_NR5GC_IRAT_UECapabilityTransfer(nr_Cell1); // Wait until UE has attached on EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Preamble"); // Now register on NR // Now register on NR v_GMM_MobilityInfo := f_NR5GC_Preamble_Steps1_13(nr_Cell1, -, SUPPORTED); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> f_NR5GC_IRAT_Init(NR_6); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); //WA#11_1_6 f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // First pass capability info to EUTRA f_NR5GC_IRAT_UECapabilityTransfer(nr_Cell1); // Wait until UE has attached on EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Preamble"); // Now register on NR // Now register on NR f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //WA#11_1_6 moved here <<SKIPPED CODE>>

## fl_TC_11_1_6_NR5GC_TestBody

| Function name | fl_TC_11_1_6_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation, at Step 5 the TTCN checks if the IE UL Data Status is present in SERVICE REQ. This is not needed, as the UE may not have any UL data at this point, and the Prose also does not mention to check this. Hence it is proposed to relax the check for IE UL Data Status at Step 5. |
| Summary of change | Instead of setting ? for the IE UL Data Status, set it to *. |
| TTCN module | EPS_Fallback_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 5" siclog@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, ?, -, ?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 5" siclog@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, ?, -, *)) { //WA#11_1_6 ? --> * f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } <<SKIPPED CODE>>