3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220523

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0815 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of f_IMS_ThreeWaySession_Creation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In contrast to the prose in TTCN the same Contact URI is used for the first and second dialog during Three way session creation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | px_IMS_CalleeContactUri2 used for 2nd dialog in f_IMS_ThreeWaySession_Creation NOTE: The same change is done in the 5G sister function f_IMS_ThreeWaySession_NR5GC_Testbody. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN remains inconsistent with the prose. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.21a, 15.21c, G.15.20, G.15.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

function f_IMS_ThreeWaySession_Creation(IMS_TestProcedure_Type p_TestProcedure) runs on IMS_PTC { var charstring v_ConferenceFactoryUri := f_IMS_BuildConferenceUri(conferenceFactoryUri); // @sic R5-150745 sic@ var charstring v_TemporaryConferenceUri := f_IMS_BuildConferenceUri(temporaryConferenceUri); // @sic R5-150745 sic@ var charstring v_FinalConferenceUri := f_IMS_BuildConferenceUri(finalConferenceUri); // @sic R5-150745 sic@ var CharStringList_Type v_MediaTypeList := f_IMS_ConferenceEventPackage_MediaTypeList(p_TestProcedure); // @sic R5-214877: different media for speech call (audio) and video call (speech, video) sic@ var InviteUserToConference_StateInfo_Type v_Dialog1_StateInfo := valueof(cs_InviteUserToConference_StateInfo_Init(px_IMS_CalleeUri, px_IMS_CalleeContactUri)); var InviteUserToConference_StateInfo_Type v_Dialog2_StateInfo := valueof(cs_InviteUserToConference_StateInfo_Init(px_IMS_CalleeUri2, px_IMS_CalleeContactUri2)); var boolean v_RadioBearersEstablished := false; var template (omit) SUBSCRIBE_Request v_SubscribeRequest; // optional SUBSCRIBE for conference event package var ThreeWaySession_SessionInfo_Type v_SessionInfo_Dialog1; var ThreeWaySession_SessionInfo_Type v_SessionInfo_Dialog2; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp_Dialog3; var SipUrl v_ContactUrl_UE; //-------------------------------------------- // DIALOG 1 //-------------------------------------------- // Setup MO speech call f_IMS_CC_StartCall(p_TestProcedure, v_Dialog1_StateInfo.CalleeUri); // C.21/25 Step 1 v_SessionInfo_Dialog1 := fl_IMS_ThreeWaySession_CreateSession(p_TestProcedure, v_Dialog1_StateInfo.CalleeUri, v_Dialog1_StateInfo.CalleeContactUri, v_RadioBearersEstablished); v_RadioBearersEstablished := true; f_IMS_TestBody_Set(true); //Steps 1 to 4 - Messages in Annex C.8 - Set ongoing call on hold // @siclog "Step 1 - 4" siclog@ f_UT_CallHold(MMI); f_IMS_MOCallHoldResume(HOLD, v_SessionInfo_Dialog1.InviteRequestWithSdp.Invite, v_SessionInfo_Dialog1.MessageHeader_PrevResponse, v_SessionInfo_Dialog1.LatestSdpOffer, v_Dialog1_StateInfo.CalleeContactUri); f_IMS_PreliminaryPass(__FILE__, __LINE__, ""); //-------------------------------------------- // DIALOG 2 //-------------------------------------------- f_IMS_PTC_ImsInfo_ActivateDialog(secondDialog); f_IMS_PTC_ImsInfo_DialogSetSessIdTX("11111112"); // @sic R5-220212 sic@ // @siclog "Step 5" siclog@ Step 1 of C.21 select (p_TestProcedure) { // @sic R5s180053: AT command trigger added to start second call after holding 1st call sic@ case (IPCAN_MO_SpeechCall) { f_UT_RequestIMSCall(MMI, v_Dialog2_StateInfo.CalleeUri); } case (IPCAN_MO_VideoCall) { f_UT_RequestIMSVideoCall(MMI, v_Dialog2_StateInfo.CalleeUri); } } // @siclog "Step 6 - 17" siclog@ Steps defined in Annex C.21 - MO call v_SessionInfo_Dialog2 := fl_IMS_ThreeWaySession_CreateSession(p_TestProcedure, v_Dialog2_StateInfo.CalleeUri, v_Dialog2_StateInfo.CalleeContactUri, v_RadioBearersEstablished); f_IMS_PreliminaryPass(__FILE__, __LINE__, ""); // @siclog "Step 17A" siclog@ f_UT_CallMPTY(MMI); // There is no IPCAN signalling involved => f_IMS_CC_StartCall shall not be used // @siclog "Step 17B - 17E" siclog@ f_IMS_MOCallHoldResume(HOLD, v_SessionInfo_Dialog2.InviteRequestWithSdp.Invite, v_SessionInfo_Dialog2.MessageHeader_PrevResponse, v_SessionInfo_Dialog2.LatestSdpOffer, v_Dialog2_StateInfo.CalleeContactUri); //-------------------------------------------- // DIALOG 3 //-------------------------------------------- f_IMS_PTC_ImsInfo_ActivateDialog(thirdDialog); f_IMS_PTC_ImsInfo_DialogSetSessIdTX("11111113"); // @sic R5-220212 sic@ // @siclog "Step 18 - 30" siclog@ Steps defined in Annex C.10 v_InviteRequestWithSdp_Dialog3 := f_IMS_ConferenceCreation_AnnexC10C38_Steps2_9(p_TestProcedure, v_ConferenceFactoryUri, v_TemporaryConferenceUri, v_FinalConferenceUri, v_RadioBearersEstablished); v_ContactUrl_UE := f_MessageHeader_GetContactSipUrl(v_InviteRequestWithSdp_Dialog3.Invite.msgHeader); //-------------------------------------------- // Subscribe to conference event package (A.19 steps 10 - 13), invite user A/B (A.20), terminate dialog 1/2 //-------------------------------------------- // @siclog "Step 27 - 46D" siclog@ v_SubscribeRequest := f_IMS_ThreeWaySession_SubscribeInvite(v_SessionInfo_Dialog1, v_SessionInfo_Dialog2, v_Dialog1_StateInfo, v_Dialog2_StateInfo, v_FinalConferenceUri, v_MediaTypeList); /* @sic R5-213528: f_IMS_ThreeWaySession_SubscribeInvite sic@ @sic R5-214877: v_MediaTypeList sic@ */ f_IMS_PreliminaryPass(__FILE__, __LINE__, ""); //-------------------------------------------- // Release session //-------------------------------------------- // @siclog "Step 47 - 48" siclog@ Terminate conference f_IMS_CallReleaseMT(v_ContactUrl_UE); //-------------------------------------------- // Unsubscribe //-------------------------------------------- // @siclog "Step 49 - 50" siclog@ f_IMS_ConferenceEventPackage_Unsubscribe(v_SubscribeRequest, v_FinalConferenceUri); /* @sic R5-176937 - new steps sic@ */ f_IMS_TestBody_Set(false); }