3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220531

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0816 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS emergency call test case 10.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add Emergency Call test case 10.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to Emergency Call test case 10.2 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR TS34-229-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Prose CR is associated |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	7

2.3	Change 3	8

2.4	Change 4	9

3	Branches executed	10

4	Execution Log Files	10

4.1	Qualcomm SM 8450+SDX65 5G UE	10

5	References	10

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 10.2 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	10.2

ATS Version:	iwd-TTCN3-B2020-09_D22wk12

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR,

UE used:	Qualcomm SM 8450+SDX65 5G UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_10_1and2_NR5GC_IMS2 |
| --- | --- |
| Reason for change | - Test 10.2 is emergency call with registration therefore condition A7 is applied instead of A6 when calling f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1 function. - It is necessary to coordinate resource reservation (PDU session Modification) with the call establishment, otherwise depending on the timing, it could happen that resources reservation starts when the call is already released. - An incoming coordination from IPCAN is not handled after call is released - f_NR5GC_EmergencyCallRelease is waiting for a trigger from IMS component, that is currently not sending it. |
| Summary of change | - Changed condition A6 to A7 when calling f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1. - Added ‘WaitForTrigger’ from IPCAN (trigger added into f_NR5GC_EmergencyCallPDUSession – see change 2) so the call can be released only after IPCAN is done with resource reservation. - f_IMS_CC_EndCall – added to handle trigger from IPCAN - Added f_IMS_IPCAN_SendCoOrdMsg(IPCAN) to be handled by f_NR5GC_EmergencyCallRelease |
| TTCN module | IMS\NR5GC_10\IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_10_1and2_NR5GC_IMS2(boolean p_GeolocationUsage := false) runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var SipUrl v_ContactUrlUE; var IMS_DATA_REQ v_IMS_DATA_REQ; var REGISTER_Request v_RegisterRequest; var boolean v_CondA7 := true; var boolean v_GeolocationUsage := p_GeolocationUsage; /* the test case intends to use GeoLocationInformation if supported by the UE (PICS) INVITE (step 1 of Annex C.22) refers to condition A8 of A.2.1 */ f_IMS_PTC_Init(PDN_2); // Test Case Body f_IMS_TestBody_Set(true); f_IMS_IMS_WaitForTrigger(OtherPDN); // Steps 1-8 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed. // EXCEPTION: In parallel to the events described in steps 9-10 below the events specified in step of Table 4.9.11.2.2-2 of TS 38.508-1 [21] take place // Steps 9-10 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed. //EXCEPTION: In parallel to the events described in steps 12-20 below the events specified in 11-13 of annex Table 4.9.11.2.2-1 of TS 38.508-1[21] are performed. f_IMS_CC_StartCall(IPCAN_EmergencyCall_NoRegistration); // 4.9.11.2.2-2 of TS 38.508-1 //@siclog "Step 2" siclog@ //Step 1 of annex A.3 (emergency registration) //Check: Does the UE send a correctly composed initial REGISTER request for IMS emergency registration? v_IMS_DATA_REQ := f_IMS_REGISTER_InitialRequest(-, omit, -, -, -, v_CondA7); v_RegisterRequest := v_IMS_DATA_REQ.Request.Register; f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // Reconfig of UE client ports in case the UE uses UDP and different than default (5060) ports f_IMS_Register_SecurityInit(v_RegisterRequest); //@siclog "Step 3" siclog@ //Step 2 of annex A.3 (emergency registration) IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterRequest)))); //@siclog "Step 4" siclog@ //Step 3 of annex A.3 (emergency registration) //Check: Does the UE correctly authenticate itself by sending another REGISTER request with a correctly composed Authorization header using the AKAv1-MD5 algorithm? v_IMS_DATA_REQ := f_IMS_REGISTER_SubsequentRequest(-, -, v_CondA7); v_RegisterRequest := v_IMS_DATA_REQ.Request.Register; f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@siclog "Step 5" siclog@ //Step 4 of annex A.3 (emergency registration) v_RegisterRequest := v_IMS_DATA_REQ.Request.Register; IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterRequest, true)))); //@siclog "Step 6" siclog@ //Step 1 of annex A.6 (emergency call) //Check: Does the UE send a correctly composed INVITE request? v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(A_2_1_A6, v_GeolocationUsage); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); //@siclog "Step 7" siclog@ //Step 2 of annex A.6 (emergency call) //@siclog "Step 8" siclog@ //Step 3 of annex A.6 (emergency call) //@siclog "Step 9" siclog@ //Step 4 of annex A.6 (emergency call) //@siclog "Step 10" siclog@ //Step 5 of annex A.6 (emergency call) //Check: Does the UE send ACK? f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ, v_GeolocationUsage); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10"); //@siclog "Step 11" siclog@ //Step 1 of annex A.8 (MT Release of Voice Call) //@siclog "Step 12" siclog@ //Step 2 of annex A.8 (MT Release of Voice Call) //Check: Does the UE send 200 OK for the BYE request and ends the call? v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 12"); f_IMS_TestBody_Set(false); f_IMS_IMS_SendCoOrdMsg(OtherPDN); }

After Change

function f_TC_10_1and2_NR5GC_IMS2(boolean p_GeolocationUsage := false) runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var SipUrl v_ContactUrlUE; var IMS_DATA_REQ v_IMS_DATA_REQ; var REGISTER_Request v_RegisterRequest; var boolean v_CondA7 := true; var boolean v_GeolocationUsage := p_GeolocationUsage; /* the test case intends to use GeoLocationInformation if supported by the UE (PICS) INVITE (step 1 of Annex C.22) refers to condition A8 of A.2.1 */ f_IMS_PTC_Init(PDN_2); // Test Case Body f_IMS_TestBody_Set(true); f_IMS_IMS_WaitForTrigger(OtherPDN); // Steps 1-8 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed. // EXCEPTION: In parallel to the events described in steps 9-10 below the events specified in step of Table 4.9.11.2.2-2 of TS 38.508-1 [21] take place // Steps 9-10 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed. //EXCEPTION: In parallel to the events described in steps 12-20 below the events specified in 11-13 of annex Table 4.9.11.2.2-1 of TS 38.508-1[21] are performed. f_IMS_CC_StartCall(IPCAN_EmergencyCall_NoRegistration); // 4.9.11.2.2-2 of TS 38.508-1 //@siclog "Step 2" siclog@ //Step 1 of annex A.3 (emergency registration) //Check: Does the UE send a correctly composed initial REGISTER request for IMS emergency registration? v_IMS_DATA_REQ := f_IMS_REGISTER_InitialRequest(-, omit, -, -, -, v_CondA7); v_RegisterRequest := v_IMS_DATA_REQ.Request.Register; f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // Reconfig of UE client ports in case the UE uses UDP and different than default (5060) ports f_IMS_Register_SecurityInit(v_RegisterRequest); //@siclog "Step 3" siclog@ //Step 2 of annex A.3 (emergency registration) IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterRequest)))); //@siclog "Step 4" siclog@ //Step 3 of annex A.3 (emergency registration) //Check: Does the UE correctly authenticate itself by sending another REGISTER request with a correctly composed Authorization header using the AKAv1-MD5 algorithm? v_IMS_DATA_REQ := f_IMS_REGISTER_SubsequentRequest(-, -, v_CondA7); v_RegisterRequest := v_IMS_DATA_REQ.Request.Register; f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@siclog "Step 5" siclog@ //Step 4 of annex A.3 (emergency registration) v_RegisterRequest := v_IMS_DATA_REQ.Request.Register; IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterRequest, true)))); //@siclog "Step 6" siclog@ //Step 1 of annex A.6 (emergency call) //Check: Does the UE send a correctly composed INVITE request? v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(A_2_1_A7, v_GeolocationUsage); //A7 = with registration (A6 = without) f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); //@siclog "Step 7" siclog@ //Step 2 of annex A.6 (emergency call) //@siclog "Step 8" siclog@ //Step 3 of annex A.6 (emergency call) //@siclog "Step 9" siclog@ //Step 4 of annex A.6 (emergency call) //@siclog "Step 10" siclog@ //Step 5 of annex A.6 (emergency call) //Check: Does the UE send ACK? f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ, v_GeolocationUsage); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10"); f_IMS_IPCAN_WaitForTrigger(IPCAN); //@siclog "Step 11" siclog@ //Step 1 of annex A.8 (MT Release of Voice Call) //@siclog "Step 12" siclog@ //Step 2 of annex A.8 (MT Release of Voice Call) //Check: Does the UE send 200 OK for the BYE request and ends the call? v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_CC_EndCall(IPCAN_EmergencyCall_NormalService, IPCAN_SpeechAndEmergencyCall); //To receive coordination NR5GC f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 12"); f_IMS_TestBody_Set(false); // Inform NR that call is released (caught by f_NR5GC_EmergencyCallRelease) f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IMS_SendCoOrdMsg(OtherPDN); }

## Change 2

| Function name | f_NR5GC_EmergencyCallPDUSession |
| --- | --- |
| Reason for change | This is coordination counter-part for Change 1 bullet 2. |
| Summary of change | Added coordination trigger which is awaited by IMS component as per Change 1 bullet 2. |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment |  |

Before Change

function f_NR5GC_EmergencyCallPDUSession(NR_CellId_Type p_NR_CellId, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2, integer p_PDUEstAlreadyRxd := 0) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; f_NR5GC_PDUSessionEstablishment(p_NR_CellId, 1, p_SRB, -, // @sic R5s210518 sic@ omit, omit, crs_SSC_Mode(-, '001'B), // @sic R5s210518 sic@ -, f_BuildDefaultQoSRules("2"), -, -, -, -, -, -, crs_SSC_Mode('A'H, '001'B), // @sic R5s210518, R5s210771 sic@ -, -, -, -, -, -, -, cr_NG_Request_Type('011'B), -, -, -, -, -, -, -, -, "2", -, noRrcConnectionRelease, // @sic R5s210515 sic@ don't need the extra delay here as there's further IMS signalling p_PDUEstAlreadyRxd); // @sic R5-217286 sic@ // Get the Info for the PDU Session just created v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // Wait until need to add voice QoS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR5GC_ModifyPDUSession_AddVoice(p_NR_CellId, valueof(v_GSM_MobilityInfo)); // @sic R5s210518 sic@ }

After Change

function f_NR5GC_EmergencyCallPDUSession(NR_CellId_Type p_NR_CellId, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2, integer p_PDUEstAlreadyRxd := 0) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; f_NR5GC_PDUSessionEstablishment(p_NR_CellId, 1, p_SRB, -, // @sic R5s210518 sic@ omit, omit, crs_SSC_Mode(-, '001'B), // @sic R5s210518 sic@ -, f_BuildDefaultQoSRules("2"), -, -, -, -, -, -, crs_SSC_Mode('A'H, '001'B), // @sic R5s210518, R5s210771 sic@ -, -, -, -, -, -, -, cr_NG_Request_Type('011'B), -, -, -, -, -, -, -, -, "2", -, noRrcConnectionRelease, // @sic R5s210515 sic@ don't need the extra delay here as there's further IMS signalling p_PDUEstAlreadyRxd); // @sic R5-217286 sic@ // Get the Info for the PDU Session just created v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // Wait until need to add voice QoS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR5GC_ModifyPDUSession_AddVoice(p_NR_CellId, valueof(v_GSM_MobilityInfo)); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); // @sic R5s210518 sic@ }

## Change 3

| Function name | f_TC_10_2_NR5GC_IMS1 |
| --- | --- |
| Reason for change | The f_IMS_CC_Postamble (if called with TestProcedure parameter) is informing IPCAN to release PDU session, however that was already done from IMS2 and doing it again from IMS1 would lead to runtime error as there are no more PDU sessions. |
| Summary of change | f_IMS_CC_Postamble is not called with the TestProcedure parameter. |
| TTCN module | IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_10_2_NR5GC_IMS1() runs on IMS_PTC { // Emergency Call with emergency registration / Success / Location information not available / 5GS f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED); f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); // wait for trigger after IMS call release f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService); }

After Change

function f_TC_10_2_NR5GC_IMS1() runs on IMS_PTC { // Emergency Call with emergency registration / Success / Location information not available / 5GS f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED); f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); // wait for trigger after IMS call release f_IMS_CC_Postamble(); }

## Change 4

| Testcase name | TC_10_2_NR5GC |
| --- | --- |
| Reason for change | Adding and starting of IMS2 component is missing in testcase definition. |
| Summary of change | Added creation and start of IMS2. |
| TTCN module | IMS_Testsuite_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

testcase TC_10_2_NR5GC() runs on MTC_IMS system SYSTEM_IMS { // @purpose // Emergency Call with emergency registration / Success / Location information not available / 5GS var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(300); v_IMS1 := IMS_PTC.create("IMS1") alive; f_MTC_IMS_CreateMapAndConnectPTCs_NR5GC(system, v_IMS1, v_IMS2); v_IMS1.start(f_TC_10_2_NR5GC_IMS1()); t_GuardTimer.start; f_MTC_MainLoop_IMS(t_GuardTimer); }

After Change

testcase TC_10_2_NR5GC() runs on MTC_IMS system SYSTEM_IMS { // @purpose // Emergency Call with emergency registration / Success / Location information not available / 5GS var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(300); v_IMS1 := IMS_PTC.create("IMS1") alive; v_IMS2 := IMS_PTC.create("IMS2") alive; f_MTC_IMS_CreateMapAndConnectPTCs_NR5GC(system, v_IMS1, v_IMS2); v_IMS1.start(f_TC_10_2_NR5GC_IMS1()); v_IMS2.start(f_TC_10_2_NR5GC_IMS2()); t_GuardTimer.start; f_MTC_MainLoop_IMS(t_GuardTimer); }

# Branches executed

On Anritsu, the test case was executed on NR band n41 using NR ciphering nea1 and integrity algorithm nia1.

# Execution Log Files

## Qualcomm SM 8450+SDX65 5G UE

The Qualcomm SDX UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\ TC_10_2_LOG.xml

- PICS/PIXIT parameter file:

Anritsu\ TC_10_2_PIXIT.txt

# References

| [1] | R5s220532 - Supporting information for addition of IMS 5GS emergency call test case 10.2 |
| --- | --- |