3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220554

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2493 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC idle mode test case 6.2.1.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - IMS registration is not being handled at step 18A because the IMS PTC is trying to query information from the NR PTC instead of the EUTRA PTC. - A UE may send RRCSetupRequest on NR before the t_min timer has expired in TTCN, because the timer is started slightly later than the corresponding timer in the UE because of the delay following RRCConnectionRelease on EUTRA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Use function f_EUTRA_SwitchOnAndResetIMS() to reset IMS component and inform it that EUTRA IRAT is active. - Release the RRC connection after sending the trigger to NR PTC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_6_2_1_5_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | IMS registration is not being handled at step 18A because the IMS PTC is trying to query information from the NR PTC instead of the EUTRA PTC. |
| Summary of change | Use function f_EUTRA_SwitchOnAndResetIMS() to reset IMS component and inform it that EUTRA IRAT is active. |
| TTCN module | Idle_PLMNSelection_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_6_2_1_5_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { // @sic R5-204406, R5s200889 sic@ // The SS adjusts the NR and E-UTRAN Cell power levels according to row T0 f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE); // @siclog "Step 1" siclog@ // Power on the UE f_UT_SwitchOnUE(UT); // @siclog "Step 3-12b8" siclog@ // Steps 2-10b8 of the registration procedure described in TS 38.508-1 table 4.5.2.2-1 take place on E-UTRAN Cell 1 f_EUTRA38_NR5GC_Preamble(eutra_Cell1, TESTMode_OFF, RRC_IDLE); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger); // @siclog "Step 13-18 void" siclog@ // @siclog "Step 21-27a1" siclog@ f_EUTRA38_Send_IRATParametersToNR(eutra_Cell1); } }

After Change

function fl_TC_6_2_1_5_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { // @sic R5-204406, R5s200889 sic@ // The SS adjusts the NR and E-UTRAN Cell power levels according to row T0 f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE); // @siclog "Step 1" siclog@ // Power on the UE f_EUTRA_SwitchOnAndResetIMS(); // @siclog "Step 3-12b8" siclog@ // Steps 2-10b8 of the registration procedure described in TS 38.508-1 table 4.5.2.2-1 take place on E-UTRAN Cell 1 f_EUTRA38_NR5GC_Preamble(eutra_Cell1, TESTMode_OFF, RRC_IDLE); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger); // @siclog "Step 13-18 void" siclog@ // @siclog "Step 21-27a1" siclog@ f_EUTRA38_Send_IRATParametersToNR(eutra_Cell1); } }

## Change 2

| Function name | fl_TC_6_2_1_5_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | A UE may send RRCSetupRequest on NR before the t_min timer has expired in TTCN, because the timer is started slightly later than the corresponding timer in the UE because of the delay following RRCConnectionRelease on EUTRA. |
| Summary of change | Release the RRC connection after sending the trigger to NR PTC. |
| TTCN module | Idle_PLMNSelection_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_6_2_1_5_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { // @sic R5-204406, R5s200889 sic@ // The SS adjusts the NR and E-UTRAN Cell power levels according to row T0 f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE); // @siclog "Step 1" siclog@ // Power on the UE f_UT_SwitchOnUE(UT); // @siclog "Step 3-12b8" siclog@ // Steps 2-10b8 of the registration procedure described in TS 38.508-1 table 4.5.2.2-1 take place on E-UTRAN Cell 1 f_EUTRA38_NR5GC_Preamble(eutra_Cell1, TESTMode_OFF, RRC_IDLE); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger); // @siclog "Step 13-18 void" siclog@ // @siclog "Step 21-27a1" siclog@ f_EUTRA38_Send_IRATParametersToNR(eutra_Cell1); } }

After Change

function fl_TC_6_2_1_5_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { // @sic R5-204406, R5s200889 sic@ // The SS adjusts the NR and E-UTRAN Cell power levels according to row T0 f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE); // @siclog "Step 1" siclog@ // Power on the UE f_UT_SwitchOnUE(UT); // @siclog "Step 3-12b8" siclog@ // Steps 2-10b8 of the registration procedure described in TS 38.508-1 table 4.5.2.2-1 take place on E-UTRAN Cell 1 f_EUTRA38_NR5GC_Preamble(eutra_Cell1, TESTMode_OFF, RRC_IDLE, noRrcConnectionRelease); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger); f_EUTRA_RRC_ConnectionRelease(eutra_Cell1); // @siclog "Step 13-18 void" siclog@ // @siclog "Step 21-27a1" siclog@ f_EUTRA38_Send_IRATParametersToNR(eutra_Cell1); } }