3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220561

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2496 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 7.1.1.3.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the Table 7.1.1.3.9.3.3-3: PUSCH-Config (Table 7.1.1.3.9.3.3-2) The pusch-AggregationFactor should be set to n8 for ((TDD and SCS15) or FR2) In current TTCN implementation it is incorrectly set for ((FDD and SCS15) or FR2) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the TTCN thet pusch-AggregationFactor n8 will be used for ((TDD and SCS15) or FR2) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conformant UE may fail TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.9.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_9_1_Step24to29 |
| --- | --- |
| Reason for change | According to the Table 7.1.1.3.9.3.3-3: PUSCH-Config (Table 7.1.1.3.9.3.3-2) The pusch-AggregationFactor should be set to n8 for ((TDD and SCS15) or FR2) In current TTCN implementation it is incorrectly set for ((FDD and SCS15) or FR2) |
| Summary of change | Correct the TTCN thet pusch-AggregationFactor n8 will be used for ((TDD and SCS15) or FR2) |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_7_1_1_3_9_NR5GC_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { //Correct Handling of UL HARQ process / PUSCH Aggregation var template (value) CellGroupConfig v_CellGroupConfig; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; //@siclog "Steps 0A-0B" siclog@ //SS transmits in the indicated downlink assignment an NR RRCReconfiguration. (Note 1) //UE transmits NR RRCReconfigurationComplete message to the SS. (Note 2) //Configure pusch-AggregationFactor=n4 or n8 in UE and SS //Set pusch-AggregationFactor as per Table 7.1.1.3.9.3.3-3: PUSCH-Config (Table 7.1.1.3.9.3.3-2) if ((f_NR_CellInfo_GetIsFDD(nr_Cell1) and f_NR_CellInfo_GetSCS(nr_Cell1) == kHz15) or not(f_NR_CellInfo_GetIsFR1(nr_Cell1))) { v_NR_UplinkBWP.Dedicated.R15.pusch_Config.setup.pusch_AggregationFactor := n8; } else { v_NR_UplinkBWP.Dedicated.R15.pusch_Config.setup.pusch_AggregationFactor := n4; } f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP, tsc_NR_BWP_Id); v_CellGroupConfig := cs_38508_CellGroupConfig( tsc_NR_CellGroupId_MCG,-,-,-,-, cs_NR_SpCellConfigDef(omit, -, -, cs_NR_ServingCellConfigDef(-, -, -, cs_NR_UplinkConfigDef(v_NR_UplinkBWP.Dedicated.R15, omit)))); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration(nr_Cell1, -,-, v_V1530Ext); v_NR_UplinkBWP.Common := omit; //@sic R5s220332 sic@ f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, {v_NR_UplinkBWP})); // Configure MAC in TestMode - after signalling on SRBs //@sic R5s220332 sic@ v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_TC_7_1_1_3_9_NR_TestBody(p_NR_DRB_Id); }

After Change

function fl_TC_7_1_1_3_9_NR5GC_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { //Correct Handling of UL HARQ process / PUSCH Aggregation var template (value) CellGroupConfig v_CellGroupConfig; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; //@siclog "Steps 0A-0B" siclog@ //SS transmits in the indicated downlink assignment an NR RRCReconfiguration. (Note 1) //UE transmits NR RRCReconfigurationComplete message to the SS. (Note 2) //Configure pusch-AggregationFactor=n4 or n8 in UE and SS //Set pusch-AggregationFactor as per Table 7.1.1.3.9.3.3-3: PUSCH-Config (Table 7.1.1.3.9.3.3-2) if ((not f_NR_CellInfo_GetIsFDD(nr_Cell1) and f_NR_CellInfo_GetSCS(nr_Cell1) == kHz15) or not(f_NR_CellInfo_GetIsFR1(nr_Cell1))) { v_NR_UplinkBWP.Dedicated.R15.pusch_Config.setup.pusch_AggregationFactor := n8; } else { v_NR_UplinkBWP.Dedicated.R15.pusch_Config.setup.pusch_AggregationFactor := n4; } f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP, tsc_NR_BWP_Id); v_CellGroupConfig := cs_38508_CellGroupConfig( tsc_NR_CellGroupId_MCG,-,-,-,-, cs_NR_SpCellConfigDef(omit, -, -, cs_NR_ServingCellConfigDef(-, -, -, cs_NR_UplinkConfigDef(v_NR_UplinkBWP.Dedicated.R15, omit)))); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration(nr_Cell1, -,-, v_V1530Ext); v_NR_UplinkBWP.Common := omit; //@sic R5s220332 sic@ f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, {v_NR_UplinkBWP})); // Configure MAC in TestMode - after signalling on SRBs //@sic R5s220332 sic@ v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_TC_7_1_1_3_9_NR_TestBody(p_NR_DRB_Id); }