3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220614

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2524 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC emergency call test case 11.4.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During step 12-13 for the release of all active PDU sessions, the variable v_PDUSessionInfo is passed as an “inout” parameter to the function f_Check_NG_PDUSessionReleaseComplete(), and is updated inside this function to remove the PDU session which has just been released. This means that after releasing the first PDU session, the 2nd one will now be in position [0] in the updated list but the test case willl try to access position [1], leading to a run-time error. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the loop for steps 12-13 to always access index 0 of variable v_PDUSessionInfo, because the PDU session information will be updated to remove session information on each loop iteration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Execution of this test case can give a run-time error if there is more than 1 PU session to release at steps 12-13. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_11_4_4_NR5GC_TestBody() |
| --- | --- |
| Reason for change | During step 12-13 for the release of all active PDU sessions, the variable v_PDUSessionInfo is passed as an “inout” parameter to the function f_Check_NG_PDUSessionReleaseComplete(), and is updated inside this function to remove the PDU session which has just been released. This means that after releasing the first PDU session, the 2nd one will now be in position [0] in the updated list but the test case willl try to access position [i=1], leading to a run-time error. |
| Summary of change | Update the loop for steps 12-13 to always access index 0 of variable v_PDUSessionInfo, because the PDU session information will be updated to remove session information on each loop iteration. |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_11_4_4_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SRB_COMMON_IND v_ReceivedAsp; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; ………………………… // @siclog "Step 12" siclog@ for (i := 0; i<v_NoOfSessions; i:=i+1) { // @sic R5s210515 R5-216200 sic@ v_PDUSessionIdToRelease := v_PDUSessionInfo[i].SessionId; v_MsgToSend := f_Get_NG_PDUSessionReleaseCommand(v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00011010'B), //5GSM cause Insufficient resources cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00101'B)); //T3396 = 5 minutes // p_NGSM_CongestionReattemptInd, SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB2, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend)));//@sic R5s210808sic@ SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell11, tsc_NR_RbId_SRB2, //@sic R5s210808sic@ cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; v_PDN_Index := f_Get_PDN_IndexForPDUSessionId (v_PDUSessionInfo, v_PDUSessionIdToRelease); //@sic R5-220042 sic@ if (isvalue(v_PDN_Index)) { //@sic R5-220042 sic@ f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(valueof(v_PDN_Index))); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Cannot find PDN Index of PDU Session being released"); } if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToRelease)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Complete failed"); } } …………………. }

After Change:

function fl_TC_11_4_4_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SRB_COMMON_IND v_ReceivedAsp; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; …………………….. // @siclog "Step 12" siclog@ for (i := 0; i<v_NoOfSessions; i:=i+1) { // @sic R5s210515 R5-216200 sic@ v_PDUSessionIdToRelease := v_PDUSessionInfo[0].SessionId; v_MsgToSend := f_Get_NG_PDUSessionReleaseCommand(v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00011010'B), //5GSM cause Insufficient resources cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00101'B)); //T3396 = 5 minutes // p_NGSM_CongestionReattemptInd, SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB2, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend)));//@sic R5s210808sic@ SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell11, tsc_NR_RbId_SRB2, //@sic R5s210808sic@ cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; v_PDN_Index := f_Get_PDN_IndexForPDUSessionId (v_PDUSessionInfo, v_PDUSessionIdToRelease); //@sic R5-220042 sic@ if (isvalue(v_PDN_Index)) { //@sic R5-220042 sic@ f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(valueof(v_PDN_Index))); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Cannot find PDN Index of PDU Session being released"); } if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToRelease)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Complete failed"); } } ………………… }