3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220623

# Online, , 21st Feb 2022 – 3rd Mar 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2222 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 11.1.5 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Datang LinkTester |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-6 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add Rel-15 NR5GC test case 11.1.5 to the NR5GC ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 11.1.5 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	6

2.3	Change 3	7

2.4	Change 4	9

2.5	Change 5	12

2.6	Change 6	18

2.7	Change 7	19

2.8	Change 8	20

3	Branches executed	24

4	Execution Log Files	24

4.1	HiSilicon Balong 5000	24

5	References	24

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.1.5 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Pengchong Shen
	shenpengchong@datangmobile.cn

## Verification Test Summary

Test Case: 	11.1.5

ATS Version:	iwd-TTCN3-B2020-09_D21wk37

System Simulator used:	Datang LinkTester 5G Protocol Conformance Test System ECT9610

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | fl_TC_11_1_5_NR5GC_TestBody |
| --- | --- |
| Reason for change | - The cause of service triggered by pagging should no be mo-data. - In order to cooperate with the fl_TAUReject_AttachWithoutN26 function on the LTE side, only GutiParameters, PduSessionInfo and UECapability should be sent. |
| Summary of change | - The second and third parameters of f_NR5GC_RRCIdleToConnected_Steps2_4 uses the default value. - Use f_NR5GC_Send_GutiParametersToEUTRA, f_NR5GC_SendPduSessionInfoListToEUTRA and f_NR5GC_IRAT_UECapabilityTransfer instead of f_NR5GC_TrackingAreaUpdateFromN1. |
| TTCN module | EPS_Fallback_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_11_1_5_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); //@siclog "Step 2" siclog@ f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Steps 3 - 9" siclog@ v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(nr_Cell1, '0001'B, mo_Data); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); // @siclog "Step 9A" siclog@ f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5-210133 sic@ f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Change Power Level")); // @sic R5-216190 sic@ //@siclog "Step 10" siclog@ f_UT_RequestIMSCall(UT, px_IMS_CalleeUri); //@siclog "Step 11" siclog@ //Step 1 of TS 34.229-5 Annex A.9.1, done on IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //@siclog "Step 13" siclog@ //The SS transmits an RRCRelease message (IE redirectedCarrierInfo including EUTRA cells). v_RedirectedCarrierInfo := cs_NR_RedirectedCarrierInfoToEUTRA(v_CoOrd_SysInfo.Eutra[0].Arfcn); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCRelease(tsc_NR_RRC_TI_Def, -, v_RedirectedCarrierInfo))); f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetNextSendOccasion_DL(nr_Cell1)); // Send both authentication and NR sysinfo paramters to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); }

After Change

function fl_TC_11_1_5_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); //@siclog "Step 2" siclog@ f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Steps 3 - 9" siclog@ v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(nr_Cell1); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); // @siclog "Step 9A" siclog@ f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5-210133 sic@ f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Change Power Level")); // @sic R5-216190 sic@ //@siclog "Step 10" siclog@ f_UT_RequestIMSCall(UT, px_IMS_CalleeUri); //@siclog "Step 11" siclog@ //Step 1 of TS 34.229-5 Annex A.9.1, done on IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //@siclog "Step 13" siclog@ //The SS transmits an RRCRelease message (IE redirectedCarrierInfo including EUTRA cells). v_RedirectedCarrierInfo := cs_NR_RedirectedCarrierInfoToEUTRA(v_CoOrd_SysInfo.Eutra[0].Arfcn); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCRelease(tsc_NR_RRC_TI_Def, -, v_RedirectedCarrierInfo))); f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetNextSendOccasion_DL(nr_Cell1)); // Send both authentication and NR sysinfo paramters to EUTRA f_NR5GC_Send_GutiParametersToEUTRA(nr_Cell1); f_NR5GC_SendPduSessionInfoListToEUTRA(); f_NR5GC_IRAT_UECapabilityTransfer(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); }

## Change 2

| Function name | f_TC_11_1_5_NR5GC_EUTRA |
| --- | --- |
| Reason for change | - The MaxReferencePower of cell 3 should be set to -73. |
| Summary of change | - Add f_EUTRA_CellInfo_InitMaxReferencePower. |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_11_1_5_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { ////MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with redirection / Single registration mode without N26 interface / E-UTRAN cell reselection using cell status reservation / Success f_EUTRA38_IRAT_Init(c31, HANDLE_UM_DATA); // @sic R5s210414 sic@ //Create Eutra Cell 2 and set it as reserved for operator use and modify Sysinfo updated f_EUTRA_CellInfo_SetSysInfo_CellReservedForOpUse (eutra_Cell3, reserved); // @sic R5-216189 sic@ //Create and configure cells f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_CellConfig_Def(eutra_Cell3); // @sic R5-216189 sic@ f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_11_1_5_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E2_CONNECTED); }

After Change

function f_TC_11_1_5_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { ////MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with redirection / Single registration mode without N26 interface / E-UTRAN cell reselection using cell status reservation / Success f_EUTRA38_IRAT_Init(c31, HANDLE_UM_DATA); // @sic R5s210414 sic@ //Create Eutra Cell 2 and set it as reserved for operator use and modify Sysinfo updated f_EUTRA_CellInfo_SetSysInfo_CellReservedForOpUse (eutra_Cell3, reserved); // @sic R5-216189 sic@ f_EUTRA_CellInfo_InitMaxReferencePower (eutra_Cell3, -73); //Create and configure cells f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_CellConfig_Def(eutra_Cell3); // @sic R5-216189 sic@ f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_11_1_5_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E2_CONNECTED); }

## Change 3

| Function name | fl_TC_11_1_5_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | - According to 38.523-1 table 11.1.5 3.2-2. It should to add PDN CONNECTIVITY REQUEST process. - According to 38.523-1 table 11.1.5 3.2-2. The eutraFrequency in the RRC Release should be cell 3. - The power of cell1 should be set to -85 in step 9A. - It should to ensure that the ‘100 trying’ is sent after setp27. |
| Summary of change | - Add PDN CONNECTIVITY REQUEST process after step 27. - Transmit the frequency information of cell3 at the beginning of the test body. - Add cell1 power modify setp in seto 9A. - Add f_IMS_IPCAN_SendCoOrdMsg after setp 27. |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_11_1_5_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { // Send EUTRA frequency to NR and wait for NR info f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Change Power Level"); // @sic R5-216190 sic@ // @siclog "Step 9A" siclog@ f_EUTRA38_SetCellPower(eutra_Cell3, tsc_CellRS_EPRE_73, tsc_CellRS_EPRE_73); // @siclog "Step 14 - 27" siclog@ fl_TAUReject_AttachWithoutN26 (); // @siclog "Step 30 -32" siclog@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // Test body finishes on IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

After Change

function fl_TC_11_1_5_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var EPS_BearerIdentity v_EpsBearerId; var SRB_COMMON_IND v_ReceivedAsp; timer t_wait := 5.0; // Send EUTRA frequency to NR and wait for NR info f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell3})); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Change Power Level"); // @sic R5-216190 sic@ // @siclog "Step 9A" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE_FR2); f_EUTRA38_SetCellPower(eutra_Cell3, tsc_CellRS_EPRE_73, tsc_CellRS_EPRE_73); // @siclog "Step 14 - 27" siclog@ fl_TAUReject_AttachWithoutN26 (); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); alt { [] SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_Handover, ?)))) -> value v_ReceivedAsp { v_EpsBearerId := f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2 (eutra_Cell1, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.pDN_CONNECTIVITY_REQUEST); a_EUTRA_IdleUpdated_ConnectToAdditionalPDN_Step3(eutra_Cell1); a_EUTRA38_MultiPDN_ConnectToAdditionalPDN_Step4(eutra_Cell1, v_EpsBearerId); } [] t_wait.timeout { } } // @siclog "Step 30 -32" siclog@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // Test body finishes on IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

## Change 4

| Function name | f_TC_11_1_5_NR5GC_IMS1 |
| --- | --- |
| Reason for change | - After INVITE, it should to send CoOrdMsg trigger RRCRelease message. - Missing Response183 sending steps. - Modify with change 3.4 |
| Summary of change | - Add f_IMS_IPCAN_SendCoOrdMsg after step 11. - Add f_IMS_MOCallSetup_AnnexC21_Step4 in step 28-29. - Add f_IMS_IPCAN_WaitForTrigger after step 27. |
| TTCN module | EPS_Fallback_NR5GC_IMS.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_11_1_5_NR5GC_IMS1()runs on IMS_PTC { var IMS_IPCAN_Coordination_MSG v_IMS_IPCAN_CoordMsg; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var template (value) MessageHeader v_MessageHeader_Response183; var default v_MyDefaultVar := null; fl_IMSFallback_PreambleRegistrations(); // @siclog "Step 11" siclog@ v_InviteRequestWithSdp := f_IMS_MOCallSetup_ReceiveINVITE_Common(-, -); // Wait for indication RAT has changed to EUTRA OtherIPCAN.receive(cmr_IPCAN_IMS_Response) -> value v_IMS_IPCAN_CoordMsg; f_IMS_PTC_SetRanType(v_IMS_IPCAN_CoordMsg.IPCAN_IMS_Response.IpcanInfo.RanType); //@siclog "Step 28 - 29" siclog@ v_MyDefaultVar := activate (a_C46_ParallelBehavior ()); //Steps 28 - 29 Map to 36.508 Table 4.5A.6.3-1 Steps 9a1- 11a1 which maps to 34.229-1 C.21 Steps 3 -4 f_IMS_MOCallSetup_Send100Trying(v_InviteRequestWithSdp); v_MessageHeader_Response183 := f_IMS_InviteResponse_183_MessageHeaderTX(v_InviteRequestWithSdp.Invite, px_IMS_CalleeContactUri, omit, -, NORMAL_CALL); deactivate (v_MyDefaultVar); // Trigger EUTRA for Dedicated Bearer Setup f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); //Carry on with the C.21 Steps 5-13 f_IMS_MOCallSetup_AnnexC21_Steps5_13(v_InviteRequestWithSdp, valueof(v_MessageHeader_Response183)); // @siclog "Step 36" siclog@ f_Delay(1.0); // @siclog "Step 37" siclog@ f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); f_IMS_IpcanReleaseWithOptionalDeregistrationOtherIPCAN(); // @sic R5s210662 sic@ }

After Change

function f_TC_11_1_5_NR5GC_IMS1()runs on IMS_PTC { var IMS_IPCAN_Coordination_MSG v_IMS_IPCAN_CoordMsg; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var template (value) MessageHeader v_MessageHeader_Response183; var default v_MyDefaultVar := null; fl_IMSFallback_PreambleRegistrations(); // @siclog "Step 11" siclog@ v_InviteRequestWithSdp := f_IMS_MOCallSetup_ReceiveINVITE_Common(-, -); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); // Wait for indication RAT has changed to EUTRA OtherIPCAN.receive(cmr_IPCAN_IMS_Response) -> value v_IMS_IPCAN_CoordMsg; f_IMS_PTC_SetRanType(v_IMS_IPCAN_CoordMsg.IPCAN_IMS_Response.IpcanInfo.RanType); f_IMS_IPCAN_WaitForTrigger(OtherIPCAN); //@siclog "Step 28 - 29" siclog@ v_MyDefaultVar := activate (a_C46_ParallelBehavior ()); //Steps 28 - 29 Map to 36.508 Table 4.5A.6.3-1 Steps 9a1- 11a1 which maps to 34.229-1 C.21 Steps 3 -4 f_IMS_MOCallSetup_Send100Trying(v_InviteRequestWithSdp); v_MessageHeader_Response183 := f_IMS_InviteResponse_183_MessageHeaderTX(v_InviteRequestWithSdp.Invite, px_IMS_CalleeContactUri, omit, -, NORMAL_CALL); f_IMS_MOCallSetup_AnnexC21_Step4(v_InviteRequestWithSdp, v_MessageHeader_Response183); deactivate (v_MyDefaultVar); // Trigger EUTRA for Dedicated Bearer Setup f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); //Carry on with the C.21 Steps 5-13 f_IMS_MOCallSetup_AnnexC21_Steps5_13(v_InviteRequestWithSdp, valueof(v_MessageHeader_Response183)); // @siclog "Step 36" siclog@ f_Delay(1.0); // @siclog "Step 37" siclog@ f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); f_IMS_IpcanReleaseWithOptionalDeregistrationOtherIPCAN(); // @sic R5s210662 sic@ }

## Change 5

| Function name | fl_TAUReject_AttachWithoutN26 |
| --- | --- |
| Reason for change | - According to 24.301, cl. 5.5.1.2.2, the UE shall include the valid GUTI into the EPS mobile identity IE, not mapped NG_GUTI. |
| Summary of change | - Change the forth parameter of cdr_ATTACH_REQUEST_N1ToS1 to ‘v_GutiParams’. |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TAUReject_AttachWithoutN26 () runs on EUTRA_Multi5GS_PTC { var SRB_COMMON_IND v_ReceivedAsp; var NAS_MSG_Indication_Type v_NasInd; var template (present) NAS_MSG_Indication_Type v_NAS_Expected; var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; var template (value) MobileIdentity v_MappedGUTI; var GutiParameters_Type v_GutiParams; var template AdditionalUpdateType v_AdditionalUpdateType; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_Tac; var template TrackingAreaId v_TAI:= omit; var template (present) NAS_KsiValue v_KsiValue; var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList; var template (omit) integer v_SessionId; v_CoOrd_Guti := f_EUTRA_NR_WaitForCoOrd_GutiParams(NR); v_MappedGUTI := f_EUTRA38_NR_Convert5G_CoOrdToEPS_GUTI (v_CoOrd_Guti, omit); v_PduSessionInfoList := f_EUTRA38_WaitForPDUSessionInfoList(eutra_Cell1); // @sic R5s210149 sic@ // Tell IMS PTC the RAT has changed f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_EUTRA_IMS_IpcanInfo())); v_AdditionalUpdateType := f_GetAdditionalUpdateType (NORMAL); v_KsiValue := f_EUTRA_SecurityKSIasme_Get(); v_GutiParams:= f_EUTRA_CellInfo_GetGuti(eutra_Cell1); v_PLMN := f_Asn2Nas_PlmnId(v_GutiParams.PLMN_Identity); v_Tac := f_EUTRA_CellInfo_GetTAC(eutra_Cell1); v_TAI := cr_TAI(v_PLMN, bit2oct(v_Tac)); // @siclog "Step 7 - 8" siclog@ v_NAS_Expected := (cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_TAU_Request_N1ToS1(f_GetEPSTAUType(NORMAL), v_KsiValue, cr_DRXparameter_Any ('5C'O) ifpresent, v_AdditionalUpdateType, v_MappedGUTI, v_TAI, tsc_EpsUpdate_Active, -, -, omit, cr_UeRadioCap ifpresent)), cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected, cdr_ATTACH_REQUEST_N1ToS1(f_GetEAttachType(NORMAL), v_AdditionalUpdateType, v_KsiValue, v_MappedGUTI, v_TAI), cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_Handover, f_GetPdnType()))); // @siclog "Step 8a1/8b1" siclog@ v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_Cell1, tsc_RRC_TI_Def, ?, v_NAS_Expected); if (ischosen (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST)) { // @siclog "Step 8b2" siclog@ SRB.send(cas_SRB1_NasPdu_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_NoSecurityProtection, cs_508_TRACKING_AREA_UPDATE_REJECT(tsc_EmmCause_UEIdentityNotDerived)))); // @siclog "Step 8b3" siclog@ SRB.receive(car_SRB1_NasPdu_IND(eutra_Cell1, cr_NAS_IndicationWithPiggybacking((tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected), cdr_ATTACH_REQUEST_IMSIonly(f_GetEAttachType(NORMAL), v_AdditionalUpdateType, v_TAI), cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_Handover, f_GetPdnType())))) -> value v_ReceivedAsp; v_NasInd := v_ReceivedAsp.Signalling.Nas[0]; } // @siclog "Step 9 - 20" siclog@ v_SessionId := f_ProtocolConfigOptions_GetSessionId(v_NasInd.Pdu.PiggybackedPduList[0].Msg.pDN_CONNECTIVITY_REQUEST.protocolConfigurationOptions); if (not isvalue (v_SessionId)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Id not present in PCO"); } f_EUTRA38_MultiPDN_InitialRegistration_Step5_16 (eutra_Cell1, TESTMode_OFF, v_NasInd, -, v_PduSessionInfoList); // @sic R5s210149 sic@ }

After Change

function fl_TAUReject_AttachWithoutN26 () runs on EUTRA_Multi5GS_PTC { var SRB_COMMON_IND v_ReceivedAsp; var NAS_MSG_Indication_Type v_NasInd; var template (present) NAS_MSG_Indication_Type v_NAS_Expected; var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; var template (value) MobileIdentity v_MappedGUTI; var GutiParameters_Type v_GutiParams; var template AdditionalUpdateType v_AdditionalUpdateType; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_Tac; var template TrackingAreaId v_TAI:= omit; var template (present) NAS_KsiValue v_KsiValue; var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList; var template (omit) integer v_SessionId; v_CoOrd_Guti := f_EUTRA_NR_WaitForCoOrd_GutiParams(NR); v_MappedGUTI := f_EUTRA38_NR_Convert5G_CoOrdToEPS_GUTI (v_CoOrd_Guti, omit); v_PduSessionInfoList := f_EUTRA38_WaitForPDUSessionInfoList(eutra_Cell1); // @sic R5s210149 sic@ // Tell IMS PTC the RAT has changed f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_EUTRA_IMS_IpcanInfo())); v_AdditionalUpdateType := f_GetAdditionalUpdateType (NORMAL); v_KsiValue := f_EUTRA_SecurityKSIasme_Get(); v_GutiParams:= f_EUTRA_CellInfo_GetGuti(eutra_Cell1); v_PLMN := f_Asn2Nas_PlmnId(v_GutiParams.PLMN_Identity); v_Tac := f_EUTRA_CellInfo_GetTAC(eutra_Cell1); v_TAI := cr_TAI(v_PLMN, bit2oct(v_Tac)); // @siclog "Step 7 - 8" siclog@ v_NAS_Expected := (cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_TAU_Request_N1ToS1(f_GetEPSTAUType(NORMAL), v_KsiValue, cr_DRXparameter_Any ('5C'O) ifpresent, v_AdditionalUpdateType, v_MappedGUTI, v_TAI, tsc_EpsUpdate_Active, -, -, omit, cr_UeRadioCap ifpresent)), cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected, cdr_ATTACH_REQUEST_N1ToS1(f_GetEAttachType(NORMAL), v_AdditionalUpdateType, v_KsiValue, f_GutiParameters2MobileIdentity(omit, v_GutiParams), v_TAI), cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_Handover, f_GetPdnType()))); // @siclog "Step 8a1/8b1" siclog@ v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_Cell1, tsc_RRC_TI_Def, ?, v_NAS_Expected); if (ischosen (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST)) { // @siclog "Step 8b2" siclog@ SRB.send(cas_SRB1_NasPdu_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_NoSecurityProtection, cs_508_TRACKING_AREA_UPDATE_REJECT(tsc_EmmCause_UEIdentityNotDerived)))); // @siclog "Step 8b3" siclog@ SRB.receive(car_SRB1_NasPdu_IND(eutra_Cell1, cr_NAS_IndicationWithPiggybacking((tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected), cdr_ATTACH_REQUEST_IMSIonly(f_GetEAttachType(NORMAL), v_AdditionalUpdateType, v_TAI), cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_Handover, f_GetPdnType())))) -> value v_ReceivedAsp; v_NasInd := v_ReceivedAsp.Signalling.Nas[0]; } // @siclog "Step 9 - 20" siclog@ v_SessionId := f_ProtocolConfigOptions_GetSessionId(v_NasInd.Pdu.PiggybackedPduList[0].Msg.pDN_CONNECTIVITY_REQUEST.protocolConfigurationOptions); if (not isvalue (v_SessionId)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Id not present in PCO"); } f_EUTRA38_MultiPDN_InitialRegistration_Step5_16 (eutra_Cell1, TESTMode_OFF, v_NasInd, -, v_PduSessionInfoList); // @sic R5s210149 sic@ }

## Change 6

| Function name | cdr_ATTACH_REQUEST_N1ToS1 |
| --- | --- |
| Reason for change | - According to 24.301, cl. 5.5.1.2.2, the UE shall include the valid GUTI into the EPS mobile identity IE. |
| Summary of change | - Instead additionalGuti to epsMobileIdentity. |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

template (present) NAS_UL_Message_Type cdr_ATTACH_REQUEST_N1ToS1(template (present) NAS_AttDetValue_Type p_AttachTypeValue, template AdditionalUpdateType p_AdditionalUpdateType, template (present) NAS_KsiValue p_KsiValue, template (present) MobileIdentity p_Guti, template TrackingAreaId p_TAI) modifies cr_508_ATTACH_REQUEST := { // FFS: OldGUTI is not included in this message definition! aTTACH_REQUEST := { nasKeySetId := cr_NAS_KeySetIdentifier(p_KsiValue, tsc_Spare1), additionalGuti := p_Guti, lastVisitedRegisteredTai := p_TAI, oldGUTI_Type := cr_GUTI_Type_Native } }

After Change

template (present) NAS_UL_Message_Type cdr_ATTACH_REQUEST_N1ToS1(template (present) NAS_AttDetValue_Type p_AttachTypeValue, template AdditionalUpdateType p_AdditionalUpdateType, template (present) NAS_KsiValue p_KsiValue, template MobileIdentity p_Guti, template TrackingAreaId p_TAI) modifies cr_508_ATTACH_REQUEST := { // FFS: OldGUTI is not included in this message definition! aTTACH_REQUEST := { nasKeySetId := cr_NAS_KeySetIdentifier(p_KsiValue, tsc_Spare1), epsMobileIdentity := p_Guti, lastVisitedRegisteredTai := p_TAI, oldGUTI_Type := cr_GUTI_Type_Native } }

## Change 7

| Function name | cr_PDN_CONNECTIVITY_REQUEST_N1ToS1 |
| --- | --- |
| Reason for change | - According to 38.523-1 table 11.1.5 3.3-7 and 36.508 table 4.7.2-20. ESM information transfer flag should be set to ‘Not present or any allowed value’. |
| Summary of change | - Change the fifth parameter of cr_PDN_CONNECTIVITY_REQUEST_Common to ‘*’ |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

template (present) NAS_UL_Message_Type cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(template (present) B3_Type p_RequestTypeValue, template (present) PdnTypeValue p_PDNTypeValue, template ProtocolConfigOptions p_PCO := cr_ProtocolConfigOptionsAny ifpresent, template ExtdProtocolConfigOptions p_ExtdProtocolConfigOptions := cr_ExtdPCOAny ifpresent) := cr_PDN_CONNECTIVITY_REQUEST_Common ('0'H, ?, p_PDNTypeValue, p_RequestTypeValue, -, -, p_PCO, -, -, -, p_ExtdProtocolConfigOptions);

After Change

template (present) NAS_UL_Message_Type cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(template (present) B3_Type p_RequestTypeValue, template (present) PdnTypeValue p_PDNTypeValue, template ProtocolConfigOptions p_PCO := cr_ProtocolConfigOptionsAny ifpresent, template ExtdProtocolConfigOptions p_ExtdProtocolConfigOptions := cr_ExtdPCOAny ifpresent) := cr_PDN_CONNECTIVITY_REQUEST_Common ('0'H, ?, p_PDNTypeValue, p_RequestTypeValue, *, -, p_PCO, -, -, -, p_ExtdProtocolConfigOptions);

## Change 8

| Function name | f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2_FB |
| --- | --- |
| Reason for change | - In EPS Fall Back, The internet type IP link already exists, it should use the f_IP_ChangeDrbMappingEUTRA function to establish the IP link. |
| Summary of change | - Add new function f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2_FB |
| TTCN module | EUTRA38_NR5GC_InitialRegistration.ttcn |
| MCC160 Comment |  |

New function

function f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2_FB(EUTRA_CellId_Type p_CellId, PDN_CONNECTIVITY_REQUEST p_PDNConnReq, boolean p_ConfigureSRB2 := false, template (omit) NasCount_Type p_NasCountUL := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromNR := omit) runs on EUTRA_Multi5GS_PTC return EPS_BearerIdentity { var ProcedureTransactionIdentifier v_PTId_UE; var template (omit) ProtocolConfigOptions v_PcoFromUE; var template (value) ProtocolConfigOptions v_PcoToUE; var template (omit) ExtdProtocolConfigOptions v_ExtdPcoFromUE := omit; var template (value) AccessPointName v_APN_Tx := f_EUTRA38_MultiPDN_GetAPN(p_CellId, p_PDNConnReq.accessPointName); var PDU_PDN_DNN_Type v_PDN_Type; var PDN_Index_Type v_PdnIndex; var boolean v_IsIMS := false; var EPS_BearerIdentity v_EpsDefaultBearerId; var PDN_Address v_PDN_Address; var BEARER_CONTEXT_TYPE v_BearerContextNumber; var template (omit) ESM_Cause v_ESM_Cause := omit; // @sic R5s170882 sic@ var PDN_TypeToBeUsed_Type v_PDN_TypeToBeUsed := f_EUTRA_MobileInfo_GetPdnTypeToBeUsed(); /* @sic R5s170882 sic@ */ var template (omit) SRB_ToAddModList v_SRB_ToAddModList := omit; var template (value) EUTRA38_DRBInfoList_Type v_NewDRBs := {}; var template (value) RadioBearerList_Type v_SS_ConfigList; var template (value) DRB_ToAddModList v_DRB_ToAddModList; var EUTRA38_DRBInfoList_Type v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); var integer v_NewDRBId; var EUTRA_SecurityParams_Type v_AuthParams := f_EUTRA_Security_Get(); var boolean v_StartPDN := true; // @sic R5s210149 sic@ var integer i; v_PTId_UE := p_PDNConnReq.procedureTransactionIdentifier; v_PDN_Type := f_EUTRA38_MultiPDN_GetPDNType(p_PDNConnReq.accessPointName); /* get APN without any ESM information transfer @sic R5-206296 sic@ */ v_PcoFromUE := p_PDNConnReq.protocolConfigurationOptions; v_ExtdPcoFromUE := p_PDNConnReq.extdProtocolConfigurationOptions; v_PdnIndex := f_GetMultiPdnIndex (v_PDN_Type); // @sic R5-206296 sic@ v_PDN_Address := f_EUTRA_GetPDNAddress(f_CheckPCOforIPallocationViaNas(v_PcoFromUE), v_PdnIndex); select(v_PDN_Type) { // @sic R5-206296 sic@ case (Internet_DNN) { v_BearerContextNumber := DEF_1; } case (IMS_DNN, Emergency_PDN) { // @sic R5s210149 sic@ v_IsIMS := true; v_BearerContextNumber := DEF_2; } } // get new default DRB and add to existing list @sic R5-206289 sic@ v_NewDRBId := f_EUTRA38_GetNextDRBId (v_PDN_Type, true); v_EpsDefaultBearerId := int2hex(v_NewDRBId + 4, 1); v_NewDRBs[0] := cs_DRB_Info (v_NewDRBId, hex2int(v_EpsDefaultBearerId), true, cs_OneDRB_ConfigAM(v_NewDRBId), cs_508_DRB_ToAddMod_DEFAULT_AM(v_NewDRBId), v_PDN_Type); v_ExistingDRBs := f_EUTRA38_DRBInfo_AddListOfEntries (valueof(v_ExistingDRBs), valueof(v_NewDRBs)); f_EUTRA38_MobileInfo_SetDRBInfoList (valueof(v_ExistingDRBs)); if (p_ConfigureSRB2) { // @sic R5-206289 sic@ v_SRB_ToAddModList := {cs_508_SRB2_ToAddMod_DEFAULT}; // Need to configure the whole list for (i:=0; i < lengthof(v_ExistingDRBs); i :=i+1) { v_SS_ConfigList[i] := v_ExistingDRBs[i].SS_Config; v_DRB_ToAddModList[i] := v_ExistingDRBs[i].DRB_ToAdd; } // If called after TAU, then NAS Count MUST be a value // This will also configure all DRBs v_AuthParams := f_EUTRA_RRC_ActivateSecurity(p_CellId, v_AuthParams, valueof(p_NasCountUL), v_SS_ConfigList); f_EUTRA_Security_Set(v_AuthParams); } else { // Only need to configure the latest DRB (all others already configured) v_SS_ConfigList[0] := v_NewDRBs[0].SS_Config; v_DRB_ToAddModList[0] := v_NewDRBs[0].DRB_ToAdd; f_EUTRA_SS_CommonRadioBearerConfig(p_CellId, valueof(v_SS_ConfigList)); f_EUTRA_SS_RRC_RestartSecurity(p_CellId); } if (isvalue(p_PduInfoFromNR)) { // @sic R5s210149 sic@ // The UE has just moved from NR. If so, data from NR should be used, rather than created anew for (i := 0; i < lengthof (p_PduInfoFromNR); i := i + 1) { if (v_PdnIndex == valueof(p_PduInfoFromNR[i].PdnIndex)) { // Data from NR should be used, rather than created anew v_StartPDN := false; // no need to change DRB mapping as this has already been done in calling function break; } } } if (v_StartPDN) { // completely new PDN connection @sic R5s210149 sic@ f_IP_ChangeDrbMappingEUTRA(IP, p_CellId, v_NewDRBId, v_PdnIndex); } v_PcoToUE := f_NAS_ProtocolConfigOptionsTX(f_EUTRA38_MultiPDN_GetPCOs(p_CellId, v_PdnIndex, v_IsIMS, v_EpsDefaultBearerId, v_ExtdPcoFromUE, v_PcoFromUE)); if (isvalue (v_ExtdPcoFromUE)) { // FFS - At the moment, 36.508 table 4.7.3-6 specifies ExtdPCO should be not present } if (pc_IPv6 and pc_IPv4) { /* @sic R5s170882 sic@ */ select (v_PDN_TypeToBeUsed) { case (forceIPv4only) { v_ESM_Cause := cs_ESM_Cause_tv('00110010'B); } // PDN type IPv4 only allowed case (forceIPv6only) { v_ESM_Cause := cs_ESM_Cause_tv('00110011'B); } // PDN type IPv6 only allowed case (pdnTypeAsSupportedByUE) { v_ESM_Cause := omit; } } } /* Step 8 of procedure in 36.508 cl. 4.5.3.3 */ f_EUTRA38_MultiPDN_RRCConnectionReconfiguration_Common(p_CellId, tsc_RRC_TI_Def, v_SRB_ToAddModList, v_DRB_ToAddModList, 1, 0, {cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_508_ActivateDefEpsBearerContextRequest(v_EpsDefaultBearerId, v_PTId_UE, v_APN_Tx, // @sic R5-206296 sic@ v_PDN_Address, v_ESM_Cause, /* @sic R5s170882 sic@ */ v_PcoToUE, v_BearerContextNumber))}); return v_EpsDefaultBearerId; }

# Branches executed

NR Band 41, nea2, Nia2.

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 passed this test case on Datang LinkTester 5G Protocol Conformance Test System ECT9610. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test Case Execution log file:
\ TC_11_1_5_NR5GC_N41-FDD3.log

- PIXCIT settings used:
\ Pics(Hisilicon-IRAT).xml

\ Pixit(TC_11_1_5_NR5GC).xml

# References

| [1] | R5s211707: Supporting information for agreement of NR5GC test case 11.1.5 in FR1 |
| --- | --- |